SSC Project SR-1463

Structural Challenges Arctic Ships

Interim Draft

Reference:  BMT6696

Date:  March 2010

Structural Challenges faced by Arctic Ships

Interim Draft

March, 2010

Submitted by:

BMT FLEET TECHNOLOGY LIMITED

311 Legget Drive

Kanata, ON

K2K 1Z8

TABLE OF CONTENTS

1.	Introduction	1

2.	Outline of areas and topics	1

3.	Topic Outline	2

3.1	Changing Environmental Conditions	2

3.1.1	Climate Change	2

3.1.1.1	Projected Changes	2

3.1.1.2	Possible Impacts	2

3.1.2	Ice Cover	2

3.1.2.1	Types	3

3.1.2.2	Thickness	3

3.1.2.3	Ice Pressure	4

3.1.2.4	Ridge Rubble Consolidation	5

3.1.2.5	Lead Systems	5

3.2	Ice Loads on Ships	5

3.2.1	Ice Mechanics	5

3.2.1.1	Compressive strength	6

3.2.1.2	Other types of ice strength	9

3.2.2	Ice Load Measurements	9

3.2.2.1	Local Loads	9

3.2.2.2	Global Loads	11

3.3	Material and Structural Response	11

3.3.1	Structural Design and Assessment	11

3.3.1.1	Types	11

3.3.2	Vessel Fabrication	11

3.3.2.1	Types	11

3.3.3	Material Behavior	11

3.3.3.1	Types	12

3.4	Risk and Hazard Assessment	12

3.4.1	Information Technology	12

3.4.1.1	Computer Simulation	12

3.4.1.2	Remote Sensing	13

Remote Sensing Technologies	14

3.4.1.3	Databases	16

3.5	Regulatory and Other Issues	17

3.5.1	Human Resources	18

3.5.1.1	HR for Design	18

3.5.1.2	HR for Construction	18

3.5.1.3	HR for Operations	18

3.5.1.4	HR for Regulation	19

3.5.2	Remote Facilities	19

3.5.2.1	Navigation Support	19

3.5.2.2	Search and Rescue	20

3.5.2.3	Environmental Response	20

3.5.2.4	Vessel Repair	21

3.6	Issues Map and Research Needs	21

Appendix 1A : ice EnvironmeNt	22

Appendix 1B: Ice Loads and design of Ice Class Ships	23

Appendix 1C: Material and Structural STREngth of Ice Class ships	41

Appendix 1D: RISK and Hazard Assessment	43

Appendix 1E: Regulatory and Other ISSUES	45

Appendix 1f: Bibliography on ALL TOPICS	46

TABLE OF FIGURES

Figure 1: Sea ice extent during March and September (2009). Source-Richter-Menge et al.	3

Figure 2: Modeled seasonal cycle of ice thickness. Source-Rothrock et al., 1999	4

Figure 3: Rubble fields. source- M.L. Druckenmiller et al., 2009	5

Figure 4: Typical Lab test arrangement for uni-axial compression strength test of sea ice. source- (Timco and Weeks, 2010)	6

Figure 5: Uni-axial compressive strength data for iceberg and freshwater ice at −10 °C. source- (Jones, 2007)	7

Figure 6: Failure envelope for the brittle strength of saline ice (Iliescu and Schulson, 2004)	7

Figure 7: Borehole jack for in-situ compression test in ice. source- (photo by Lanthier from Timco and Weeks, 2010)	8

Figure 8: Comparison of first-year, second-year and multi-year borehole  strength during the decay season. (Johnston et.al., 2003)	8

Figure 9: Multi-year borehole strength as a function of temperature. (Johnston et.al., 2003)	9

Figure 10. Local pressure data from event #410 on the USCGC Polar Sea, from 1983. (Daley 2007)	10

Figure 11. Contact and pressure patterns in Hobson's Choice test. (Muhonen 1991)	10

# Introduction

This draft is a work in progress. It is presented to show the developing structure of the literature review. The main topics have been grouped in to five primary areas, each with multiple topics and sub-topics. For each sub-topic, a description and a list of keywords is presented below. As well, the relevant appendix is listed. In the appendix, the actual references are found. This interim report is intended to show the developing content of the report, although it remains a working draft at this point.

# Outline of areas and topics

The topics listed in the proposal have been grouped and subdivided as shown below. Section 3 expands on this list and developed a list of keywords that is used in the literature search.

| Area | Topic | Sub Topics | Key Issues |
| --- | --- | --- | --- |
| Changing | Climate Change |  |  |
| Environmental Conditions |  | Environmental Changes | Coverage, Thickness, Loss of MY ice |
|  |  | Potential Impacts | More variability, uncertainty |
|  | Ice Cover |  |  |
|  |  | First Year Ice |  |
|  |  | Multiyear Ice | Differentiation of MY thicknesses |
|  |  | Thickness |  |
|  |  | Pressure | Identification and prediction of pressure |
|  |  | Pressure Ridges |  |
|  |  | Rubble Ice |  |
|  |  | Consolidation |  |
|  |  | Lead Systems |  |
| Ice Load | Ice Loads |  |  |
| Scenarios | Developing Technologies | Computer Simulation |  |
| Material / | Materials |  |  |
| Structural | Design /Assessment |  |  |
| Response | Fabrication |  |  |
| Risk and Hazard | Key Risks |  |  |
| Assessment | Assessment Methods |  |  |
| Regulatory and | Regulation |  |  |
| Other Factors | Remote Facilities |  |  |
|  | People |  |  |
|  | Information Technology |  |  |

# Topic Outline

## Changing Environmental Conditions

### Climate Change

#### Projected Changes

Polar regions are amongst the most extensively modelled areas of the world for climate change forecasting.  In general, Global Climate Models (GCMs) used in the Arctic Climate Impact Assessment  and the Intergovernmental Panel on Climate Change Fourth Assessment Report (2007) predict continuous declines in sea ice coverage through the 21st century. At the extreme some simulations show that by the middle of the century, the entire Arctic Ocean could be ice-free for a short period in the summer. However, it is also important to note that no simulations have indicated that the winter sea ice cover of the Arctic Ocean will disappear during this century.

On a more local basis, the Canadian Arctic Archipelago is predicted to retain significant summer ice coverage and large concentrations of multi-year ice for longer than any other area of the Arctic.

Description

Key words

List of references

see Appendix 1A

#### Possible Impacts

Description

Key words

List of references

see Appendix 1A

### Ice Cover

There has been a substantial decrease in sea ice cover over the past few decades. Since the 1950s, there is a reduction in sea ice coverage by 10-15% (IPCC – Intergovernmental Panel on Climate change). The extent of sea ice cover is maximum in March and minimum in September. There is a twofold increase/decrease in sea cover between March and September (IPCC). The ocean and atmosphere play an intrinsic role in the extent of sea ice cover. A negative trend is apparent in the time series of the variability of ice extent at 2% in March and 7% in September (Richter-Menge et al., 2008). The mean ice edge position retreated significantly over a period of 150 years with greater retreat during the last century (Shapiro et al., 2001)

Figure 1: Sea ice extent during March and September (2009). Source-Richter-Menge et al.

#### Types

Description

Sea ice is comprised of both first year and multiyear ice. The presence of multiyear ice adds a significant dimension to the design issues. Since multiyear ice concentration has been showing a declining trend in the past decade, its implication to Arctic shipping is vital. The presence of less multiyear ice means there is a probability of extended Arctic navigation season. Passive and active microwave satellite remote sensing observations are used to monitor the extent and concentration of sea ice. It has been found out that two different ice regimes were not differentiable by microwave remote sensing under similar climatic conditions (David. G et al., 2010)

Key words

First year ice, multiyear ice, ice cover, decadal variability, sea ice

List of references

see Appendix 1A

#### Thickness

Description

Thickness of sea ice is one of the main restricting factors in commercial Arctic shipping and it is poorly documented. The speed at which commercial vessels can go through ice is directly related to the thickness of the ice. The thickness also plays a very important role in the structural design of the ship. The thickness of the ice also decreases with ice cover area during the melt season. It is more difficult to monitor ice thickness. Measurements of ice thickness can be made in situ. Satellite based techniques such as ICESat altimeter (Kwok et al., 2006) and obtaining ice thickness from satellite based estimates of ice freeboard (Laxon et al., 2003) are already in use, but these observations have been spatially and temporally limited. Ice thicknesses have also been measured by using submarines. Scientific Ice Expeditions (SCICEX) program have acquired many ice draft data in the 1990s (Gossett, 1999) (Margo, H et al., 2003). Data from submarine based observations indicate that at the end of the melt season the permanent ice cover  thinned by an average of 1.3 m between 1956– 1978 and the 1990s, from 3.1 to 1.8 m (Rothrock et al., 1999).

Figure 2: Modeled seasonal cycle of ice thickness. Source-Rothrock et al., 1999

There is a significant loss of older, thicker multiyear ice drifting out of the Arctic through the Fram strait (Rigor and Wallace, 2004). On the other hand, measurements of the seasonal ice cover do not indicate any statistically significant change in thickness in recent decades (Melling et al., 2005). The thickness of first year ice in level floes ranges from a few tenths of a meter near the southern margin of the marine cryosphere to 2.5 m in the high Arctic at the end of winter. In the present climate, old multi-year ice floes without ridges are about 3 m thick at the end of winter (ACIA).

Table 1: level ice thickness. source- S. Løset et al., 1999

The above table shows the measurement of monthly level ice thickness in various regions. The air temperature regimes play a role in the overall thickness of ice.

Key words

Ice thickness, thickness variability, ice draft distribution, sea ice, interannual variability

List of references

Appendix 1A

#### Ice Pressure

Description

Ice pressure is dependent upon many factors such as wind speed, current direction and current speed etc. The sea ice under pressure has the potential to stop the ship in its tracks by inhibiting its forward motion. Since most of the shipping will be done near coastal areas, fast ice is an inherent danger which can strand a ship.

Key words

Ice pressure, fast ice, ice floes, sea ice, floe size, wind speed, ocean currents, ice forces

List of references

Appendix 1A

#### Ridge Rubble Consolidation

Description

Ice ridges  are formed when sea ice floes collide with each other under pressure. These can happen near the sea ice land interface too. The ridges form when the floes buckle and break into blocks due to the compression of the ice pack. These ridges can be up to 30 m thick (ACIA). Waves are an additional cause of ridging near open water, notably in the Labrador, Greenland, and Barents Seas. Because of ridging and rafting, the average thickness of first-year sea ice is typically twice that achievable by freezing processes alone (Melling and Riedel, 1996). Heavily deformed multi-year floes near the Canadian Archipelago can average more than 10 m thick.

Figure 3: Rubble fields. source- M.L. Druckenmiller et al., 2009

A multiyear ridge is fully consolidated and has low salinity. The sail height of ridges can reach up to 6 m in height.

Key words

ridge, rubble, keel, sail height, wind speed, sea ice

List of references

Appendix 1A

#### Lead Systems

Description

Leads are ice free areas between ice floes which the ship can use for transit. Since a ship is like a vehicle, the lead systems can be used to navigate the ship through ice floes without sustaining any structural damage. Lead systems are short lived unlike polynyas.

Key words

Lead system, polynya, ice floes, ice edge, heat flux

List of references

Appendix 1A

## Ice Loads on Ships

### Ice Mechanics

The previous section discussed the various types of sea ice that ships encounter. When ship-ice interaction occurs, local and global loads occur on the ship. The loads will depend on the mechanical behavior of ice, and thus the mechanical properties of ice are certainly relevant.  Empirical evidence of ice loads on ships shows that the ship-ice interaction process is quite complex. It is not easy to show a strong link between mechanical properties of ice measured in a lab (or field) and the load phenomena on ships. Nevertheless, mechanical properties of ice are a starting point.

#### Compressive strength

Description

Local ice contact with ice will always involve compression of the ice edge. The standard test arrangement for measuring the uni-axial strength of ice is shown in Figure 4 (Timco and Weeks, 2010). Both freshwater and glacial ice have no salt, and yet the compressive strength at high (i.e. brittle) strain rates is noticeably different as shown in Figure 5 (Jones, 2007). The strength of saline ice (sea ice) under combined bi-axial stresses is shown in Figure 6 (Iliescu and Schulson, 2004). It shows that while uni-axial strength may be only around 5 MPa, biaxial stresses (confining stresses) can raise the apparent strength to over 20 MPa. As ice decays in spring and summer, the strength diminishes markedly. Figure 7 shows the use of a borehole jack device for measuring in-situ confined compressive strength of ice. Masterson et. al (1997) discuss the relationship between bore-hole jack tests and uni-axial tests. Figure 8 shows how strength (as measured by borehole jack) decays in first-year, second-year and multi-year ice during the decay season (Johnston et.al., 2003). First year ice decays more due o the presence of brine in the ice. Multi-year ice, which has almost no brine, maintains its strength to a much greater extent. This is an important factor when considering ship operations in the arctic in late spring and summer.   Figure 9 shows how borehole jack strength in old ice varies with temperature (i.e. from winter to summer) and can be nearly 40MPa (Johnston et.al., 2003). The temperature effect on triaxial strength of ice was investigated in Fish et.al.. (1997).

Figure 4: Typical Lab test arrangement for uni-axial compression strength test of sea ice. source- (Timco and Weeks, 2010)

Figure 5: Uni-axial compressive strength data for iceberg and freshwater ice at −10 °C. source- (Jones, 2007)

Figure 6: Failure envelope for the brittle strength of saline ice (Iliescu and Schulson, 2004)

Figure 7: Borehole jack for in-situ compression test in ice. source- (photo by Lanthier from Timco and Weeks, 2010)

Figure 8: Comparison of first-year, second-year and multi-year borehole  strength during the decay season. (Johnston et.al., 2003)

Figure 9: Multi-year borehole strength as a function of temperature. (Johnston et.al., 2003)

Key words

compressive strength, uni-axial strength, biaxial strength, borehole jack, failure envelope,

List of references

see Appendix 1B

#### Other types of ice strength

Description

A comprehensive review of various mechanical properties of ice is presented in Hooke at. al (1980). Timco and Weeks, (2010) present a more recent review of mechanical properties. Fracture properties of ice at high strain rates are reviewed in Dutta et. al. (2004). Ice load and pressure measurements are often observed with ice, and are the subject of much continuing debate. Scale affects are discussed in Gagnon et. al. (2001) and Goldstein et.al. (2009). Flexural strength is a key parameter for ice loads on ships in normal operations. The flexural strength of first year sea ice in the Barents Sea is discussed in Krupina et.al. (2007)

Key words

tensile strength, shear strength, flexural strength, fracture strength,

List of references

see Appendix 1B

### Ice Load Measurements

#### Local Loads

Description

Figure 10. Local pressure data from event #410 on the USCGC Polar Sea, from 1983. (Daley 2007)

Figure 11. Contact and pressure patterns in Hobson's Choice test. (Muhonen 1991)

Key words

List of references

see Appendix 1B

#### Global Loads

Description

Key words

List of references

Appendix 1B

## Material and Structural Response

### Structural Design and Assessment

#### Types

Description

...

Key words

List of references

Appendix 1C

### Vessel Fabrication

Known issues to be explored:

- Large shipyards may not have the automated capability to construct ships with frame-spacings small enough for ice classed vessels.

- …

#### Types

Description

.

Key words

List of references

Appendix 1C

### Material Behavior

The main concerns regarding material behavior for ships operating in cold regions is the effect of the cold temperatures on the behavior of the material when it is stressed.  Arctic going ships are generally constructed of steel; however some research is currently being carried out on fiber-reinforce-plastic (FRP) hull – ice interactions.  FRP hulls generally find service as escape/evacuation vehicles for other arctic going vehicles.

Recent research into the effect of cold temperatures on ship steels has shown that the fracture strain of steel decreases for experiments involving dry ice and acetone as the cooling medium versus liquid nitrogen.  This is significant because the dry ice/acetone environment is supposed to more closely resemble an arctic environment than the liquid nitrogen setup; implying that other previous laboratory experiments into the effect of cold temperatures on steel material behavior (which mostly used liquid nitrogen cooling) may be overestimating the quasi-static failure strain.   …  (Kim et al. 2009, 117-124) has investigated the ASTM A500 steel.

#### Types

Description

Kim, B. J., S. K. Park, B. S. Jang, and J. K. Paik. 2009. Crushing behaviour of steel plated structures in cold temperature: Experimental and numerical studies.

Key words

List of references

Appendix 1C

## Risk and Hazard Assessment

### Information Technology

#### Computer Simulation

Description

Parallelization

Traditionally, computer hardware consisted of a single computer processing unit (cpu), and hence software was designed to execute single processes in a specific order.  Within the past decade, the trend for both computer hardware and software has been towards parallel processing.  Parallel processing is the ability to carry out multiple simultaneous processes (i.e. “threads”).  This requires specialized hardware and software.

Computer simulations have been used for decades to aid in all aspects of ship analysis, design and construction.  Traditionally, methods like strip theory (for ship motions), implicit/explicit finite element analysis (for structural/thermal analyses) and Navier-Stokes equations (for fluid flow) have been implemented for general purpose use on a single cpu, or else special implementations were written for execution on multi-million dollar, room sized super computers.  Exceptions to this rule have been general purpose implementations of these programs for computer clusters.  Computer clusters are groups of commercially available off the shelf personal computers.  Computer clusters are an economical way of approaching the processing power of super computers.  Since the early 1990’s, some personal computers (most notably server-class machines) have been sold with multiple processors.  Since 2005, personal computers have been sold with processors that have multiple “cores” in a single processor.  A multi-core cpu can carry out multiple simultaneous or “parallel” processes.  Commercial off the shelf personal computers are available today with up to 4 multi-core cpus, each having up to 6 cores (i.e. a total of 24 parallel processes per personal computer).  This progression toward parallel computing has led many providers of computer simulation software to release “parallelized” implementations of their code.  Readily available parallelized code, relatively inexpensive cluster computing and a concern that cpu’s are reaching their technological boundaries has opened the market to a new parallel computing solution:  massively parallel multi-core general purpose graphics processing units (gpgpu).  Nvidia currently leads the market in massively parallel gpgpu offerings, but AMD has similar architecture available, and Intel is presently in development.  A gpgpu is very similar to a multi-core cpu, except it consists of many more cores (present Nvidia ‘Fermi’ architecture offers 512 cores per gpgpu) and each core is designed to execute a highly simplified task.  It is possible for one personal computer to contain multiple gpgpus, and it is also possible to cluster these personal computers; essentially a cluster of clusters.  This arrangement notionally leads to computer clusters that have tens of thousands of parallel computing cores.  With this type of architecture, real-time finite element simulations are possible.  In fact, real time finite element simulations are currently taking place in gpgpu environments that are nowhere near as sophisticated as described above.  A company called Pixelux is presently licensing software that executes real-time finite element analysis for the gaming community.

Computer Software

Include X-FEM (Extended Finite Element Method)

http://www.scitopics.com/The_Extended_Finite_Element_Method_X_FEM.html

Key words

Computer simulation, massively parallel computing, general purpose gpu, cluster computing, X-FEM

List of references

Appendix 1D

Halfhill, T. 2009. Looking beyond graphics. In-Stat, .

Paik, J. K. 2010. Some recent advances and future trends in nonlinear structural mechanics for ships and offshore structures. Marine Technology 47, (1): 17-26 (accessed 22 March 2010).

Paik, J. K., and J. K. Seo. 2009. Nonlinear finite element method models for ultimate strength analysis of steel stiffened-plate structures under combined biaxial compression and lateral pressure actions-part II: Stiffened panels. 47, (8-9): 998-1007.

Su, Biao, Kaj Riska, and Torgeir Moan. 2010. A numerical method for the prediction of ship performance in level ice. Cold Regions Science and Technology 60, (3) (3): 177-88.

#### Remote Sensing

Description

Remote sensing as it relates to arctic shipping involves collecting data on the following arctic environmental factors:  ice, waves, bathymetry, and weather phenomena such as wind, atmospheric pressure and temperature.

Much literature is present regarding cold region remote sensing.  Much of this literature is not relevant to ships operating in these regions and has not been included in this literature survey.  Many of these remote sensing techniques however, may be adapted for uses applicable to arctic shipping.  An overview of existing and adaptable remote sensing techniques is presented below

##### Remote Sensing Technologies

###### Submarine

###### Ground

- CReSIS Sensor Developments

- Synthetic Aperture Radar (SAR)

- Accumulation Radar

###### Airborne

Ground Penetrating Radar (GPR) - a near-surface, non-invasive geophysical technique.  Provides images of the dielectric properties of the top few tens of meters of the earth.  Resolution is approximately metre scale.  Radar data can be used to detect the presence of liquid organic contaminants, many of which have dielectric properties distinctly different from those of the other solid and fluid components in the subsurface. GPR images are interpreted to obtain models of the large-scale architecture of the subsurface and to assist in estimating hydrogeologic properties such as water content, porosity, and permeability.

(Knight 2001, 229-255)

CReSIS Sensor Developments

- Multichannel Coherent Radar Depth Sounder (MCoRDS)

- Accumulation Radar

- Snow Radar

- Ku-band Radar Altimeter

- UAS Radar

###### Satellite

Available satellite technologies consist of optical imaging sensors, microwave imaging sensors and non-imaging sensors.

Optical imaging sensors detect either reflected or emitted radiation.

Sensors detecting visible light from the sun that is reflected off objects on earth are good for observing sea ice because sea ice has a high albedo compared with the surrounding ocean.  Being dependent on visible light, these sensors are limited in arctic application during the winter months by a persistent lack of daylight.  Further, cloud cover limits their use year round.

Sensors detecting infrared radiation emitted from objects on earth are also good for observing sea ice because the sea ice temperature is generally colder than that of the surrounding ocean.  Limitations on the use of these sensors come from infrared radiation from clouds, and the near similar temperatures between melting ice and sea water during the warmer seasons.

The following satellites and sensors are commonly used to identify and map sea ice using both visible and infrared sensors:  the Defense Meteorological Satellite Program (DMSP) Operational Linescan System (OLS), the National Oceanic and Atmospheric Administration (NOAA) Advanced Very High Resolution Radiometer (AVHRR) and the National Aeronautics and Space Administration (NASA) Moderate Resolution Imaging Spectroradiometer (MODIS).

Passive microwave imaging sensors detect emitted microwave radiation from objects on earth.  Microwaves emitted by ice can penetrate cloud cover, and are significantly different in magnitude than those emitted by the surrounding ocean.  Because the microwave radiation emitted by ice is small in magnitude, it is difficult to detect unless the observation area is large; therefore, sea ice details (e.g. pack ice concentration) are generally unavailable.  These sensors are valuable for detecting the presence of sea ice in a geographic area, and information availability is not limited by sunlight or clouds.  These sensors have been used to monitor sea ice since 1972.  Ice observations from the following sensors are available from NSIDC:  Electrically Scanning Microwave Radiometer (ESMR), NASA’s Scanning Multichannel Microwave Radiometer (SMMR) DMSP Special Sensor Microwave/Imager (SSM/I) and the Advanced Microwave Scanning Radiometer–Earth Observing System (AMSR-E) sensor.

Active microwave imaging systems emit microwave radiation toward objects on the earth and detect the reflected microwaves.  This provides them a much finer resolution than passive systems.  One type of active system that is used for remote ice sensing is the Synthetic Aperture Radar (SAR).  This system is a special type of imaging radar system from which sea ice characteristics may be determined.  Since the amount of reflected energy depends on the characteristics of the ice, this type of sensor may be used to identify thick multi-year ice versus thin first year ice.  The RADARSAT mission, managed by the Canadian Space Agency, is the primary SAR mission today.  In addition to identifying multi-year ice, SAR instruments can detect small leads in sea ice allowing them to help route ships through ice-covered regions.  SAR images are currently used by the Canadian Ice Service and the National Ice Center.

###### Ice

(Quincey and Luckman 2009, 547-567) provides a current state-of-the-art review of ice related satellite remote sensing technologies, methods and missions.

Ice remote sensing includes obtaining data regarding ice salinity (i.e. an analogue for ice age), thickness, snow cover, temperature, location, extent, and topography (e.g. pack ice concentration, as well as ice features such as ridges, hummocks, inclusions…).

Ice remote sensing consists of techniques from ground level observation to airborne and satellite data collection.  The various techniques are described below.

###### Available Methods for Obtaining Ice-related Data

The current methods of obtaining ice related data from satellite sensors consist of speckle and feature tracking, interferometry, passive microwave and scatterometry, and altimetry…

There is presently much research being undertaken to correlate wave propagation through sea ice with ice thickness...

###### Waves

Remote sensing of waves is important for both ship operating in waves, as well as for data used to determine ice thickness.

###### Bathymetry

Arctic bathymetric charts are not as accurate…

###### Weather

###### Applicability to Arctic Shipping

Key words

List of references

Appendix 1D

#### Databases

Description

Existing databases of ice related information include:

- Iceberg Databases

  - BMT Fleet Technology – Iceberg Sightings Database

  - NSIDC-IIP – Iceberg Sightings Database

  - NRC-IOT – Iceberg Collisions

  - NRC-CHC – Iceberg Management

- Marine Icing Databases

  - NRC-CHC

- Ice Charts Database

  - NRC-IOT – Ice Charts Database (1810-1958)

  - Canadian Ice Service

Key words

Iceberg database, ice chart database, ice service

List of references

Appendix 1D

## Regulatory and Other Issues

The main regulatory systems affecting Arctic operations include:

International

United Nations Convention on the Law of the Sea (UNCLOS 82)

International Maritime Organization Guidelines for Ships Operating in Polar Waters

Antarctic Treaty

National

Canadian Arctic Shipping Pollution Prevention Regulations (ASPPR)

Russian Northern Sea Route:

- Regulations for Navigation on the Seaways of the Northern Sea Route

- Regulations for Icebreaker-Assisted Pilotage of Vessels on the NSR

- Requirements for Design, Equipment and Supply of Vessels Navigating the NSR

- Much of the current regulatory development for Arctic waters references Article 234 of UNCLOS, the so-called “Arctic clause”, which is quoted in full below:

Article 234

Ice-covered areas

Coastal States have the right to adopt and enforce non-discriminatory laws and regulations for the prevention, reduction and control of marine pollution from vessels in ice-covered areas within the limits of the exclusive economic zone, where particularly severe climatic conditions and the presence of ice covering such areas for most of the year create obstructions or exceptional hazards to navigation, and pollution of the marine environment could cause major harm to or irreversible disturbance of the ecological balance. Such laws and regulations shall have due regard to navigation and the protection and preservation of the marine environment based on the best available scientific evidence.

It is important to note that while the Arctic does have Coastal States (and recognized, though somewhat disputed exclusive economic zones (EEZ), the Antarctic does not.  The Antarctic Treaty aims to preserve the Antarctic from development, while all of the Arctic Coastal States orient their Arctic policies to a greater or lesser extent towards sustainable development of resources and infrastructure.

Currently, the main regulatory thrust internationally is to reformulate the IMO recommendatory Guidelines into a mandatory Code for ships operating in Polar Waters.  The schedule for this foresees ratification and implementation by 2012.

At a national level, Canada is moving to incorporate the IMO approach and the IACS Polar Classes into its Arctic regulations.

Key words

Arctic shipping regulations, polar shipping regulations

List of references

Appendix 1E

### Human Resources

#### HR for Design

Description

Key words

List of references

Appendix 1E

#### HR for Construction

Description

Key words

List of references

Appendix 1E

#### HR for Operations

Description

There are navigation seasons for ships when they can proceed without an ice breaker escort. These seasons vary for different regions in the Arctic. The table shows the operating seasons for different regions. The navigation season can be extended by the use of icebreakers to assist shipping.

Table 2: Navigation Season - Source: Donn K. haglund, Maritime transport in support of arctic resource development

Reliable short wave and long wave communications are a necessity. Short wave VHF and UHF work very well irrespective of the atmospheric conditions in the Arctic, but it is restricted to line of sight. Long wave communications are more prone to polar-unique conditions. Satellite communications are reliable with the use of polar orbiting satellites. Use of satellite phones have become an important link in ship to ship and ship to shore communications in recent times.

Key words

List of references

Appendix 1E

#### HR for Regulation

Description

....

Key words

List of references

Appendix 1E

### Remote Facilities

#### Navigation Support

Description

Key words

List of references

Appendix 1E

#### Search and Rescue

Description

In a 2007 analysis by the Canadian Coast Guard (2007), it was seen that the SAR effectiveness in Arctic areas was less than 90%. The national benchmark assumed for the analysis of SAR system in Canada was 90%. The SAR effectiveness is defined as the percentage of lives saved out of the total number of lives at risk. This is to be achieved during conventional incidents.According to the Canadian Coast Guard, conventional units are defined as:

- resources are able to respond within a short period of time;

- the search object is located by the responding resource on scene in a timely manner;

- environmental, geographic, and hydrographic conditions have little impact on the successful resolution of the incident; and,

- the responding resource has the necessary capability and capacity to effectively resolve the incident.

SAR services for the Arctic maritime environment are very challenging. Incidents of Arctic SAR are not termed as “conventional incidents”, but are termed as “difficult incidents”, due to their harsh conditions. The level of system effectiveness typically accepted for SAR effectiveness in the Arctic is around 50+ %. The 2007 SAR system effectiveness evaluation revealed higher-than-expected levels of service: 69.23% for the waters of the Northwest Territories Area; 86.67% for the James Bay Area; 81.48% for the eastern Arctic Area; and, 93.10% for the Nunavut Area.

For Northern Canada, there is a lack of SAR response units. The current SAR capacity in Northern Canada will not be able to meet the increased demands of the future. This would be partly due to year round commercial shipping in the future. 3 CCGA units already exist in the Arctic at Rankin Inlet, Cambridge Bay and Iqaluit.

Key words

List of references

Appendix 1E

#### Environmental Response

Description

Environmental response refers to the policies and regulations which are present and developed to reduce the impact of impact of marine pollution incidents on the environment. This is a very important piece of regulation because the Arctic Ocean is one of the pristine and untouched ecosystems left in the world and any type of damage to the environment due to marine pollution will have a lasting impact. Moreover, due to the remote locations where the ships would ply make it difficult for response to be efficient.

Since ships in the arctic would use ballast water in their voyage, it is necessary that these ballast waters are disposed of properly to reduce the contamination of Arctic waters with invasive species. A large number of waste water lagoons are being operated on the Northwest Territories for many years.

The basic objective is that a la goon should provide an effective method of wastewater disposal without creating objectionable conditions or public health hazards in the vicinity or downstream in the receiving water course.

Wastewater lagoon systems for the Arctic and sub-Arctic regions may be classified into four types:

1. Single cell, long-retention.

2. Primary, short-retention.

3. Secondary, long-retention

4. Aerated

Due to the fragile nature of the Arctic ecosystems, stringent measures are needed for shipping in the Arctic. Canada and United States are already developing stringent measures for shipping and offshore operations in the Arctic.

Key words

List of references

Appendix 1E

#### Vessel Repair

Description

For any type of vessel repair facility in the Arctic, the location of the facility is of utmost importance. The location of the facility shall not impede the entry of vessels into the facilities. Most of the bays and inlets around the Canadian Arctic archipelago have fast ice and pack ice for most of the season and any facilities present in these areas have to make sure that they are accessible throughout the shipping season. Many of the northern ports have been built in natural harbours to provide protections from the environment.

The main challenge with a repair facility in the Arctic is that there would be a need for ships to be dry-docked for repairs as the damage would have been below the waterline. This would usually be the case since the most of the damage would occur due to interaction with ice floes.

A dry-dock in the Arctic would be a dead load on the soil. The location of the facilty must consider low-loadbearing soils and changes in soil conditions due to permafrost or possible modification of permafrost.

Key words

List of references

Appendix 1E

## Issues Map and Research Needs

Information Technology

# Appendix 1A : ice EnvironmeNt

References related to ice cover characteristics

Barry, Gene, Torkild Carstens, Kenneth Croasdale, Robert Frederking, and Tom Brown. 1991. Ice break-up model for northumberland strait. Paper presented at 11th International Conference on Port and Ocean Engineering under Arctic Conditions - POAC'91, September 24, 1991 - September 28, .

Brigham, L., Cerne, M., Cole, K., et al. AMSA Bering Strait Region Case Study

Daly, S.F., Arcone, S.A.,"Airborne Radar Survey of a Brash Ice Jam in th St. Clair River" CRREL Report No. 89-2, Hanover, N.H., 1989.

Gossett, J., Arctic research using nuclear submarines, Sea Technol., 37(3), 33{40, 1996.

Kankaanpää, P., "Structure of First Year Pressure Ridges in the Baltic", Proc. POAC '89, Luleå, Sweden, 1989.

Kwok, R., G.F. Cunninham, H.J. Zwally, and D. Yi (2006) ICESat over Arctic sea ice: Interpretation of altimetric and reflectivity profiles. J. Geophys. Res.,111, C06006, doi: 10.1029/2005JC003175.

Laxon, S., N. Peacock, and D. Smith (2003) High interannual variability of sea ice thickness in the Arctic Region. Nature, 425, 947–950.

Leppäranta, M., Hakala, R., "Field Measurements of the Structure and Strength of First-year Ice Ridges in the Baltic Sea", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Lewis, J.E., Leppäranta, M., Granberg, H.B., "Airborne Laser Profiling of Ice Ridges in the Baltic Sea", Proc. POAC '89, Vol.3, Luleå, Sweden, 1989.

Loset, S., and T. Carstens. 1996. Sea ice and iceberg observations in the western barents sea in 1987. Cold Regions Science and Technology 24, (4): 323-40, Colony, R., "A New Look at Sea Ice Thickness", Proc. POAC '87, Fairbanks, Alaska, 1987.

Margo H. Edwards, Bernard J. Coakley, SCICEX Investigations of the Arctic Ocean System, Chemie der Erde - Geochemistry, Volume 63, Issue 4, 2003, Pages 281-328, ISSN 0009-2819, DOI: 10.1078/0009-2819-00039.

Matthew L. Druckenmiller a,⁎, Hajo Eicken a, Mark A. Johnson b, Daniel J. Pringle a,c, Christina C. Williams a Toward an integrated coastal sea-ice observatory: System components and a case study at Barrow, Alaska, Cold Regions Science and Technology 56 (2009) 61–72

McPhee, M.G., "Inferring Ice/Ocean Surface Roughness from Horizontal Current Measurements", Proc. Seventh Conference on Offshore Mechanics and Arctic Engineering, Houston, 1988.

Moritz, R.E., Colony, R., "Statistics of Sea Ice Motion, Fram Strait to North Pole", Proc. Seventh Conference on Offshore Mechanics and Arctic Engineering, Houston, 1988.

Richter-Menge, J., J. Overland, A. Proshutinsky, V. Romanovsky, L. Bengtsson, L. Brigham, M. Dyurgerov, J.C. Gascard, S. Gerland, R. Graversen, C. Haas, M. Karcher, P. Kuhry, J. Maslanik, H. Melling, W. Maslowski, J. Morison, D. Perovich, R. Przybylak, V. Rachold, I. Rigor, A. Shiklomanov, J. Stroeve, D. Walker, and J. Walsh (2006) State of the Arctic Report. NOAA OAR Special Report, NOAA/OAR/PMEL, Seattle, WA, 36 pp.

Rothrock, D.A., Y. Yu, and G.A. Maykut (1999) Thinning of the Arctic sea-ice cover. Geophys. Res. Lett., 26, 3469–3472.

Sayed, M., Frederking, R., "measurement of Ridge Sails in the Beaufort Sea", Can. Jnl. of Civil Engineering, Vol.16, No.1, February 1989.

Wright, Christopher, Seymour, Jonatha, Clinton, Graeme, Lumsden, Wayne and Stubbs, Thom. Canadian Arctic Shipping Assessment - Main Report June 2007

# Appendix 1B: Ice Loads and design of Ice Class Ships

The topics include mechanical properties of ice,  models and measurements of ice loads on ships as well as papers concerned with general design for ice loads.

Mechanical Properties of Solid Ice

Dutta, Piyush K., David M. Cole, Erland M. Schulson, and Devinder S. Sodhi. 2004. A fracture study of ice under high strain rate loading. International Journal of Offshore and Polar Engineering 14, (3): 182-8.

Fish, Anatoly M., and Yuri K. Zaretsky. 1997. Temperature effect on strength of ice under triaxial compression. Paper presented at Part 3 (of 4), May 25, 1997 - May 30, .

Gagnon, R. E., S. J. Jones, R. Frederking, P. A. Spencer, and D. M. Masterson. 2001. Large-scale hull loading of sea ice, lake ice, and ice in tuktoyaktuk harbour. Journal of Offshore Mechanics and Arctic Engineering 123, (4): 159-69.

Goldstein, Robert V., Nikolai M. Osipenko, and Matti Lepparanta. 2009. Relaxation scales and the structure of fractures in the dynamics of sea ice. Cold Regions Science and Technology 58, (1-2): 29-35.

Hobbs, P. V. (1974) Ice Physics. Clarendon Press, Oxford.

Hooke, R. L., M. Mellor, W. F. Budd, J. W. Glen, A. Higashi, T. H. Jacka, S. J. Jones, et al. 1980. MECHANICAL PROPERTIES OF POLYCRYSTALLINE ICE: AN ASSESSMENT OF CURRENT KNOWLEDGE AND PRIORITIES FOR RESEARCH. Cold Regions Science and Technology 3, (4): 263-75.

Iliescu, Daniel, and Erland M. Schulson. 2004. The brittle compressive failure of fresh-water columnar ice loaded biaxially. Acta Materialia 52, (20): 5723-35.

Jones, Stephen J. 1997. High strain-rate compression tests on ice. Journal of Physical Chemistry B 101, (32): 6099-101.

Jones, Stephen J. 2007. A review of the strength of iceberg and other freshwater ice and the effect of temperature. Cold Regions Science and Technology 47, (3): 256-62.

Jones, Stephen J., R. E. Gagnon, A. Derradji, and A. Bugden. 2003. Compressive strength of iceberg ice. Canadian Journal of Physics 81, (1-2): 191-200.

Jordaan, I.J., and McKenna, R.F., 1991 “Processes of deformation and fracture of ice in compression”, in Ice-Structure Interaction, Jones S.J., et.al. (Eds), IUTAM-IAHR Symposium St. John’s, Newfoundland, Canada, Pub. by Springer Verlag.

Kamio, Zenji, Junichiro Ushikoshi, Hisao Matsushita, Takashi Terashima, and Hiroshi Saeki. 2002. Mechanical strength of consolidated sea ice rubble- in case of first-year sea ice at notoro lagoon, hokkaido. Paper presented at Proceedings of the Twelfth (2002) International Offshore and Polar Engineering Conference, May 26, 2002 - May 31, .

Kim, Hyonny, and John N. Keune. 2007. Compressive strength of ice at impact strain rates. Journal of Materials Science 42, (8): 2802-6.

Krupina, Nina A., and Nickolay V. Kubyshkin. 2007. Flexural strength of drifting level first-year ice in the barents sea. Paper presented at 17th 2007 International Offshore and Polar Engineering Conference, ISOPE 2007, July 1, 2007 - July 6, .

Li, Zhijun, Yongxue Wang, and Guangwei Li. 2002. On the flexural strength of DUT-1 synthetic model ice. Cold Regions Science and Technology 35, (2): 67-72.

Masterson, D. M., W. P. Graham, S. J. Jones, and G. R. Childs. 1997. A comparison of uniaxial and borehole jack tests at fort providence ice crossing, 1995. Canadian Geotechnical Journal 34, (3): 471-5.

Murrell, S.A.F., Sammonds, P.R., and Rist, M.A., 1991 “Strength and failure modes of pure ice and multi-year sea ice under uniaxial loading”, in Ice-Structure Interaction, Jones S.J., et.al. (Eds), IUTAM-IAHR Symposium St. John’s, Newfoundland, Canada, Pub. by Springer Verlag.

Palmer, A.C., and Sanderson, T.J.O., 1991,“Fractal crushing of ice and brittle solids”, Proc. R.Soc. Lond. A, 433: 469-477.

Peyton H.R. 1966 "Sea ice strength" Univ. of Alaska Geophysical Institute Report AUG R-182, Dec. 1966.

Shafrova, Svetlana, and Knut V. Hyland. 2008. The freeze-bond strength in first-year ice ridges. small-scale field and laboratory experiments. Cold Regions Science and Technology 54, (1): 54-71.

Shafrova, Svetlana, and Per Olav Moslet. 2006. In-situ uniaxial compression tests of level ice part II: Ice strength spatial distribution. Paper presented at 25TH International Conference on Offshore Mechanics and Arctic Engineering, OMAE 2006, June 4, 2006 - June 9, .

Swamidas, A. S. J., Ian J. Jordaan, Stephen J. Jones, and Richard E. McKenna. 1991. Modelling of the ice failure processes in ship/ice interaction. Paper presented at 11th International Conference on Port and Ocean Engineering under Arctic Conditions - POAC'91, September 24, 1991 - September 28, .

Timco, G. W., and W. F. Weeks. 2010. A review of the engineering properties of sea ice. Cold Regions Science and Technology 60, (2): 107-29.

Totman, C. A., O. E. Uzorka, J. P. Dempsey, and D. M. Cole. 2007. Sub-size fracture testing of FY sea ice. Paper presented at 6th International Conference on Fracture Mechanics of Concrete and Concrete Structures, FraMCoS-6, June 17, 2007 - June 22, .

Tremblay, L. -B, and M. Hakakian. 2006. Estimating the sea ice compressive strength from satellite-derived sea ice drift and NCEP reanalysis data. Journal of Physical Oceanography 36, (11): 2165-72.

Wilchinsky, Alexander V., and Daniel L. Feltham. 2004. Dependence of sea ice yield-curve shape on ice thickness. Journal of Physical Oceanography 34, (12): 2852-6.

Mechanical Properties of Rubble/Fragmented Ice

Coon, M.D., "Mechanical Behavior of Compacted Arctic Ice Floes", Offshore Technology Conference, Dallas Tx., 1972.

Dean, A.M., "Field Techniques for Obtaining Engineering Characteristics of Frazil Ice Accumulations", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

Gale, A.D., Wong, T.T., Sego, D.C., Morgenstern, N.R., "Stress-Strain Behavior of Cohesionless Broken Ice", Proc. POAC '87, Fairbanks, Alaska, 1987.

Gosink, J.P., Osterkamp, T.E., "Frazil Ice Nucleation by Ejecta from Supercooled Water", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

Greisman, P., "Brash Ice Behavior", Report CG-D-30-81 by the United States Coast Guard, Groton CT., 1981.

Hopkins, M.A., Hibler, W.D., "On Modelling the Energetics of the Ridging Process", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Ostaja-Starzewski, M., Jessup, R., Venkatesh, S., "Micromechanics Approach to Sea Ice Dynamics", Proc. Canadian East Coast Conf. on Sea Ice, Bedford Inst. of Oceanography, Dartmouth, N.S., Canada, Jan. 1986.

Roth, D.R., Marcellus, R.W.W., "Bearing Capacity of Broken Ice Zones", Report by Canadian Marine Engineering Limited to Public Works Canada, 1986

Sayed, M., Frederking, R., "On Modelling of Ice Ridge Formation", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

Timco, G.W., Sayed, M., "Model Tests of the Ridge Building Process in Ice", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

Wong, T.T., Gale, A.D., Sego, D.C., Morgenstern, N.R., "Shear Box Tests on Broken Ice", Proc. POAC '87, Fairbanks, Alaska, 1987.

Ramming Very Thick Ice;

Blanchet, D., Kivisild, H.R., Grinstead, J., "Equations for Local Ice Energies During Ship Ramming", Presented at Proc. POAC '87, Fairbanks, Alaska, 1987.

Daley, C.G., " Dynamic Ship/Ice Impact - Results of Parametric Model Testing" Proc. Ice Technology Conference, Boston, 1986.

Daley, C.G., "BAFFIN - A Dynamic Ship-Ice Interaction Model", Proc. SNAME Ice Tech '84, Calgary, Canada, 1984.

Ghoneim, G.A.M., "Local and Global Strength Aspects for Icebreaking Ships", Proc. IPTC Vancouver, Canada, 1986.

Ghoneim, G.A.M., Johansson, B.M., Smyth, M.W., Grinstead, J., "Global Ship Ice Impact Forces Determined from Full-Scale and Analytical Modelling of the Icebreakers Canmar Kigoriak and Robert LeMeur", Transactions of the SNAME Annual Meeting, New York, 1984.

Howard, D., Menon,B., Daley, C., "Parametric Modelling of a 150,000 Tonne Tanker - Hydroelastic response during Multi-year Ice Impact" Report by Fleet Technology Limited to Canadian Coast Guard Northern, Transport Canada Report No. TP10031, Nov. 1989.

Huther, M., Beghin, D., Mogensen, O., "Hull Girder Minimum Section Modulus of Large Merchant Ice Breakers",  Proc. SNAME Ice Tech '84, Calgary, Canada, 1984.

Hysing, T., "Form Design of Vessels. Part II: Numerical Simulation Model", Marine Structures in Ice, NHL Report 81-03, Trondheim, Norway, 1981.

Laskow, V., "Ship-Ice Interaction: A Designers Approach", SNAME Arctic Section Spring Meeting, Calgary, Canada, 1982.

Matsuishi, M., Ikeda, J., Hawakami, H., Hirago, M., "Ship-Ice Floe Collision Analysis Considering the Elastic Deflection of Hull Girder", Proc. SNAME Ice Tech '84, Calgary, Canada, 1984.

Nawwar, A, Noble, P.G., "Ice Force Math Model Investigations", Report by Arctec Canada Limited to Transportation Development Centre, Montreal, Report No. TP 2596, 1980.

Noble, P.G., Tam, W.K., Menon, B., Bayly, I.M., "Ice Forces and Accelerations on a Polar Class Icebreaker", Proc. POAC '79, Trondheim, Norway, 1979.

Nogid, L.M., "Impact of Ships with Ice", Trans. Leningrad Shipbuild. Inst. No. 26, BSRA Translation No. 1867, 1959.

Riska, K., "An Analysis of Factors Influencing Ship Response in Collision with Multi-Year Ice Floes", Proc. POAC '89, Luleå, Sweden, 1989.

Riska, K., "On the Mechanics of the Ramming Interaction between a Ship and a Massive Ice Floe", Thesis for degree of Doctor of Technology, Technical Research Centre of Finland, Publications 43, Espoo, Finland, 1987.

Riska, K., "Ship Ramming Multi-year Ice Floes, Model Test Results", Technical Research Centre of Finland, Research Notes 818, Espoo, Finland, 1988.

Riska, K., "Theoretical Modelling of Ice-Structure Interaction", to be Presented at IUTAM/IAHR Symp. on Ice-Structure Interaction, St. John's, Canada, 1989.

Riska, K., Daley, C., "M.V. Arctic Ramming Model Test Results", Report 1 of the Joint Research Project Arrangement III between Technical Research Centre of Finland and Canadian Coast Guard, Espoo 1986.

Thompson, E.W., Cheung, H.C., "Continuation on Stress Analysis - Mathematical Model of the Hull Girder Structure of the M.V. Arctic", Report by Melville Shipping Ltd. to Canadian Coast Guard, 1982.

Thompson, E.W., Cheung, H.C., "Development of Prediction Model for Dynamic Response and Bow Loading of Ships" Report by Melville Shipping Ltd. to Transportation Development Centre, Montreal, Report No. TP 4191, 1983.

Tunik, A.L., "Hull Girder Bending Forces Due to Ramming Icebreaking", Proc. POAC '85, Narssarssuaq, Greenland, 1985.

Tunik, A.L., Minnick, P.V., St. John, J.W., Chen, Y.K., Chen, A.P.Y., "Ramming Forces on the USCGC Polar Sea", Proc. IAHR Ice Symp. Sapporo, Japan, 1988.

Vaughan, H., "Design Formula for the Shear Area and Section Modulus of Ice-Breaking Ships", RINA Transactions, 1983.

Vaughan, H., "Flexural Response of Ice-Breaking Ships to Impact Loads", RINA Transactions, 7p., 1986.

Ice Ridge Transit

Johansson, B.,M., Liljeström, G.C., "ODEN A State-of-the-Art Icebreaker", Proc. POAC '89, Luleå, Sweden, 1989.

Keinonen, A.,"An Analytical Method for Calculating the Pure Ridge Resistance Encountered by Ships in First Year Ice Ridges" Thesis for the degree of Doctor of Technology, Helsinki University of Technology, 1979.

Keinonen, A.J., "Ship Performance and Resistance in Ice Ridges", Proc. WEGEMT Seventh Graduate School, Ships and Structures in Ice, Helsinki, 82p., 1983.

Lee,J., Wang, Y.S., "Vessel Transit Through Ridged Ice", Proc. Sixth Conference on Offshore Mechanics and Arctic Engineering, Houston, 1987.

Mäkinen, E., Keinonen, A.J., Laine, A., "Ice Resistance Measurements in Ridges with I/B APU in the Baltic Sea", Proc. POAC '75, Fairbanks, Alaska, 1975.

Tunik, A.L., "Continuous Motion of a Ship in Ridged Ice", Proc. SNAME Ice Tech '84, Calgary, Canada, 1984.

Level Ice Load Mechanics

Daley, C., "Studies in Connection with Updating CASPPR - Group IV. Further Development of the Ship/Ice Collision Probability Model - ASPEN", By Arctec Canada Limited, for Canadian Coast Guard Northern, Transport Canada Report No. TP6274E, Sept. 1986.

Jebaraj, C., Swamidas, A.S.J., Jones, S.D., "Dynamics of Ship/Ice Interaction", Proc. Polar Tech'88, Trondheim, Norway, 1988.

Kashteljan, V.I., "Approximate Determination of the Forces which Destroy the Ice Cover", Problems of the Arctic, No.5, 1960.

Kerr, A.D., "The Bearing Capacity of Floating Ice Plates Subjected to Static of Quasi-Static Loads", Jnl. of Glaciology, Vol17, No.76, 1976.

Mansour, A., Seirig, A., "Computer-based Simulation of Ice-Breaking by Impact", Jnl. of Energy Resources Technology, Vol 105, No.4, 6p., 1983.

Nevel, D.E., "A Semi-Infinite Plate on an Elastic Foundation" CRREL Research Report No. 136, 1965.

Riska, K., "on the Role of Failure Criterion of Ice in Determinating Ice Loads", VTT Ship Lab Report No.7, Espoo, March, 1980.

St. John, J.W., and Minnick, P., "A Design Method For Icebreaker Hull Loads Due to Level Icebreaking" Proc. SNAME Ice Tech '90, Calgary, Canada, March 1990.

Valanto, P., "Experimental and Theoretical Investigation of the Icebreaking Cycle in Two Dimensions", Ph.D. Dissertation, Univ. of California at Bekkeley, August 1989.

Valanto, P., "Experimental Study of the Icebreaking Cycle in 2-D", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Varsta, P., "Modelling of Impact Between Ship Hull and Ice", Proc. POAC '83, Helsinki, Finland, 1983.

Varsta, P., "On the Mechanics of Ice Load on Ships in Level Ice in The Baltic Sea", Thesis for degree of Doctor of Technology, Technical Research Centre of Finland, Publications 11, Espoo, Finland, 1983.

Broken Channel Ice Loads

Kostilainen, V., "Modelling the Ship Performance in Ice-Clogged Channels", Proc. WEGEMT Seventh Graduate School, Ships and Structures in Ice, Helsinki, 44p., 1983.

Mellor, M., "Ship Resistance in Thick Brash Ice", Pub. in Cold Regions Science and Technology, 3(1980) pp. 305-321.

St. John, J.W., Coburn, J.L., Kotras, T.V., "Study of Ice Clogged Channel Clearing Problems", Report by Arctec Inc., Report No. USCG-D-34-81, NTIS no. AD A104 671/3, 1981.

Tsoi, L.G., "Modelling of Vessel's Movement in a Channel Broken Up by Icebreaker", Proc. POAC '83, Helsinki, Finland, 10p., 1983.

Ship -  Ice Floe Interaction Models

Aboulazm, A.F., "Muggeridge, D., " Analytical Investigation of Ship Resistance in Broken or Pack Ice", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Bhat, S.U., Choi, S.K., Wierzbicki, T., Karr, D.G., "Failure Analysis of Impacting Ice Floes", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Hamza, H., Shih, L.Y., "Numerical Models of Pack Ice Interaction with Ships", Proc. IAHR Ice Symp. Sapporo, Japan, 1988.

Koehler, P.E., Jorgensen, L., "Ship Ice Impact Analysis", Proc. Fourth Conference on Offshore Mechanics and Arctic Engineering, Dallas, 1985.

Nawwar, A.M., El-Tahan, H., "A Simple Model for Collision of Floating Bodies", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Popov, Yu.N., et al. Strength of Ships Sailing in Ice, Sudostroyeniye Publishing House, Leningrad, 1967, (English Translation by U.S. Army Foreign Science and Technology Center)

Takekuma, K., Kayo, Y., "Hull Resistance Test in Floe of Synthetic Ice", Mitsubishi Technical Review, Vol 17, No.4, 6p., (in Japanese, w/ Eng. Summary) 1980.

Vinogradov, O.C., "Simulation Methodology of Vessel-Ice Floes Interaction Problem", Proc. Fifth Conference on Offshore Mechanics and Arctic Engineering, Tokyo, 1986.

Level Ice Resistance Measurements and Models

Bacher, H., "Prognosmetoder för fartygs ismotstånd i projektskedet" (Prediction Methods for Ice Resistance during Preliminary Design Stage), Diploma Thesis, Helsinki University of Technology, 1983. (in Swedish)

Carter, D., "Ice Forces on Fixed Structures and Ship Hulls",Report to Transportation Development Centre, Montreal, Report No. TP 7457E,1986.

Carter, D., "Ship Resistance to Continuous Motion in Level Ice",Report to Transportation Development Centre, Montreal, Report No. TP 3679E, 142p.,1983.

Chu, F., "Ice Resistance in Homogeneous Ice Fields", Thesis for the degree of Doctor of Technology, Helsinki University of Technology, 1974.

Edwards, R.Y., et al. "Influence of Major Characteristics of Icebreaker Hulls on their Powering Requirements and Maneuveribility in Ice", Transactions of the SNAME Annual Meeting, Vol.84, New York, 1976.

Edwards, R.Y., et.al. "Full Scale and Model Tests of a Great Lakes Icebreaker", Transactions of the SNAME Annual Meeting, Vol.80, New York, 1972,

Enkvist, E., "On The Ice Resistance Encountered by Ships Operating in the Continuous Mode of Icebreaking", Thesis for the degree of Doctor of Technology, Helsinki University of Technology, 1972.

Enkvist, E., "Survey of Experimental Indications of the Relation between the Submersion and Breaking Components of Level Ice Resistance to Ships", Proc. POAC '83, Helsinki, Finland, 1983.

Ettema, R., Stern, F., Lazaro, J., "Dynamics of Continuous-Mode Icebreaking by a Polar-Class Hull - Part 1: Mean Response", Jnl. of Ship Research, Vol33, No.2, 1989.

Ishibashi, Y., "Estimation of Icebreaking Resistance by Ship Motion Simulation",Proc. SNAME Ice Tech '84, Calgary, Canada, 10p.,1984.

Jones, S.J., " A Review of Ship Performance in Level Ice", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Kashteljan, V.I., et.al., "Ice Resistance to Motion of a Ship" Sudostroenie, Leningrad, 1968.

Kheysin, D.Y., "On the Ice Navigation Speed of Ships in Extremely Solid Ice", The Navigational Qualities of Ships, Ed. by Kheysin, D.Y., Popov, I.N., CRREL Translation TL 417, 1973.

Kotras, T.V., Baird, A.V., Naegle, J.N., "Predicting Ship Performance in Level Ice", Transactions of the SNAME Annual Meeting, New York, 1983.

Lewis, J.W., "Recent Developments in Prediction of Level Ice Resistance", Proc. WEGEMT Seventh Graduate School, Ships and Structures in Ice, Helsinki, 54p., 1983.

Lewis, J.W., and Edwards, R.Y.Jr., "Methods for Predicting Icebreaking and Ice Resistance Characteristics of Icebreakers", Transactions of the SNAME Annual Meeting, Vol.78, New York, 1970.

Lindqvist, G., "A Straightforward Method for Calculation of Ice Resistance of Ships", Proc. POAC '89, Luleå, Sweden, 1989.

Luk, C.H., "A Three-Dimensional Plasticity and Momentum Model for Ship Resistance in Level Ice", Proc. Seventh Conference on Offshore Mechanics and Arctic Engineering, Houston, 1988.

Milano, V.R., "Ship Resistance to Continuous Motion in Ice", Ph.D. Thesis, Stevens Institute of Technology, Hoboken, N.J., 1972.

Milano, V.R., "Ship Resistance to Continuous Motion in Ice", Transactions of the SNAME Annual Meeting, Vol.80, New York, 1972.

Naegle, J.N., "Ice-Resistance Prediction and Motion Simulation for Ships Operating in Continuous Mode of Icebreaking", Ph.D. Thesis, University of Michigan, Ann Arbor, 1980.

Poznyak, I.I., Ionov, B.P., "The Division of Icebreaker Resistance into Components", Proc. SNAME STAR Symp., Ottawa, 1981.

Runeberg, R., "On Steamers for Winter Navigation and Icebreaking", Proc. Institution of Civil Engineers, Vol. XCVII, part III, London, 1888-89.

Shimansky, Yu.,A., "Conditional Guages of the Ice Qualities of Ships", Trans. Arctic Scientific Research Institute, Vol.130, 1938.

Stern, F., Ettema, R., Lazaro, J., "Dynamics of Continuous-Mode Icebreaking by a Polar-Class Hull - Part 2: Spectral Analysis", Jnl. of Ship Research, Vol33, No.3, 1989.

Vance, G.P., "A Scaling System for Vessels in Ice", Proc. SNAME Ice Tech '75, Montreal, Canada, 1975.

Vinogradov, I.V., "Ships for Ice Navigation", Moscow, Oborongiz, 1946.

White, R.M., "Prediction of Icebreaker Capabilities", Royal Institution of Naval Architects, 1961.

General Ice Contact Load Measurements and Models

Ashby, M.F., Palmer, A.C., Thouless, M., Goodman, D.J., Howard, M.W., Hallam, S.D., Murrell, S.A.F., Jones, N., Sanderson, T.J.O., and Ponter, A.R.S., 1986. “Non-simultaneous failure and ice loads on arctic structures”. Proceedings of the Offshore Technology Conference, Paper No. OTC-5127, Houston.

Bhat, S.U., 1990, “Modeling of size effects in ice mechanics using fractal concepts” Jnl. Offshore Mechanics Arctic. Engineering, Vol. 112, pp 370-376.

Blanchet, D., Churcher, A., Fitzpatrick, J., Badra-Blanchet, P., "An Analysis of Observed Failure Mechanisms for Laboratory, First-year and Multi-Year Ice", Proc. IAHR Ice Symp. Sapporo, Japan, 1988.

Choi, Jaewoong, Gunil Park, Youngsung Kim, Kibok Jang, Seungmun Park, Munkeun Ha, Han Yu, Alex Iyerusalimskiy, and James St John. 2009. ICE load monitoring system for large arctic shuttle tanker. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Choi, K., and S. Y. Jeong. 2008. Global ice load prediction for icebreaking vessels. In Proceedings of the eighteenth., ed. J. S. Chung, 638-643.

Choi, K., J. H. Nam, D. H. Kim, and S. Y. Jeong. 2009. Global ICE load estimation on icebreaking vessels under normal operating conditions. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Choi, Kyungsik, and Seong-Yeob Jeong. 2008. Global ice load prediction for icebreaking vessels. Paper presented at 18th 2008 International Offshore and Polar Engineering Conference, ISOPE 2008, July 6, 2008 - July 11, .

Choi, Kyungsik. 2008. Recent trend in design parameters of ice-transiting vessels. International Journal of Offshore and Polar Engineering 18, (4): 282-7.

Coburn, J.L., "A Rational Basis for the Selection of Ice Strengthening Criteria for Ships", Proc. SNAME STAR Symp., Ottawa, 17p., 1981.

Cormeau, A., Maes, M., Jordaan, I.J., Timco, G., "Analysis of Strength Reduction Caused by Systems of Microcracks in Ice", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

Croasdale, K.R., 1980 "Ice forces on fixed, rigid structures" Part II of CRREL Special Report 80-26, IAHR Working Group on Ice Forces on Structures, State-of-the-Art Report, T.Carstens, Editor, June 1980.

Croasdale, K.R., Morgenstern, N.R. and Nuttall, J.B., 1977 "Indentation tests to investigate ice pressures on vertical piers", Jnl. of Glaciology, Vol.19, No.81.

Daley, C. G. 2007. Reanalysis of ice pressure-area relationships. 44, (4): 234-44.

Daley, C. G., 1991 “Ice edge contact - a brittle failure process model” Acta Polytechnica Scandinavica, Mechanical Engineering Series No. 100, 92 pp. Published by the Finnish Academy of Technology, Helsinki.

Daley, C. G.,1992  “Ice edge contact and failure” Cold Regions Science and Technology, 21 (1992) pp 1-23.

Daley, C., and H. Yu. 2009. Assessment of ICE loads on stern regions of ICE class ships. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Daley, C., Howard, D., Frere, D., "Measurement of Ice Forces on Small Vessels", Report by Fleet Technology Limited to Transportation Development Centre, Montreal, Report No. TP 9451E, 1988.

Daley, C., St.John, J.W., Meyer, J.R., Brown, R., Glen, I.F., "Consolidation of Local Ice Impact Pressures Aboard the USCGC Polar Sea", by Arctec Canada Limited and Arctec Incorporated, to Transportation Development Centre, Montreal, Report No. TP8533E, Sept. 1986.

Daley, C.G., 1994 “Compilation of MSI tests results and comparison to ASPPR”, Report by Daley R&E to National Research Council of Canada, Transport Canada Report No. TP 12151E.

Daley, C.G., St. John, J.W., Seibold, F., Bayly, I., "Analysis of Extreme Ice Loads Measured on USCGC Polar Sea", Transactions of the SNAME Annual Meeting, New York, 15p., 1984.

Daley, Claude, Jukka Tuhkuri, and Kaj Riska. 1998. The role of discrete failures in local ice loads. Cold Regions Science and Technology 27, (3): 197-211.

Det norske Veritas Ltd., "State-of-the-Art Methods for Computing Global and Local Loads in Ice Structure Interaction", Report to Public Works Canada, Ottawa, Canada 1986.

Dome Petroleum Limited "Final Report on the Full Scale Measurements of the Ice Impact Loads and Response of the KIGORIAK, August and October, 1981", Internal Report, 1982. (Unpublished)

Edwards, R.Y., et.al. "Results of Full Scale Trials in Ice of CCGS Pierre Radisson", Proc. SNAME STAR Symp., Ottawa, 1981.

El-Tahan, H., Swamidas, A.S.J., Arockiasamy, M., "Impact/Indentation Strength of Iceberg and Artificial Snow Ice", Jnl. of Offshore Mechanics and Arctic Engineering, Vol.110, Feb. 1987.

Enkvist, Ernst, Petri Varsta, and Kaj Riska. 1979. SHIP-ICE INTERACTION. SAE Preprints: 977-1002.

Fransson, L., Olofsson, T., and Sandkvisk , J., 1991 “Observations of the failure process in ice blocks crushed by a flat indentor”, Proceedings POAC ‘91, St. Johns, Canada.

Frederking, R., "Dynamic Ice Forces on an Inclined Structure", IUTAM Symp. on the Physics and Mechanics of Ice, Copenhagen, 1979.

Frederking, R., Blanchet, D., Jordaan, I.J., Kennedy, N.K., Sinha, N.K. and Stander, E., 1990 “Field tests of ice indentation at medium scale, ice island, April 1989” Client Report for Canadian Coast Guard and Transportation Development Centre, By Institute for Research in Construction, National Research Council, Ottawa, October, 1990.

Frederking, R., Timco, G.W., "Field Measurements of the Shear Strength of Columnar-Grained Sea Ice", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

Gagnon, R. 2004. Analysis of laboratory growler impact tests. Cold Regions Science and Technology 39, (1): 1-17.

Gagnon, R. E. 2007. Results of numerical simulations of growler impact tests. Cold Regions Science and Technology 49, (3) (9): 206-14.

Gagnon, R. E., S. J. Jones, R. Frederking, P. A. Spencer, and D. M. Masterson. 2001. Large-scale hull loading of sea ice, lake ice, and ice in tuktoyaktuk harbour. Journal of Offshore Mechanics and Arctic Engineering 123, (4): 159-69.

Gagnon, R.E. 1991 “Heat generation during crushing experiments on freshwater ice” In: Proc. of the 8th Int. Symp. on the Physics and Chemistry of Ice, Hokkaido University Press, Sapporo, pp. 447-455.

Gagnon, Robert, David Cumming, Ron Ritch, Robin Browne, Michelle Johnston, Robert Frederking, Richard McKenna, and Freeman Ralph. 2008. Overview accompaniment for papers on the bergy bit impact trials. Cold Regions Science and Technology 52, (1): 1-6.

Gagnon, Robert. 2003. High-speed video analysis of fracture propagation in a thick-edge-loaded freshwater ice sheet. Canadian Journal of Physics 81, (1-2): 261-9.

German and Milne "Report on Tests to Measure Ice Forces on the Hull of an Icebreaker and the Penetration of an Icebreaker in Ice" MOT Req. T8257-3-0010 for the Canadian Coast Guard, Sept. 1973.

German, J.G., et.al. "Data Collection System on Board the Ice-Strengthened Cargo Vessel M.V. Arctic for Evaluation of Ship Performance in Ice", Proc. SNAME STAR Symp., Ottawa, 1981.

Gershunov, E.M., "Scale Effect in Ice", Cold Regions Science and Technology, Vol 12, Elsevier Sci. Pub., 1986.

Ghoneim, G. A. M., and A. J. Keinonen. 1983. FULL-SCALE IMPACT TESTS OF CANMAR KIGORIAK IN THICK ICE. Paper presented at 7th International Conference on Port and Ocean Engineering under Arctic Conditions, .

Glen, I. F., C. G. Daley, and G. Tam. 1985. ANALYSIS OF THE STRUCTURE OF THE PROPOSED CCG POLAR CLASS 8 ICEBREAKER UNDER EXTREME ICE LOADS. Paper presented at Society of Naval Architects and Marine Engineers - Transactions. Papers Presented at the 93rd Annual Meeting. .

Glen, I.F., Blount, H., "Measurement of Ice Impact Pressures and Loads Onboard CCGS Louis S. ST.Laurent", Proc. Third Conference on Offshore Mechanics and Arctic Engineering, New Orleans, 7p., 1984.

Glen, I.F., et.al. "Results of Full Scale Measurements Aboard CCGS Louis S. St. Laurent During a 1980 Fall Arctic Probe", Report 737C by Arctec Canada Limited to Canadian Coast Guard, April, 1981.

Glen, Ian F., and George Comfort. 1983. ICE IMPACT PRESSURE AND LOAD: INVESTIGATION BY LABORATORY EXPERIMENTS AND SHIP TRIALS. Paper presented at 7th International Conference on Port and Ocean Engineering under Arctic Conditions, .

Grace, I. F., and R. A. Ibrahim. 2008. Modelling and analysis of ship roll oscillations interacting with stationary icebergs. Proceedings of the Institution of Mechanical Engineers, Part C: Journal of Mechanical Engineering Science 222, (10): 1873-84.

Gylden, R., Riska, K.,, "Ice Load Measurements On Board MS Kemira, Winter 1989", Helsinki University of Technology, Laboratory of Naval Architecture and Marine Engineering, Report M-93, Otaniemi, 1989.

Hakala, M., Riska, K., "Results from Statistical Measuring System on board M.V. Arctic 1982", Report by VTT to Canadian Coast Guard Northern, 286p., 1983.

Hoffmann, L., "Impact Forces and Friction Coefficients on the Forebody of the German Polar Research Vessel POLARSTERN", Proc. POAC '85, Narssarssuaq, Greenland, 1985.

Holsapple, K.A., Schmidt, R.M., " On the Scaling of Crater Dimensions, 2. Impact Processes", Jnl. of Geophysical Research, Vol.87, No.B3, 1982.

Ibrahim, R. A., N. G. Chalhoub, and Jeffery Falzarano. 2007. Interaction of ships and ocean structures with ice loads and stochastic ocean waves. Applied Mechanics Reviews 60, (1-6): 246-90.

Jebaraj, C., A. S. J. Swamidas, and L. Y. Shih. 1988. Numerical modelling of ship/ice interaction. Paper presented at Offshore and Arctic Operations Synposium 1989, January 22, 1989 - January 25, .

Jeong, Seong-Yeob, and Kyungsik Choi. 2009. A review on ice resistance prediction formula for icebreaking vessels. Paper presented at 19th (2009) International OFFSHORE AND POLAR ENGINEERING CONFERENCE, June 21, 2009 - June 26, .

Joensuu, A., Riska, K., "Jään ja Rakenteen Välinen Kosketus"(Contact Between Ice and Structure) Helsinki University of Technology, Laboratory of Naval Architecture and Marine Engineering, Report M-88, Otaniemi, 1988. (in Finnish)

Johansson, B.M., Keinonen, A.J., Mercer, B., "Technical Development of an Environmentally Safe Tanker", Proc. SNAME STAR Symp., Ottawa, 1981.

Johnston, M., G. W. Timco, R. Frederking, and M. Miles. 2008. Measuring global impact forces on the CCGS terry fox with an inertial measurement system called MOTAN. 52, (1): 67-82.

Jones, N., "A Note on Ice Scaling", Jnl. of Ship Research, Vol. 30, No. 2, June 1986.

Jones, S. J. 1989. Review of ship performance in level ice. Paper presented at Proceedings of the Eighth International Conference on Offshore Mechanics and Arctic Engineering - 1989, March 19, 1989 - March 23, .

Jordaan, I.J., "Numerical and Finite Element Techniques in Calculation of Ice-Structure Interaction", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

Jordaan,. I.J., Maes, M., Nadreau, J-P, "The Crushing and Clearing of Ice in Fast Spherical Indentation Tests", Proc. Seventh Conference on Offshore Mechanics and Arctic Engineering, Houston, 1988.

Kawakami, S., et al, "Impact Experiments on Ice", Jnl. of Geophysical Research, Vol.88, No. B7, 1983.

Kheysin, D.Ye., Likhomanov, V.A., Kurdyumov, V.A., "Determination of Specific Breakup Energy and Contact Pressures Produced by the Impact of a Solid Against Ice", Symp. on Physical Methods in Studying Snow and Ice, Leningrad, CRREL Translation TL539, 1973.

Kivisild, H. R., D. Blanchet, and C. Revill. 1987. SHIP/ICE INTERACTION PRESSURES AND ENERGIES DURING SHIP RAMMING. Paper presented at Proceedings of the Sixth (1987) International Offshore Mechanics and Arctic Engineering Symposium. .

Koehler, P. E., and L. Jorgensen. 1985. SHIP ICE IMPACT ANALYSIS. Paper presented at Proceedings of the Fourth International Offshore Mechanics and Arctic Engineering Symposium. Presented at the 1985 ASME Energy-Sources Technology Conference & Exhibition. .

Korri, P., Varsta, P., "On the Ice Trial of a 14500 DWT Tanker on the Gulf of Bothnia", Proc. of NSTM-79 Helsinki, Finland.

Korzhavin, K.N. 1962 “Action of ice on engineering structures” USSR Acad. of Sci. Siberian Branch. CRREL Draft Translation No. 260, Hanover, USA, 1971.

Krupina, Nina A., Vladimir A. Likhomanov, Alexey V. Chernov, and Yury P. Gudoshnikov. 2009. Full-scale ice impact study of icebreaker kapitan nikolaev: General description. Paper presented at 19th (2009) International OFFSHORE AND POLAR ENGINEERING CONFERENCE, June 21, 2009 - June 26, .

Kry, P.R., 1978 "A statistical prediction of effective ice crushing stresses on wide structures" Proceedings IAHR Ice Symposium , Luleå, Sweden.

Kujala, P. 1994. On the statistics of ice loads on ship hull in the baltic. Acta Polytechnica Scandinavica, Mechanical Engineering Series(116): 1-98.

Kujala, P. 1996. Semi-empirical evaluation of long term ice loads on a ship hull. Marine Structures 9, (9): 849-71.

Kujala, P., Vuorio, J., "Results and Statistical Analysis of Ice Load Measurements On Board Icebreaker SISU in Winters 1979 to 1985", Winter Navigation Research Board Report 43, 52p., 1986

Kujala,P., "Results of Long Term Ice Load Measurements On Board the Chemical Tanker Kemira in the Baltic Sea During the Winters 1985 to 1988", Winter Navigation Research Board Report 47, 1989.

Leira, B. J., L. Borsheim, and Asme. 2008. ESTIMATION OF ICE LOADS ON A SHIP HULL BASED ON STRAIN MEASUREMENTS. Proceedings of the 27th international conference on offshore mechanics and arctic engineering - 2008, vol 3.

Leira, B. J., L. Brsheim, Espeland, and J. Amdahl. 2009. Assessment of ice-induced loads on ship hulls based on continuous response monitoring. Paper presented at MARSTRUCT 2009, 2nd International Conference on Marine Structures - Analysis and Design of Marine Structures, March 16, 2009 - March 18, .

Levine, G.H., et.al., "A Full Scale Test Program on a Great Lakes Ore Carrier during Winter Operations" Report to the U.S. Maritime Administration by Arctec Inc., July, 1973.

Likhomanov, V.A., "The Strength of Icebreakers and Transport Vessels (According to data from strain-gauge tests)", The Navigational Qualities of Ships, Ed. by Kheysin, D.Y., Popov, I.N., CRREL Translation TL 417, 1973.

Likhomanov, Vladimir A., Nina A. Krupina, Alexey V. Chernov, and Yury P. Gudoshnikov. 2009. Results of definition of the global ice load during in-situ research on impact of the icebreaker "kapitan nikolaev" on various ice formations. Paper presented at 19th (2009) International OFFSHORE AND POLAR ENGINEERING CONFERENCE, June 21, 2009 - June 26, .

Masterson, D. M., and R. M. W. Frederking. 1993. Local contact pressures in ship/ice and structure/ice interactions. Cold Regions Science and Technology 21, (2): 169-85.

Matlock, H., W. P. Dawkins, and J. J. Panak. 1971. Analytical model for ice-structure interaction. Journal of the Engineering Mechanics Division 97, (4) (Aug.): 1083-92.

Matskevitch, D. G. 1997. Eccentric impact of an ice feature: Linearized model. Cold Regions Science and Technology 25, (3): 159-71.

Mejlaender-Larsen, M., and H. Nyseth. 2007. Ice load monitoring. Paper presented at RINA, Royal Institution of Naval Architects International Conference - Design and Construction of Vessels Operating in Low Temperature Environments, May 30, 2007 - May 31, .

Mueller, C., "Strengthening of Hull Structures in Ice", Proc. WEGEMT Seventh Graduate School, Ships and Structures in Ice, Helsinki, 54p., 1983.

Muhonen, A., 1991 “Medium scale indentation tests - PVDF pressure measurements, ice face measurements and Interpretation of crushing video”, Client Report by Helsinki University of Technology, Ship Laboratory, Feb.20, 1991.

Muller, L., Payer, H.G., "Loads on Research Vessel POLARSTERN Under Arctic Conditions", Proc. POAC '87, Fairbanks, Alaska, 1987.

Muller, L., Payer, H.G., Moore, C., "Ice Impact on Ship Hulls",Proc. IAHR Ice Symp. Sapporo, Japan, 1988.

Murray, John J., and Donald S. Spencer. 1997. Simulation model for a turret moored tanker in pack ice cover. Paper presented at Part 1-B (of 6), April 13, 1997 - April 17, .

Phillips, L. D., and H. Tanaka. 1994. Simulation of ship-ice collision dynamics: Ice interface modeling considerations. In , 295-302Computational Mechanics Publ.

Rim, C. W., and T. K. Lee. 2009. Estimation of ICE loads on the bow of an icebreaking research vessel. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Riska, K., Frederking, R., "Ice Load Penetration Modelling", Proc. POAC '87, Fairbanks, Alaska, 1987.

Riska, K., Frederking, R., "Modelling Ice Load During Penetration into Ice", Joint Report by National Research Council of Canada and Technical Research Centre of Finland, (in Canada Transport Canada Report No. TP8237) 57p.,1987.

Riska, K., P. Kujala, and J. Vuorio. 1983. ICE LOAD AND PRESSURE MEASUREMENTS ON BOARD I. B. SISU. Paper presented at 7th International Conference on Port and Ocean Engineering under Arctic Conditions, .

Riska, K., Rantala, H. and Joensuu A., 1990 "Full scale observations of ship-ice contact" Helsinki University of Technology, Laboratory of Naval Architecture and Marine Engineering, Report M-97, 1990.

Riska, K., Varsta, P., "Structural Ice Loads in the Baltic", Proc. WEGEMT Seventh Graduate School, Ships and Structures in Ice, Helsinki, 22p., 1983.

Riska, Kaj, Shotaro Uto, and Jukka Tuhkuri. 2002. Pressure distribution and response of multiplate panels under ice loading. Cold Regions Science and Technology 34, (3): 209-25.

Riska, Kaj. 1987. ON THE MECHANICS OF THE RAMMING INTERACTION BETWEEN A SHIP AND A MASSIVE ICE FLOE. Publications - Technical Research Centre of Finland(43).

Ritch, Ron, R. Frederking, M. Johnston, Robin Browne, and Freeman Ralph. 2008. Local ice pressures measured on a strain gauge panel during the CCGS terry fox bergy bit impact study. Cold Regions Science and Technology 52, (1): 29-49.

Rychnovsky, R., "Deep Ice Penetrator", Proc. POAC '87, Fairbanks, Alaska, 1987.

Sanderson, T. J. O., Ice Mechanics: Risk to Offshore Structures, Graham & Trotman, London, UK, 1988.

Sandwell., 1990 “1990 ice indentator tests - Field test report and executive summary” Report of Project 112390 by Sandwell Inc., Calgary, Alberta, Vol. I and II November 1990.

Sandwell., 1992 “Reduction and analysis of 1990 and 1989 Hobson’s Choice ice indentation tests data” Final Report, Project 112588 by Sandwell Inc., Calgary, Alberta, to Conoco Inc. Exxon Prod. Res. Co., Mobil R and D Corp. and National Research Council of Canada, August 1992.

Sandwell., 1993 “Medium scale uniform pressure tests on first-year sea ice at Resolute Bay, N.W.T. 1993” Draft Final Report, Project 113077 by Sandwell Inc., Calgary, Alberta, to National Research Council, Institute for Mechanical Engineering, Ottawa, Vol. I and II, August 1993.

Sawamura, Junji, Kaj Riska, and Torgeir Moan. 2009. Numerical simulation of breaking patterns in level ice at ship's bow. Paper presented at 19th (2009) International OFFSHORE AND POLAR ENGINEERING CONFERENCE, June 21, 2009 - June 26, .

Sinha, N.K., 1989, “Kinetics of microcracking and dilation in polycrystalline ice” in Ice-Structure Interaction, Jones, McKenna, Tillotson and Jordaan (Eds), IUTAM/IAHR Symposium, St. John’s, Canada, 1989, publ. by Springer-Verlag, 1991

Sinha, N.K., 1991 “In situ multi-year sea ice strength using NRCC borehole indentor”, Proc. of the 10th Intl. Conf. O.M.A.E.

Sodhi, D. S., L. Button, R. Boetes, and M. Arockiasamy. 1978. ESTIMATION OF ICE FORCES ON THE HULL OF M. V. ARCTIC EXPLORER BY STRAIN GAUGE MEASUREMENTS. 1, : 475-84.

Spencer, D., and W. D. Molyneux. 2009. Predicting pack ICE loads on moored vessels. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

St. John, J.W., Daley, C.G., "Shipboard Measurements of Ice Pressures in the Bering, Chukchi and Beaufort Seas", Proc. Third Conference on Offshore Mechanics and Arctic Engineering, New Orleans, 7p., 1984.

Swearengen, J.C., "Deformation and Fracture of Arctic Sea Ice During Kinetic Energy Penetration", Proc. Seventh Conference on Offshore Mechanics and Arctic Engineering, Houston, 1988.

Temarel, P. 2007. Loads on ships and offshore structures. Paper presented at 26th International Conference on Offshore Mechanics and Arctic Engineering 2007, OMAE2007, June 10, 2007 - June 15, .

Timco, G.W., "Confined Compression Tests: Outlining the Failure Envelope of Columnar Sea Ice", Cold Regions Science and Technology, Vol 12, Elsevier Sci. Pub., 1986.

Timco, G.W., "Indentation and Penetration of Edge-Loaded Freshwater Ice Sheets in the Brittle Range", Proc. Fifth Conference on Offshore Mechanics and Arctic Engineering, Tokyo, 1986.

Timco, G.W., 1986 "Indentation and penetration of edge-loaded freshwater ice sheets in the brittle range", Proc. Fifth Conference on Offshore Mechanics and Arctic Engineering, Tokyo.

Timco, G.W., and Croasdale, K.R, “How Well Can We Predict Ice Loads?”  Proceedings 18th International Symposium on Ice, IAHR’06  Vol. 1, pp167-174, Sapporo, Japan, 2006.

Timofeev, Oleg Ya, and Nina A. Krupina. 2002. Calculation of parameters of local ice loads on data of full scale tests of ice belt structures. Paper presented at Proceedings of the Twelfth (2002) International Offshore and Polar Engineering Conference, May 26, 2002 - May 31, .

Tuhkuri, J. 1996 “Experimental investigations and computational fracture mechanics modelling of brittle ice fragmentation” Acta Polytechnica Scandinavica, Mechanical Engineering Series No. 120, Helsinki, 105p.

Tuhkuri, J., 1994 “Analysis of ice fragmentation process from measured particle size distributions of crushed ice” Cold Regions Science and Technology, 23: 69-82.

Tuhkuri, J., 1995 “Experimental observations of brittle failure process of ice and ice-structure contact” Cold Regions Science and Technology, 23: 265-278.

Tunik, A. 2000. Safe speeds of navigation in ice as criteria of operational risk. 10, (4): 285-91.

Tunik, A.L., "Dynamic Ice Loads on a Ship", Proc. IAHR Ice Symp. Hamburg, West Germany, 17p., 1984.

Varsta, P. 1983. MODELLING OF IMPACT BETWEEN SHIP HULL AND ICE. Paper presented at 7th International Conference on Port and Ocean Engineering under Arctic Conditions, .

Varsta, P., and Riska, K., 1977 “Failure process of ice edge, caused by impact with a ship’s side”, in Ice, Ships and Winter Navigation, Symp. in Oulu Univ, Oulu, Dec. 1977.

Varsta, Petri. 1983. ON THE MECHANICS OF ICE LOAD ON SHIPS IN LEVEL ICE IN THE BALTIC SEA. Publications - Technical Research Centre of Finland(23).

Vaughan, Henry. 1986. GLOBAL RESPONSE OF ICEBREAKERS RAMMING HEAVY ICE. Paper presented at Proceedings - Eleventh Ship Technology and Research (STAR) Symposium, .

Vuorio, J., Riska, K., Varsta, P., "Long Term Measurements of Ice Pressure and Ice Induced Stresses on the Icebreaker Sisu in Winter 1978", Winter Navigation Research Board Report 28, 50p., 1979.

Wang, Jungyong, Ayhan Akinturk, and Neil Bose. 2006. Numerical prediction of model podded propeller-ice interaction loads. Paper presented at 25TH International Conference on Offshore Mechanics and Arctic Engineering, OMAE 2006, June 4, 2006 - June 9, .

Wang, Zhiguo, Derek B. Muggeridge, Stephen J. Jones, and A. S. J. Swamidas. 1997. Numerical simulation of ridge and sheet ice loads on a proposed faceted conical structure. Paper presented at Part 1-B (of 6), April 13, 1997 - April 17, .

Woolgar, Reeni C., and D. Bruce Colbourne. 2010. Effects of hull-ice friction coefficient on predictions of pack ice forces for moored offshore vessels. Ocean Engineering 37, (2-3): 296-303.

Zahn, P. B., and P. V. Minnick. 1988. LOCAL IMPACT PRESSURES DUE TO FIRST YEAR ICE IN THE MARGINAL ICE ZONE. Paper presented at OMAE 1988 Houston.

Design of Ice Class Ships and Structures

API, 1993, American Petroleum Institute Recommended Practice 2A, Recommended Practice for Planning, Designing, and Constructing Fixed Offshore Platform – LRFD, July 1, 1993

API, 1995, American Petroleum Institute Recommended Practice 2N, Recommended Practice for Planning, Designing and Constructing Structures and Pipelines for Arctic Conditions, December 1, 1995

Buetzow, Mark R., Savas Athanasiadis, Maria-Celia Ximenes, Tom G. Cinelli, and Rocco Lofaro Jr. 1998. Design of hibernia shuttle tanker M.T. kometik. Paper presented at Proceedings of the 1997 105th Annual Meeting, October 15, 1997 - October 17, .

Canadian Standards Association S471-92, General Requirements, Design Criteria, the Environment, and Loads – Part 1 of the Code for the Design, Construction and Installation of Fixed Offshore Structures

Choi, Kyungsik, and Woo-Ram Lee. 2006. Recent trend in design parameters of ice-transiting vessels. Paper presented at 16th 2006 International Offshore and Polar Engineering Conference, ISOPE 2006, May 28, 2006 - June 2, .

Conachey, R. M., C. Baker, G. Wang, R. Miguez, and K. Lilley. 2007. Review of american bureau of shipping's guide for vessels operating in low temperature environments. Paper presented at RINA, Royal Institution of Naval Architects International Conference - Design and Construction of Vessels Operating in Low Temperature Environments, May 30, 2007 - May 31, .

Daley, C., A. Kendrick, and Asme. 2008. DIRECT DESIGN OF LARGE ICE CLASS SHIPS WITH EMPHASIS ON THE MIDBODY ICE BELT. Proceedings of the 27th international conference on offshore mechanics and arctic engineering - 2008, vol 3.

Daley, C., A. Kendrick, and Asme. 2008. DIRECT DESIGN OF LARGE ICE CLASS SHIPS WITH EMPHASIS ON THE MIDBODY ICE BELT. Proceedings of the 27th international conference on offshore mechanics and arctic engineering - 2008, vol 3.

Daley, C., A. Kendrick, H. Yu, and B. -J Noh. 2007. Structural design of high ice class LNG tankers. Paper presented at RINA, Royal Institution of Naval Architects International Conference - Design and Construction of Vessels Operating in Low Temperature Environments, May 30, 2007 - May 31, .

Daley, C., and H. Yu. 2009. Assessment of ICE loads on stern regions of ICE class ships. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Ghoneim, G. A., and Bor-Feng Peng. 2009. Application of recent ice class requirements for arctic EP offshore structures. Paper presented at 19th (2009) International OFFSHORE AND POLAR ENGINEERING CONFERENCE, June 21, 2009 - June 26, .

Ishimaru, J., K. Tsumura, K. Sato, T. Ishida, and M. Oka. 2007. Practical design of LNG carriers in low temperature environments. Paper presented at RINA, Royal Institution of Naval Architects International Conference - Design and Construction of Vessels Operating in Low Temperature Environments, May 30, 2007 - May 31, .

Kendrick, A., C. Daley, V. Santos-Pedro, and R. Hayward. 2007. Classification of polar ships. Paper presented at RINA, Royal Institution of Naval Architects International Conference - Design and Construction of Vessels Operating in Low Temperature Environments, May 30, 2007 - May 31, .

Kim, Hyun-Soo, Mun-Keun Ha, Dang Ahn, Soo-Hyung Kim, and Jong-Woo Park. 2005. Development of 115K tanker adopting baltic ice class 1A. Paper presented at 15th International Offrshore and Polar Engineering Conference, ISOPE-2005, June 19, 2005 - June 24, .

Likhomanov, V., O. Timofeev, I. Stepanov, V. Kashtelyan, and L. Tsoy. 1998. Ice passport for icebreaker 'pierre radisson' and passport's concept: Further development. Paper presented at Part 2 (of 4), May 24, 1998 - May 29, .

Mahmood, Mutmaz, and Arturo Revenga. 2006. Design aspects of winterized and arctic LNG carriers - A classification perspective. Paper presented at 25TH International Conference on Offshore Mechanics and Arctic Engineering, OMAE 2006, June 4, 2006 - June 9, .

Sandwell 1198 “Comparison of International Codes for Ice Loads on Offshore Structures” PERD/CHC Report 11-20,Prepared by:Sandwell Engineering Inc. and Central Marine Research & Design Institute

SNiP 2.01.07-85, Loads and influences

SNiP 2.06.01-86, Marine structures, general design provisions

SNiP 2.06.04-82*, Loads and influences on marine structures (from waves, ice and vessels)

St. John, James W., and James R. Meyer. 1986. STRUCTURAL DESIGN METHODS FOR SURFACE SHIPS OPERATING AT THE ICE EDGE. Naval Engineers Journal 98, (3): 88-94.

Tunik, Alfred. 2000. Safe speeds of navigation in ice as criteria of operational risk. International Journal of Offshore and Polar Engineering 10, (4): 285-91.

Seligman, Alain R., and Peter Zahn. 1985. DESIGN OF AN ICEBREAKING OFFSHORE SUPPLY VESSEL. Paper presented at Civil Engineering in the Arctic Offshore, .

Shemendjuk, G. P., V. A. Babtsev, and O. I. Bratukhin. 1988. DESIGN OF HULL STRUCTURES FOR ICE-NAVIGATING SHIPS. Paper presented at OMAE 1988 Houston, .

Wang, Ge, Roger Basu, Dipak Chavda, Shewen Liu, Myong-Sup Lee, Yong-Suk Suh, and Yoo-Jhung Han. 2005. Rationalization of design of side structure of ice-strengthened tankers. International Journal of Offshore and Polar Engineering 15, (3): 210-4.

Daley, C., A. Kendrick, and M. Pavic. 2007. Comparative study of ship structural regulations. Paper presented at RINA, Royal Institution of Naval Architects International Conference - Developments in Classification and International Regulations, January 24 - 25,2007.

Daley, C., A. Kendrick, H. Yu, and B. -J Noh. 2007. Structural design of high ice class LNG tankers. Paper presented at RINA, Royal Institution of Naval Architects International Conference - Design and Construction of Vessels Operating in Low Temperature Environments, May 30, 2007 - May 31, .

Wang, Ge, and Christopher J. Wiernicki. 2006. Using nonlinear finite element method to design ship structures for ice loads. Paper presented at 2005 Society of Naval Architects and Marine Engineers Annual Meeting, October 19, 2005 - October 21, .

# Appendix 1C: Material and Structural STREngth of Ice Class ships

References related to models of material and structural behavior of ships subjected to ice loads. Topics primarily include response of plating, framing and hull girder, but also include more general structural considerations and special considerations.

Material Behavior of Steel

Tomin, M., M. M. Zadeh, A. Cormeau, and M. Nessim. 1988. Fracture control for arctic vessels and structures. Paper presented at Probabilistic Methods in Civil Engineering, Proceedings of the 5th ASCE Specialty Conference, May 25, 1988 - May 27, .

Structural Strength (concepts)

Abraham, J., and C. Daley. 2009. Stability of flat bar stiffeners under lateral patch loads. Paper presented at MARSTRUCT 2009, 2nd International Conference on Marine Structures - Analysis and Design of Marine Structures, March 16, 2009 - March 18, .

Abraham, J., and C. G. Daley. 2009. Load sharing in a grillage subject to ICE loading. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Babtsev, Victor A., Alexander V. Kultsep, and Vladimir N. Triaskin. 1997. Investigation of ice load-carrying capacity of ship hull ice belt framing in elastic-plastic stage. Paper presented at Part 1-B (of 6), April 13, 1997 - April 17, .

Belenkiy, Leonid M., Yury N. Raskin, and Alfred Tunik. 2008. Plastic strength criterion for design of laterally loaded ship plates. Marine Technology 45, (2): 63-7.

Belenkiy, Leonid, Yury Raskin, and Alfred Tunik. 2005. Limit state design of ship grillages under local loads. International Shipbuilding Progress 52, (2): 169-82.

Daley, C., and A. Bansal. 2009. Discussion of plastic capacity of plating subject to patch loads. Paper presented at MARSTRUCT 2009, 2nd International Conference on Marine Structures - Analysis and Design of Marine Structures, March 16, 2009 - March 18, .

Hindley, R. J., and E. Ranki. 2009. Learning from ICE damage investigations for future arctic ships. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Hong, Lin, and Jrgen Amdahl. 2007. Plastic design of laterally patch loaded plates for ships. Marine Structures 20, (3): 124-42.

Jia, Zhibin, Anders Ulfvarson, Jonas W. Ringsberg, and Junbo Jia. 2009. A return period based plastic design approach for ice loaded side-shell/bow structures. Marine Structures 22, (3): 438-56.

Kim, B. J., S. K. Park, B. S. Jang, and J. K. Paik. 2009. Crushing behaviour of steel plated structures in cold temperature: Experimental and numerical studies. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Liu, S., H. Yu, and R. Basu. 2009. Plastic grillage analysis for ship structure under ICE loads. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Moshaiov, A., and M. R. Steinhilber. 1991. Theoretical assessment of light structural damage due to ship collision with ice. Journal of Offshore Mechanics and Arctic Engineering 113, (1): 61-6.

Nyseth, Havard, and Gabriel Holtsmark. 2006. Analytical plastic capacity formulation for plates subject to ice loads and similar types of patch loadings. Paper presented at 25TH International Conference on Offshore Mechanics and Arctic Engineering, OMAE 2006, June 4, 2006 - June 9, .

Oh, Hoonkyu, Whasoo Kim, and Jaemyung Lee. 2009. Safety of membrane type cargo containment system in LNG carrier under accidental iceberg collision. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Tuhkuri, Jukka. 1996. Sandwich structures under ice loading - theoretical and experimental investigations. Marine Structures 9, (2): 259-80, http://dx.doi.org/10.1016/0951-8339(94)00011-G.

Tunik, Alfred L. 1983. STRENGTH STANDARD FOR ARCTIC SHIPS. Paper presented at 7th International Conference on Port and Ocean Engineering under Arctic Conditions, .

Wiernicki, Christopher J. 1987. DAMAGE TO SHIP PLATING DUE TO ICE IMPACT LOADS. Marine Technology 24, (1): 43-58.

Structural Strength (Numerical Modeling)

Glen, I. F., C. G. Daley, and G. Tam. 1985. ANALYSIS OF THE STRUCTURE OF THE PROPOSED CCG POLAR CLASS 8 ICEBREAKER UNDER EXTREME ICE LOADS. Paper presented at Society of Naval Architects and Marine Engineers - Transactions. Papers Presented at the 93rd Annual Meeting. .

Hakala, Matti K. 1979. NONLINEAR FINITE ELEMENT ANALYSIS OF AN ICE-STRENGTHENED SHIP SHELL STRUCTURE. Computers and Structures 12, (4): 541-7.

Han, S., J. -Y Lee, Y. -I Park, and J. Che. 2008. Structural risk analysis of an NO96 membrane-type liquified natural gas carrier in baltic ice operation. Proceedings of the Institution of Mechanical Engineers Part M: Journal of Engineering for the Maritime Environment 222, (4): 179-94.

Kwak, Myo-Jung, Joong-Hyo Choi, Jae-Hyung Park, and Je-Hyouk Woo. 2009. Strength assessment for bow structure of arctic tanker (107K) under ship-ICE interaction. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Lee, S. G., J. S. Lee, Y. H. Baek, J. K. Paik, and B. J. Kim. 2009. Structural safety assessment in membrane-type CCS in LNGC under iceberg collisions. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Paik, J. K. 2010. Some recent advances and future trends in nonlinear structural mechanics for ships and offshore structures. Marine Technology 47, (1): 17-26 (accessed 22 March 2010).

Paik, J. K., and J. K. Seo. 2009. Nonlinear finite element method models for ultimate strength analysis of steel stiffened-plate structures under combined biaxial compression and lateral pressure actions-part II: Stiffened panels. 47, (8-9): 998-1007.

Su, Biao, Kaj Riska, and Torgeir Moan. 2010. A numerical method for the prediction of ship performance in level ice. Cold Regions Science and Technology 60, (3) (3): 177-88.

# Appendix 1D: RISK and Hazard Assessment

References related to ..............

Risk and Transit Models

Gill, R.J., et al. "A Ship Transit Model for Passage Through Ice and its Application to the Labrador Area", Proc. SNAME STAR Symp., Ottawa, 1981.

Kämäräinen, J., "Icebreaker Tanker Transit Simulation Model", Proc. Polar Tech'88, Helsinki, Finland, 1986.

Lawson, W.A., Hadley, R.D., "Evaluation of Marine Oil Transportation Systems for Arctic Areas", Proc. Polar Tech'88, Trondheim, Norway, 1988.

Daley, C.G., Ferregut, C., "Structural Risk Model of Arctic Shipping", IUTAM/IAHR Symp. on Ice-Structure Interaction, St. John's, Canada, 1989.

Remote Sensing Technology

Bobylev, L. P., E. V. Zabolotskikh, L. M. Mitnik, and O. M. Johannessenn. 2008. Neural-network based algorithm for ice concentration retrievals from satellite passive microwave data.

Forsberg, R., H. Skourup, O. Andersen, S. Laxon, A. Ridout, A. Braun, J. Johannessen, F. Siegismund, C. C. Tscherning, and P. Knudsen. 2007. Combination of spaceborne, airborne and surface gravity in support of arctic ocean sea-ice and MDT mapping.

Hong, S. 2010. Detection of small-scale roughness and refractive index of sea ice in passive satellite microwave remote sensing. Remote Sensing of Environment 114, (5): 1136-40.

Knight, Rosemary. 2001. GROUND PENETRATING RADAR FOR ENVIRONMENTAL APPLICATIONS. Annual Review of Earth and Planetary Sciences 29, (1) (05/01): 229-55, http://dx.doi.org.qe2a-proxy.mun.ca/10.1146/annurev.earth.29.1.229.

Lane, K., D. Power, I. Chakraborty, J. Youden, C. Randell, J. McClintock, and D. Flett. 2002. RADARSAT-1 synthetic aperture radar iceberg detection performance ADRO-2 A223.

Lu, P., Z. Li, B. Cheng, R. Lei, and R. Zhang. 2010. Sea ice surface features in arctic summer 2008: Aerial observations. Remote Sensing of Environment 114, (4): 693-9 (accessed 23 March 2010).

Meldrum, D. 2009. Review of satellite data telecommunication systems. Global Ocean Observing System, SOT-V-Doc- I-5.2(2), http://www.ioc-goos.org/index.php?option=com_oe&task=viewDocumentRecord&docID=3675.

Melsheimer, C., G. Heygster, and L. Toudal Pedersen. 2008. Integrated retrieval of surface and atmospheric parameters over the arctic from AMSR-E satellite microwave radiometer data using inverse methods.

Nolin, A. W., F. M. Fetterer, and T. A. Scambos. 2002. Surface roughness characterizations of sea ice and ice sheets: Case studies with MISR data. IEEE Transactions on Geoscience and Remote Sensing 40, (7): 1605-15.

NSIDC. All about sea ice - studying. in NSIDC [database online]. Available from http://nsidc.org/seaice/study/index.html.

Otto, T., J. Lu, and M. Chandra. 2009. Polarisation basis transformation of weather radar measurements in the power domain. Advances in Radio Science 7: 279-84.

Paden, J. 2003. Bistatic/Monostatic synthetic aperture radar for ice sheet measurements. University of Kansas: CReSIS, CReSIS TR 118.

Quincey, D. J., and A. Luckman. 2009. Progress in satellite remote sensing of ice sheets. Progress in Physical Geography 33, (4) (/8/1): 547-67, http://ppg.sagepub.com.qe2a-proxy.mun.ca/cgi/content/abstract/33/4/547.

Squire, V. A., J. P. Dugan, P. Wadhams, P. J. Rottier, and A. K. Liu. 1995. Of ocean waves and sea ice. Annual Review of Fluid Mechanics 27, (1): 115-68.

Tedesco, M., and P. S. Narvekar. 2010. Assessment of the NASA AMSR-E SWE product. IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing 3, (1): 141-59.

Williams, T. D., and V. A. Squire. 2010. On the estimation of ice thickness from scattering observations. Dynamics of Atmospheres and Oceans 49, (2-3): 215-33.

Ice databases

Hill, B. T. 2010. Ice charts & ship / iceberg database. in NRC-IOT [database online]. Available from http://researchers.imd.nrc.ca/~hillb/icedb/ice/.

Kubat, I., and G. W. Timco. 2005. NRC marine icing database. Paper presented at Proceedings of the 11th International Workshop on Atmospheric Icing of Structures, Montreal.

Rudkin, P. 2005. PERD comprehensive iceberg management database. National Research Council of Canada - Canadian Hydraulics Centre, PERD/CHC Report 20-72.

IIP. 2010. International ice patrol (IIP) iceberg sightings database. in NSIDC [database online]. Available from http://nsidc.org/data/docs/noaa/g00807_international_iceberg_sightings/index.html.

Verbit, S., G. Comfort, and G. W. Timco. 2006. Development of a database for iceberg sightings off Canada's east coast. Paper presented at Proceedings 18th International Symposium on ice, Sapporo, Japan.

# Appendix 1E: Regulatory and Other ISSUES

References related to

Robert N. Dawson and John W. Grainge, Proposed Design Criteria for Wastewater Lagoons in Arctic and Sub-Arctic Regions Source: Journal (Water Pollution Control Federation), Vol. 41, No. 2, Part I (Feb., 1969), pp. 237-246 Published by: Water Environment Federation

Donn K. Haglund, Maritime Transport in Support of Arctic Resource Development, Cold Regions Science and Technology, 7 (1983) 231—249 Elsevier Science Publishers B.V., Amsterdam

Canadian Coast Guard, 2007, Search and Rescue Needs Analysis

…..

# Appendix 1f: Bibliography on ALL TOPICS

Aboulazm, A.F., "Muggeridge, D., " Analytical Investigation of Ship Resistance in Broken or Pack Ice", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Abraham, J., and C. Daley. 2009. Stability of flat bar stiffeners under lateral patch loads. Paper presented at MARSTRUCT 2009, 2nd International Conference on Marine Structures - Analysis and Design of Marine Structures, March 16, 2009 - March 18, .

Abraham, J., and C. G. Daley. 2009. Load sharing in a grillage subject to ICE loading. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Ackley, S.F., Hibler, W.B., Kugrzuk, F., Kovacs, A., Weeks, W.F., 1974. Thickness and roughness variations of arctic multi-year ice. Oceans '74, IEEE International Conference on Engineering in the Ocean Environment, vol. 1, pp. 109–117. Halifax, NS, Canada.

Airaksinen, K., 1974. Free beam tests and friction tests at Pond Inlet, NWT. Polarforschung J44, 71–75.

Anderson, D.L., 1958. Preliminary results and review of sea ice elasticity and related studies. Transactions of the Engineering Institute of Canada 2, 116–122.

API, 1993, American Petroleum Institute Recommended Practice 2A, Recommended Practice for Planning, Designing, and Constructing Fixed Offshore Platform – LRFD, July 1, 1993

API, 1995, American Petroleum Institute Recommended Practice 2N, Recommended Practice for Planning, Designing and Constructing Structures and Pipelines for Arctic Conditions, December 1, 1995

Ashby, M.F., Palmer, A.C., Thouless, M., Goodman, D.J., Howard, M.W., Hallam, S.D., Murrell, S.A.F., Jones, N., Sanderson, T.J.O., and Ponter, A.R.S., 1986. “Non-simultaneous failure and ice loads on arctic structures”. Proceedings of the Offshore Technology Conference, Paper No. OTC-5127, Houston.

Ashton, G.D. (Ed.), 1986. River and Lake Ice Engineering. Water Resources Publication, Littleton, CO, USA.

Assur, A., 1958. Composition of sea ice and its tensile strength. Arctic Sea Ice, vol. 598. U.S. Nat. Acad. Sci. Publication, pp. 106–138.

Babtsev, Victor A., Alexander V. Kultsep, and Vladimir N. Triaskin. 1997. Investigation of ice load-carrying capacity of ship hull ice belt framing in elastic-plastic stage. Paper presented at Part 1-B (of 6), April 13, 1997 - April 17, .

Bacher, H., "Prognosmetoder för fartygs ismotstånd i projektskedet" (Prediction Methods for Ice Resistance during Preliminary Design Stage), Diploma Thesis, Helsinki University of Technology, 1983. (in Swedish)

Barker, A., Timco, G.W., 2003. The friction coefficient of a large ice block on a sand/ gravel beach. Proceedings 12th Workshop on the Hydraulics of Ice Covered Rivers, CRIPE'03, Edmonton, Canada.

Barry, Gene, Torkild Carstens, Kenneth Croasdale, Robert Frederking, and Tom Brown. 1991. Ice break-up model for northumberland strait. Paper presented at 11th International Conference on Port and Ocean Engineering under Arctic Conditions - POAC'91, September 24, 1991 - September 28, .

Belenkiy, Leonid M., Yury N. Raskin, and Alfred Tunik. 2008. Plastic strength criterion for design of laterally loaded ship plates. Marine Technology 45, (2): 63-7.

Belenkiy, Leonid, Yury Raskin, and Alfred Tunik. 2005. Limit state design of ship grillages under local loads. International Shipbuilding Progress 52, (2): 169-82.

Bhat, S.U., 1990, “Modeling of size effects in ice mechanics using fractal concepts” Jnl. Offshore Mechanics Arctic. Engineering, Vol. 112, pp 370-376.

Bhat, S.U., Choi, S.K., Wierzbicki, T., Karr, D.G., "Failure Analysis of Impacting Ice Floes", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Bjerkelund, C.A., Lapp, D.J., Ramseier, R.O., Sinha, N.K., 1985. The texture and fabric of second-year sea cover at Mould Bay, Prince Patrick Island, NWT, April 1983. Proceedings 1985 IEEE International Geoscience and Remote Sensing Symposium, IGARSS'85, vol. 1, pp. 426–431. Amherst, MA, USA.

Blanchet, D., Abdelnour, R., Comfort, G., 1997. Mechanical properties of first-year sea ice at Tarsiut Island. Journal of Cold Regions Engineering 1 (1), 59–83.

Blanchet, D., Churcher, A., Fitzpatrick, J., Badra-Blanchet, P., "An Analysis of Observed Failure Mechanisms for Laboratory, First-year and Multi-Year Ice", Proc. IAHR Ice Symp. Sapporo, Japan, 1988.

Blanchet, D., Kivisild, H.R., Grinstead, J., "Equations for Local Ice Energies During Ship Ramming", Presented at Proc. POAC '87, Fairbanks, Alaska, 1987.

Bobylev, L. P., E. V. Zabolotskikh, L. M. Mitnik, and O. M. Johannessenn. 2008. Neural-network based algorithm for ice concentration retrievals from satellite passive microwave data.

Brigham, L., Cerne, M., Cole, K., et al. AMSA Bering Strait Region Case Study

Brown, J.H., 1963. Elasticity and strength of sea ice. In: Kingery, W.K. (Ed.), Ice and Snow. MIT Press, Cambridge, MA, pp. 79–106.

Brown, T.G., Määttänen, M., 2009. Comparison of Kemi-I and Confederation Bridge cone ice load measurement results. Cold Regions Science and Technology 55, 3–13.

Buetzow, Mark R., Savas Athanasiadis, Maria-Celia Ximenes, Tom G. Cinelli, and Rocco Lofaro Jr. 1998. Design of hibernia shuttle tanker M.T. kometik. Paper presented at Proceedings of the 1997 105th Annual Meeting, October 15, 1997 - October 17, .

Butkovich, T.R., 1956. Strength studies of sea ice. Snow Ice and Permafrost Research Establishment (SIPRE). U.S. Army Research Report RR20. Wilmette, Ill., USA.

Butkovich, T.R., 1959. On the mechanical properties of sea ice, Thule, Greenland, 1957. Snow Ice and Permafrost Research Establishment (SIPRE). U.S. Army Research Report RR54. Wilmetre, IL., USA.

Canadian Coast Guard, 2007, Search and Rescue Needs Analysis

Canadian Standards Association S471-92, General Requirements, Design Criteria, the Environment, and Loads – Part 1 of the Code for the Design, Construction and Installation of Fixed Offshore Structures

Carter, D., "Ice Forces on Fixed Structures and Ship Hulls",Report to Transportation Development Centre, Montreal, Report No. TP 7457E,1986.

Carter, D., "Ship Resistance to Continuous Motion in Level Ice",Report to Transportation Development Centre, Montreal, Report No. TP 3679E, 142p.,1983.

Chen, A.C.T., Lee, J., 1986. Large-scale ice strength tests at slow strain rates. Proc. Offshore Mechanics and Arctic Engineering (OMAE), vol. 4, pp. 374–378. Tokyo, Japan.

Cherepanov, N.V., 1966. Structure of sea ice of great thickness. Problems of Arctic Ice Research 267, 13–18.

Cherepanov, N.V., 1974. Classification of ice of natural water bodies. Oceans '74, IEEE International Conference on Engineering in the Ocean Environment, vol. 1, pp. 97–101. Halifax, NS, Canada.

Choi, Jaewoong, Gunil Park, Youngsung Kim, Kibok Jang, Seungmun Park, Munkeun Ha, Han Yu, Alex Iyerusalimskiy, and James St John. 2009. ICE load monitoring system for large arctic shuttle tanker. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Choi, K., and S. Y. Jeong. 2008. Global ice load prediction for icebreaking vessels. In Proceedings of the eighteenth., ed. J. S. Chung, 638-643.

Choi, K., J. H. Nam, D. H. Kim, and S. Y. Jeong. 2009. Global ICE load estimation on icebreaking vessels under normal operating conditions. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Choi, Kyungsik, and Seong-Yeob Jeong. 2008. Global ice load prediction for icebreaking vessels. Paper presented at 18th 2008 International Offshore and Polar Engineering Conference, ISOPE 2008, July 6, 2008 - July 11, .

Choi, Kyungsik, and Woo-Ram Lee. 2006. Recent trend in design parameters of ice-transiting vessels. Paper presented at 16th 2006 International Offshore and Polar Engineering Conference, ISOPE 2006, May 28, 2006 - June 2, .

Choi, Kyungsik. 2008. Recent trend in design parameters of ice-transiting vessels. International Journal of Offshore and Polar Engineering 18, (4): 282-7.

Chu, F., "Ice Resistance in Homogeneous Ice Fields", Thesis for the degree of Doctor of Technology, Helsinki University of Technology, 1974.

Coburn, J.L., "A Rational Basis for the Selection of Ice Strengthening Criteria for Ships", Proc. SNAME STAR Symp., Ottawa, 17p., 1981.

Cole, D.M., Gould, L.D., Burch, W.D., 1985. A system for mounting end caps on ice specimens. Journal of Glaciology 31 (109), 362–365.

Cole, D.M., Shapiro, L.H., 1998. Observations of brine drainage networks and microstructure of first-year sea ice. Journal of Geophysical Research C: Oceans 103 (C10), 21,739–21,750.

Conachey, R. M., C. Baker, G. Wang, R. Miguez, and K. Lilley. 2007. Review of american bureau of shipping's guide for vessels operating in low temperature environments. Paper presented at RINA, Royal Institution of Naval Architects International Conference - Design and Construction of Vessels Operating in Low Temperature Environments, May 30, 2007 - May 31, .

Coon, M.D., "Mechanical Behavior of Compacted Arctic Ice Floes", Offshore Technology Conference, Dallas Tx., 1972.

Cormeau, A., Maes, M., Jordaan, I.J., Timco, G., "Analysis of Strength Reduction Caused by Systems of Microcracks in Ice", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

Cox, G.F.N., Richter, J.A., Weeks, W.F., Mellor, M., 1984a. A summary of the strength and modulus of ice samples from multiyear pressure ridges. Proceedings 3rd International Offshore Mechanics and Arctic Engineering Symposium, ASME, NY, vol. III, pp. 126–133. New Orleans, LA, USA.

Cox, G.F.N., Richter, J.A., Weeks, W.F., Mellor, M., 1985a. A summary of the strength and modulus of ice samples from multi-year pressure ridges. Journal of Energy Resources Technology 107 (3), 93–98.

Cox, G.F.N., Richter, J.A., Weeks, W.F., Mellor, M., Bosworth, H., 1984b. The mechanical properties of multi-year sea ice, Phase I: test results. CRREL Report 84-9. 105 pages, Hanover, NH, USA.

Cox, G.F.N., Richter-Menge, J.A., 1985. Triaxial compression testing of ice. Proceedings of the Conference “Arctic 85”, ASCE, pp. 476–488. San Francisco, CA, USA.

Cox, G.F.N., Richter-Menge, J.A., Weeks, W.F., Bosworth, H.W., Perron, N., Mellor, M., Durell, G., 1985b. Mechanical properties of multiyear sea ice, Phase II: test results. CRREL Report 85-16. 81 pages, Hanover, NH, USA.

Cox, G.F.N., Weeks, W.F., 1974. Salinity variations in sea ice. Journal of Glaciology 13 (67), 109–120.

Cox, G.F.N., Weeks, W.F., 1982. Equations for determining the gas and brine volumes in sea ice samples. CRREL Report 82-30, Hanover, N.H., USA.

Cox, G.F.N., Weeks, W.F., 1983. Equations for determining the gas and brine volumes in sea ice samples. Journal of Glaciology 29 (102), 306–316.

Cox, G.F.N., Weeks, W.F., 1988. Profile properties of undeformed first-year sea ice. CRREL Report 88-13, Hanover, N.H., USA.

Croasdale, K.R., 1980 "Ice forces on fixed, rigid structures" Part II of CRREL Special Report 80-26, IAHR Working Group on Ice Forces on Structures, State-of-the-Art Report, T.Carstens, Editor, June 1980.

Croasdale, K.R., Morgenstern, N.R. and Nuttall, J.B., 1977 "Indentation tests to investigate ice pressures on vertical piers", Jnl. of Glaciology, Vol.19, No.81.

Daley, C. G. 2007. Reanalysis of ice pressure-area relationships. 44, (4): 234-44.

Daley, C. G., 1991 “Ice edge contact - a brittle failure process model” Acta Polytechnica Scandinavica, Mechanical Engineering Series No. 100, 92 pp. Published by the Finnish Academy of Technology, Helsinki.

Daley, C. G.,1992  “Ice edge contact and failure” Cold Regions Science and Technology, 21 (1992) pp 1-23.

Daley, C., "Studies in Connection with Updating CASPPR - Group IV. Further Development of the Ship/Ice Collision Probability Model - ASPEN", By Arctec Canada Limited, for Canadian Coast Guard Northern, Transport Canada Report No. TP6274E, Sept. 1986.

Daley, C., and A. Kendrick. 2008. DIRECT DESIGN OF LARGE ICE CLASS SHIPS WITH EMPHASIS ON THE MIDBODY ICE BELT. Proceedings of the 27th international conference on offshore mechanics and arctic engineering - 2008, vol 3.

Daley, C., A. Kendrick, and M. Pavic. 2007. Comparative study of ship structural regulations. Paper presented at RINA, Royal Institution of Naval Architects International Conference - Developments in Classification and International Regulations, January 24 - 25,2007.

Daley, C., A. Kendrick, H. Yu, and B. -J Noh. 2007. Structural design of high ice class LNG tankers. Paper presented at RINA, Royal Institution of Naval Architects International Conference - Design and Construction of Vessels Operating in Low Temperature Environments, May 30, 2007 - May 31, .

Daley, C., and A. Bansal. 2009. Discussion of plastic capacity of plating subject to patch loads. Paper presented at MARSTRUCT 2009, 2nd International Conference on Marine Structures - Analysis and Design of Marine Structures, March 16, 2009 - March 18, .

Daley, C., and H. Yu. 2009. Assessment of ICE loads on stern regions of ICE class ships. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Daley, C., Howard, D., Frere, D., "Measurement of Ice Forces on Small Vessels", Report by Fleet Technology Limited to Transportation Development Centre, Montreal, Report No. TP 9451E, 1988.

Daley, C., St.John, J.W., Meyer, J.R., Brown, R., Glen, I.F., "Consolidation of Local Ice Impact Pressures Aboard the USCGC Polar Sea", by Arctec Canada Limited and Arctec Incorporated, to Transportation Development Centre, Montreal, Report No. TP8533E, Sept. 1986.

Daley, C.G., " Dynamic Ship/Ice Impact - Results of Parametric Model Testing" Proc. Ice Technology Conference, Boston, 1986.

Daley, C.G., "BAFFIN - A Dynamic Ship-Ice Interaction Model", Proc. SNAME Ice Tech '84, Calgary, Canada, 1984.

Daley, C.G., 1994 “Compilation of MSI tests results and comparison to ASPPR”, Report by Daley R&E to National Research Council of Canada, Transport Canada Report No. TP 12151E.

Daley, C.G., Ferregut, C., "Structural Risk Model of Arctic Shipping", IUTAM/IAHR Symp. on Ice-Structure Interaction, St. John's, Canada, 1989.

Daley, C.G., St. John, J.W., Seibold, F., Bayly, I., "Analysis of Extreme Ice Loads Measured on USCGC Polar Sea", Transactions of the SNAME Annual Meeting, New York, 15p., 1984.

Daley, Claude, Jukka Tuhkuri, and Kaj Riska. 1998. The role of discrete failures in local ice loads. Cold Regions Science and Technology 27, (3): 197-211.

Daly, S.F., Arcone, S.A.,"Airborne Radar Survey of a Brash Ice Jam in th St. Clair River" CRREL Report No. 89-2, Hanover, N.H., 1989.

Dean, A.M., "Field Techniques for Obtaining Engineering Characteristics of Frazil Ice Accumulations", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

DeFranco, S.J., Dempsey, J.P., 1994. Crack propagation and fracture resistance in saline ice. Journal of Glaciology 40, 451–462.

DeFranco, S.J., Wei, Y., Dempsey, J.P., 1991. Notch acuity effects on fracture toughness of saline ice. Annals of Glaciology 15, 230–235.

Dempsey, J.P., 1989. The fracture toughness of ice. Proc. IUTAM/IAHR Symposium on Ice/Structure Interaction, pp. 109–145. St. John's, Nfld., Canada.

Dempsey, J.P., Adamson, R.M., Mulmule, S.V., 1999. Scale effects on the in situ tensile strength and fracture of ice. Part II: first-year sea ice at Resolute, NWT. Intentional Journal of Fracture 95, 347–366.

Derradji-Aouat, A., 2003. Multi-surface failure criterion for saline ice in the brittle range. Cold Regions Science and Technology 36 (1), 47–70.

Det norske Veritas Ltd., "State-of-the-Art Methods for Computing Global and Local Loads in Ice Structure Interaction", Report to Public Works Canada, Ottawa, Canada 1986.

Dome Petroleum Limited "Final Report on the Full Scale Measurements of the Ice Impact Loads and Response of the KIGORIAK, August and October, 1981", Internal Report, 1982. (Unpublished)

Donn K. Haglund, Maritime Transport in Support of Arctic Resource Development, Cold Regions Science and Technology, 7 (1983) 231—249 Elsevier Science Publishers B.V., Amsterdam

Dutta, Piyush K., David M. Cole, Erland M. Schulson, and Devinder S. Sodhi. 2004. A fracture study of ice under high strain rate loading. International Journal of Offshore and Polar Engineering 14, (3): 182-8.

Dykins, J.E., 1967. Tensile properties of sea ice grown in a confined system. In: Oura, H. (Ed.), Proceedings of the International Conference on Low Temperature Science. Physics of Snow and Ice, vol. 1. Institute of Low Temperature Science, Sapporo, Japan, pp. 523–537.

Dykins, J.E., 1968. Tensile and flexural properties of saline ice. In: Riehl, N., Bullemer, B., Engelhardt, H. (Eds.), Physics of Ice. Plenum Press, New York, NY, pp. 251–270.

Dykins, J.E., 1971. Ice engineering— material properties for a limited range of conditions. US Navy Civil Engineering Laboratory, Technical Report R720, Port Hueneme, CA.

Edwards, R.Y., et al. "Influence of Major Characteristics of Icebreaker Hulls on their Powering Requirements and Maneuveribility in Ice", Transactions of the SNAME Annual Meeting, Vol.84, New York, 1976.

Edwards, R.Y., et.al. "Full Scale and Model Tests of a Great Lakes Icebreaker", Transactions of the SNAME Annual Meeting, Vol.80, New York, 1972,

Edwards, R.Y., et.al. "Results of Full Scale Trials in Ice of CCGS Pierre Radisson", Proc. SNAME STAR Symp., Ottawa, 1981.

Eicken, H., Krouse, H.R., Kadko, D., Perovich, D.K., 2002. Tracer studies of pathways and rates of meltwater transport through Arctic summer sea ice. Journal of Geophysical Research 107 (C10), 8046. doi:10.1029/2000JC000583.

Eicken, H., Lensu, M., Leppäranta, M., Tucker III, W.B., Gow, A.J., Salmela, O., 1995. Thickness, structure, and properties of level summer multi-year ice in the Eurasian sector of the Arctic Ocean. Journal of Geophysical Research 100, 22697–22710.

El-Tahan, H., Swamidas, A.S.J., Arockiasamy, M., "Impact/Indentation Strength of Iceberg and Artificial Snow Ice", Jnl. of Offshore Mechanics and Arctic Engineering, Vol.110, Feb. 1987.

Enkvist, E., "On The Ice Resistance Encountered by Ships Operating in the Continuous Mode of Icebreaking", Swedish Academy of Engineering Sciences in Finland, Report No. 24,Thesis for the degree of Doctor of Technology, Helsinki University of Technology, 1972.

Enkvist, E., "Survey of Experimental Indications of the Relation between the Submersion and Breaking Components of Level Ice Resistance to Ships", Proc. POAC '83, Helsinki, Finland, 1983.

Enkvist, Ernst, Petri Varsta, and Kaj Riska. 1979. SHIP-ICE INTERACTION. SAE Preprints: 977-1002.

Ettema, R., Stern, F., Lazaro, J., "Dynamics of Continuous-Mode Icebreaking by a Polar-Class Hull - Part 1: Mean Response", Jnl. of Ship Research, Vol33, No.2, 1989.

Fenco, 1977. 1977 Winter field ice survey offshore Labrador. Report submitted to Total Eastcan Exploration Ltd., Calgary, AB., Canada.

Fish, Anatoly M., and Yuri K. Zaretsky. 1997. Temperature effect on strength of ice under triaxial compression. Paper presented at Part 3 (of 4), May 25, 1997 - May 30, .

Forsberg, R., H. Skourup, O. Andersen, S. Laxon, A. Ridout, A. Braun, J. Johannessen, F. Siegismund, C. C. Tscherning, and P. Knudsen. 2007. Combination of spaceborne, airborne and surface gravity in support of arctic ocean sea-ice and MDT mapping.

Frankenstein, G.E., 1959. Strength data on lake ice. SIPRE Technical Report 59, Wilmette, IL, USA.

Frankenstein, G.E., 1961. Strength data on lake ice. SIPRE Technical Report 80, Wilmette, IL, USA.

Frankenstein, G.E., Garner, R., 1967. Equations for determining the brine volume of sea ice from −0.5 to −22.9 °C. Journal of Glaciology 6 (48), 943–944.

Fransson, L., Olofsson, T., and Sandkvisk , J., 1991 “Observations of the failure process in ice blocks crushed by a flat indentor”, Proceedings POAC ‘91, St. Johns, Canada.

Frederking, R., "Dynamic Ice Forces on an Inclined Structure", IUTAM Symp. on the Physics and Mechanics of Ice, Copenhagen, 1979.

Frederking, R., Barker, A., 2002a. Friction of sea ice on steel for condition of varying speeds. Proceedings of the 12th International Offshore and Polar Engineering Conference, pp. 766–771. Kitakyushu, Japan.

Frederking, R., Barker, A., 2002b. Friction of sea ice on various construction materials. Proceedings of the 16th IAHR International Symposium on Ice, vol. 1, pp. 442–449. Dunedin, New Zealand.

Frederking, R., Blanchet, D., Jordaan, I.J., Kennedy, N.K., Sinha, N.K. and Stander, E., 1990 “Field tests of ice indentation at medium scale, ice island, April 1989” Client Report for Canadian Coast Guard and Transportation Development Centre, By Institute for Research in Construction, National Research Council, Ottawa, October, 1990.

Frederking, R., Sudom, D., 2006. Maximum ice force on the Molikpaq during the April 12, 1986 event. Cold Regions Science and Technology 46 (3), 147–166.

Frederking, R., Timco, G.W., "Field Measurements of the Shear Strength of Columnar-Grained Sea Ice", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

Frederking, R.M.W., 1977. Plane-strain compressive strength of columnar-grained and granular snow-ice. Journal of Glaciology 18, 505–516.

Frederking, R.M.W., Hausler, F., 1978. The flexural behaviour of ice from in situ cantilever beam tests. Proceedings International Association for Hydraulic Research, IAHR Symposium on Ice Problems, vol. 1, pp. 197–215. Lulea, Sweden.

Frederking, R.M.W., Svec, O., Timco, G.W., 1988. The shear strength of ice. Proceedings 9th International Association for Hydraulic Research Symposiumon Ice, vol. 3, pp. 76–88. Sapporo, Japan.

Frederking, R.M.W., Timco, G.W., 1980. NRC ice property measurements during the Canmar Kigoriak trials in the Beaufort Sea, winter 1979–80. DBR Paper No. 947, DBR/NRC Rept., Ottawa, ON, Canada.

Frederking, R.M.W., Timco, G.W., 1983. Uni-axial compressive strength and deformation of Beaufort Sea ice. Proceedings International Conference on Port and Ocean Engineering under Arctic Condition, POAC 83, vol. I, pp. 89–98. Helsinki, Finland.

Frederking, R.M.W., Timco, G.W., 1984a. Measurement of shear strength of granular/ discontinuous columnar sea ice. Cold Regions Science and Technology 9, 215–220.

Frederking, R.M.W., Timco, G.W., 1984b. Compressive behaviour of Beaufort Sea ice under vertical and horizontal loading. Proceedings OMAE Symposium, vol. III, pp. 145–149. New Orleans, USA.

Frederking, R.M.W., Timco, G.W., 1986. Field measurements of the shear strength of columnar-grained sea ice. Proceedings 8th International Association for Hydraulic Research Symposium on Ice, vol. I, pp. 279–292. Iowa City, U.S.A.

Gagnon, R. 1987. ELASTIC CONSTANTS OF ICE ih, UP TO 2. 8 KBAR, BY BRILLOUIN SPECTROSCOPY. Paper presented at VIIth Symposium on the Physics and Chemistry of Ice. .

Gagnon, R. 1994. Generation of melt during crushing experiments on freshwater ice. Cold Regions Science and Technology 22, (4): 385-98.

Gagnon, R. 2004. Analysis of laboratory growler impact tests. Cold Regions Science and Technology 39, (1): 1-17.

Gagnon, R. 2008. Analysis of data from bergy bit impacts using a novel hull-mounted external impact panel. Cold Regions Science and Technology 52, (1): 50-66.

Gagnon, R. E. 1998. Analysis of visual data from medium scale indentation experiments at Hobson's Choice ice island. Cold Regions Science and Technology 28, (1): 45-58,.

Gagnon, R. E. 2007. Results of numerical simulations of growler impact tests. Cold Regions Science and Technology 49, (3) (9): 206-14.

Gagnon, R. E., H. Kiefte, M. J. Clouter, and E. Whalley. 1987. ACOUSTIC VELOCITIES IN ICE ih, II, III, V AND VI, BY BRILLOUIN SPECTROSCOPY. Paper presented at VIIth Symposium on the Physics and Chemistry of Ice. .

Gagnon, R. E., S. J. Jones, R. Frederking, P. A. Spencer, and D. M. Masterson. 2001. Large-scale hull loading of sea ice, lake ice, and ice in tuktoyaktuk harbour. Journal of Offshore Mechanics and Arctic Engineering 123, (4): 159-69.

Gagnon, R.E. 1991 “Heat generation during crushing experiments on freshwater ice” In: Proc. of the 8th Int. Symp. on the Physics and Chemistry of Ice, Hokkaido University Press, Sapporo, pp. 447-455.

Gagnon, Robert, David Cumming, Ron Ritch, Robin Browne, Michelle Johnston, Robert Frederking, Richard McKenna, and Freeman Ralph. 2008. Overview accompaniment for papers on the bergy bit impact trials. Cold Regions Science and Technology 52, (1): 1-6.

Gagnon, Robert. 2003. High-speed video analysis of fracture propagation in a thick-edge-loaded freshwater ice sheet. Canadian Journal of Physics 81, (1-2): 261-9.

Gale, A.D., Wong, T.T., Sego, D.C., Morgenstern, N.R., "Stress-Strain Behavior of Cohesionless Broken Ice", Proc. POAC '87, Fairbanks, Alaska, 1987.

Gammon, P. H., R. E. Gagnon, Walter Bobby, and W. E. Russell. 1983. PHYSICAL AND MECHANICAL PROPERTIES OF ICEBERGS. Paper presented at 1983 Proceedings - Fifteenth Annual Offshore Technology Conference. .

Gammon, P.H., Kietfe, H., Cloutier, M.J., Denner, W.W., 1983. Elastic constants of artificial and naturally ice samples by Brillouin spectroscopy. Journal of Glaciology 29 (103), 433–460.

Geotech, 1983. Multi-year ice strength testing program for Gulf Canada Resources Inc. Report submitted to Gulf Canada, Calgary, AB, Canada. (APOA Project 200).

Geotech, 1984. Multi-year ice strength test program, phase II. Report 9100 submitted to Gulf Canada, Calgary, AB, Canada.

German and Milne "Report on Tests to Measure Ice Forces on the Hull of an Icebreaker and the Penetration of an Icebreaker in Ice" MOT Req. T8257-3-0010 for the Canadian Coast Guard, Sept. 1973.

German, J.G., et.al. "Data Collection System on Board the Ice-Strengthened Cargo Vessel M.V. Arctic for Evaluation of Ship Performance in Ice", Proc. SNAME STAR Symp., Ottawa, 1981.

Gershunov, E.M., "Scale Effect in Ice", Cold Regions Science and Technology, Vol 12, Elsevier Sci. Pub., 1986.

Ghoneim, G. A. M., and A. J. Keinonen. 1983. FULL-SCALE IMPACT TESTS OF CANMAR KIGORIAK IN THICK ICE. Paper presented at 7th International Conference on Port and Ocean Engineering under Arctic Conditions, .

Ghoneim, G. A., and Bor-Feng Peng. 2009. Application of recent ice class requirements for arctic EP offshore structures. Paper presented at 19th (2009) International OFFSHORE AND POLAR ENGINEERING CONFERENCE, June 21, 2009 - June 26, .

Ghoneim, G.A.M., "Local and Global Strength Aspects for Icebreaking Ships", Proc. IPTC Vancouver, Canada, 1986.

Ghoneim, G.A.M., Johansson, B.M., Smyth, M.W., Grinstead, J., "Global Ship Ice Impact Forces Determined from Full-Scale and Analytical Modelling of the Icebreakers Canmar Kigoriak and Robert LeMeur", Transactions of the SNAME Annual Meeting, New York, 1984.

Gill, R.J., et al. "A Ship Transit Model for Passage Through Ice and its Application to the Labrador Area", Proc. SNAME STAR Symp., Ottawa, 1981.

Glen, I. F., C. G. Daley, and G. Tam. 1985. ANALYSIS OF THE STRUCTURE OF THE PROPOSED CCG POLAR CLASS 8 ICEBREAKER UNDER EXTREME ICE LOADS. Paper presented at Society of Naval Architects and Marine Engineers - Transactions. Papers Presented at the 93rd Annual Meeting. .

Glen, I. F., C. G. Daley, and G. Tam. 1985. ANALYSIS OF THE STRUCTURE OF THE PROPOSED CCG POLAR CLASS 8 ICEBREAKER UNDER EXTREME ICE LOADS. Paper presented at Society of Naval Architects and Marine Engineers - Transactions. Papers Presented at the 93rd Annual Meeting. .

Glen, I.F., Blount, H., "Measurement of Ice Impact Pressures and Loads Onboard CCGS Louis S. ST.Laurent", Proc. Third Conference on Offshore Mechanics and Arctic Engineering, New Orleans, 7p., 1984.

Glen, I.F., et.al. "Results of Full Scale Measurements Aboard CCGS Louis S. St. Laurent During a 1980 Fall Arctic Probe", Report 737C by Arctec Canada Limited to Canadian Coast Guard, April, 1981.

Glen, Ian F., and George Comfort. 1983. ICE IMPACT PRESSURE AND LOAD: INVESTIGATION BY LABORATORY EXPERIMENTS AND SHIP TRIALS. Paper presented at 7th International Conference on Port and Ocean Engineering under Arctic Conditions, .

Gold, L.W., 1971. Use of ice covers for transportation. Canadian Geotechnical Journal 8, 170–181.

Gold, L.W., 1977. Engineering properties of freshwater ice. Journal of Glaciology 19 (81), 197–212.

Goldstein, Robert V., Nikolai M. Osipenko, and Matti Lepparanta. 2009. Relaxation scales and the structure of fractures in the dynamics of sea ice. Cold Regions Science and Technology 58, (1-2): 29-35.

Gosink, J.P., Osterkamp, T.E., "Frazil Ice Nucleation by Ejecta from Supercooled Water", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

Gossett, J., Arctic research using nuclear submarines, Sea Technol., 37(3), 33{40, 1996.

Gow, A.J., Ackley, S.F., Buck, K.R., Golden, K.M., 1987a. Physical and structural characteristics of Weddell Sea pack ice. CRREL Report 87-14. 70 pages, Hanover, NH, USA.

Gow, A.J., Tucker, W.B., Weeks, W.F., 1987b. Physical properties of summer sea ice in the Fram Strait, June–July, 1984. CRREL Report 87-16. 81 pages, Hanover, NH, USA.

Gow, A.J., Ueda, H.T., 1984. Flexural strengths of freshwater model ice. Proceedings International Association for Hydraulic Research, IAHR Symposium on Ice Problems, vol. 1, pp. 73–82. Hamburg, Germany.

Gow, A.J., Ueda, H.T., Govoni, J.W., Kalafut, J., 1988. Temperature and structure dependence of the flexural strength and modulus of freshwater model ice. CRREL Report 88-6, Hanover, NH, USA. CRREL Report 88-6, Hanover, NH, USA.

Gow, A.J., Ueda, H.T., Richard, J.A., 1987c. Flexural strength of ice on temperate lakes. U.S. Army CRREL Report 78-9, Hanover, NH, USA.

Grace, I. F., and R. A. Ibrahim. 2008. Modelling and analysis of ship roll oscillations interacting with stationary icebergs. Proceedings of the Institution of Mechanical Engineers, Part C: Journal of Mechanical Engineering Science 222, (10): 1873-84.

Gratz, E.T., Schulson, E.M., 1994. Preliminary observations of brittle compressive failure of columnar saline ice under triaxial loading. Annals of Glaciology 19, 33–38.

Gratz, E.T., Schulson, E.M., 1997. Brittle failure of columnar saline ice under triaxial compression. Journal of Geophysical Research 102 (B3), 5091–5107.

Greisman, P., "Brash Ice Behavior", Report CG-D-30-81 by the United States Coast Guard, Groton CT., 1981.

Gylden, R., Riska, K.,, "Ice Load Measurements On Board MS Kemira, Winter 1989", Helsinki University of Technology, Laboratory of Naval Architecture and Marine Engineering, Report M-93, Otaniemi, 1989.

Hakala, M., Riska, K., "Results from Statistical Measuring System on board M.V. Arctic 1982", Report by VTT to Canadian Coast Guard Northern, 286p., 1983.

Hakala, Matti K. 1979. NONLINEAR FINITE ELEMENT ANALYSIS OF AN ICE-STRENGTHENED SHIP SHELL STRUCTURE. Computers and Structures 12, (4): 541-7.

Hamza, H., Shih, L.Y., "Numerical Models of Pack Ice Interaction with Ships", Proc. IAHR Ice Symp. Sapporo, Japan, 1988.

Han, S., J. -Y Lee, Y. -I Park, and J. Che. 2008. Structural risk analysis of an NO96 membrane-type liquified natural gas carrier in baltic ice operation. Proceedings of the Institution of Mechanical Engineers Part M: Journal of Engineering for the Maritime Environment 222, (4): 179-94.

Hausler, F.U., 1981. Multi-axial compressive strength tests on saline ice with brush-type loading platens. IAHR International Symposium on Ice, vol. 2, pp. 526–539. Lulea, Sweden.

Hibler, W.D., Ackley, S., Weeks, W.F., Kovacs, A., 1972. Top and bottom roughness of a multiyear ice floe. Proceedings of the International Association for Hydraulic Research IAHR Symposium on Ice, pp. 130–142. Leningrad, U.S.S.R.

Hill, B. T. 2010. Ice charts & ship / iceberg database. in NRC-IOT [database online]. Available from http://researchers.imd.nrc.ca/~hillb/icedb/ice/.

Hindley, R. J., and E. Ranki. 2009. Learning from ICE damage investigations for future arctic ships. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Hobbs, P. V. (1974) Ice Physics. Clarendon Press, Oxford.

Hoffmann, L., "Impact Forces and Friction Coefficients on the Forebody of the German Polar Research Vessel POLARSTERN", Proc. POAC '85, Narssarssuaq, Greenland, 1985.

Holsapple, K.A., Schmidt, R.M., " On the Scaling of Crater Dimensions, 2. Impact Processes", Jnl. of Geophysical Research, Vol.87, No.B3, 1982.

Hong, Lin, and Jrgen Amdahl. 2007. Plastic design of laterally patch loaded plates for ships. Marine Structures 20, (3): 124-42.

Hong, S. 2010. Detection of small-scale roughness and refractive index of sea ice in passive satellite microwave remote sensing. Remote Sensing of Environment 114, (5): 1136-40.

Hooke, R. L., M. Mellor, W. F. Budd, J. W. Glen, A. Higashi, T. H. Jacka, S. J. Jones, et al. 1980. MECHANICAL PROPERTIES OF POLYCRYSTALLINE ICE: AN ASSESSMENT OF CURRENT KNOWLEDGE AND PRIORITIES FOR RESEARCH. Cold Regions Science and Technology 3, (4): 263-75.

Hopkins, M.A., Hibler, W.D., "On Modelling the Energetics of the Ridging Process", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Howard, D., Menon,B., Daley, C., "Parametric Modelling of a 150,000 Tonne Tanker - Hydroelastic response during Multi-year Ice Impact" Report by Fleet Technology Limited to Canadian Coast Guard Northern, Transport Canada Report No. TP10031, Nov. 1989.

Hunkins, K., 1960. Seismic studies of sea ice. Journal of Geophysical Research 65, 3,459–3,472.

Huther, M., Beghin, D., Mogensen, O., "Hull Girder Minimum Section Modulus of Large Merchant Ice Breakers",  Proc. SNAME Ice Tech '84, Calgary, Canada, 1984.

Hysing, T., "Form Design of Vessels. Part II: Numerical Simulation Model", Marine Structures in Ice, NHL Report 81-03, Trondheim, Norway, 1981.

Ibrahim, R. A., N. G. Chalhoub, and Jeffery Falzarano. 2007. Interaction of ships and ocean structures with ice loads and stochastic ocean waves. Applied Mechanics Reviews 60, (1-6): 246-90.

IIP. 2010. International ice patrol (IIP) iceberg sightings database. in NSIDC [database online]. Available from http://nsidc.org/data/docs/noaa/g00807_international_iceberg_sightings/index.html.

Iliescu, Daniel, and Erland M. Schulson. 2004. The brittle compressive failure of fresh-water columnar ice loaded biaxially. Acta Materialia 52, (20): 5723-35.

Ishibashi, Y., "Estimation of Icebreaking Resistance by Ship Motion Simulation",Proc. SNAME Ice Tech '84, Calgary, Canada, 10p.,1984.

Ishimaru, J., K. Tsumura, K. Sato, T. Ishida, and M. Oka. 2007. Practical design of LNG carriers in low temperature environments. Paper presented at RINA, Royal Institution of Naval Architects International Conference - Design and Construction of Vessels Operating in Low Temperature Environments, May 30, 2007 - May 31, .

Jebaraj, C., A. S. J. Swamidas, and L. Y. Shih. 1988. Numerical modelling of ship/ice interaction. Paper presented at Offshore and Arctic Operations Synposium 1989, January 22, 1989 - January 25, .

Jebaraj, C., Swamidas, A.S.J., Jones, S.D., "Dynamics of Ship/Ice Interaction", Proc. Polar Tech'88, Trondheim, Norway, 1988.

Jeffries, M.G., Wright, W.H., 1988. Dynamic response of Molikpaq to ice–structure interaction. Proc. OMAE'88, vol. IV, pp. 201–220. Houston, TX, USA.

Jeffries, M.O., Sackinger, W.M., Frederking, R., Timco, G.W., 1988. Initial mechanical and  physical–structural propertymeasurements of old sea and brackish ice from the Ward Hunt Ice Shelf, Canada. Proceedings IAHR Symposium on Ice, vol. 1, pp. 177–187. Sapporo, Japan.

Jeong, Seong-Yeob, and Kyungsik Choi. 2009. A review on ice resistance prediction formula for icebreaking vessels. Paper presented at 19th (2009) International OFFSHORE AND POLAR ENGINEERING CONFERENCE, June 21, 2009 - June 26, .

Jia, Zhibin, Anders Ulfvarson, Jonas W. Ringsberg, and Junbo Jia. 2009. A return period based plastic design approach for ice loaded side-shell/bow structures. Marine Structures 22, (3): 438-56.

Joensuu, A., Riska, K., "Jään ja Rakenteen Välinen Kosketus"(Contact Between Ice and Structure) Helsinki University of Technology, Laboratory of Naval Architecture and Marine Engineering, Report M-88, Otaniemi, 1988. (in Finnish)

Johansson, B.,M., Liljeström, G.C., "ODEN A State-of-the-Art Icebreaker", Proc. POAC '89, Luleå, Sweden, 1989.

Johansson, B.M., Keinonen, A.J., Mercer, B., "Technical Development of an Environmentally Safe Tanker", Proc. SNAME STAR Symp., Ottawa, 1981.

Johnston, M., 1998. Influence of ice microstructure on microwave scattering properties of sea ice. Ph.D. Thesis no. 16884, Universite Laval, Quebec City, Canada, 311 pages.

Johnston, M., 2008. Characterizing multi-year ice in the high Arctic: evaluating two ground-based EM sensors. Report by NRC Canadian Hydraulics Centre, CHC-CTR-073. 70 pages (Controlled), Ottawa, ON, Canada.

Johnston, M., Frederking, R., Timco, G.W., 2000. Seasonal decay of first-year sea ice. NRC Canadian Hydraulics Centre Report HYD-TR-058, Ottawa, Ont., Canada.

Johnston, M., Frederking, R., Timco, G.W., 2001. Decay induced changes in the physical and mechanical properties of first-year sea ice. Proceedings Port and Ocean Engineering under Artic Conditions, POAC'01, vol. 3, pp. 1395–1404. Ottawa, Canada.

Johnston, M., Frederking, R., Timco, G.W., 2002. Properties of decaying first-year sea ice: two seasons of field measurements. Proc. 17th Int. Sym. on Okhotsk Sea and Sea Ice, pp. 303–311. Hokkaido, Japan.

Johnston, M., Frederking, R., Timco, G.W., 2003a. Properties of decaying first-year sea ice at five sites in Parry Channel. Proc. 17th Int. Conf. on Port and Ocean Engineering under Arctic Conditions (POAC), vol. 1, pp. 131–140. Trondheim, Norway.

Johnston, M., Frederking, R., Timco, G.W., 2003b. Property changes of first-year ice and old ice during summer melt. NRC Canadian Hydraulics Centre Report CHC-TR-010, TP14098E, Ottawa, Ont., Canada.

Johnston, M., G. W. Timco, R. Frederking, and M. Miles. 2008. Measuring global impact forces on the CCGS terry fox with an inertial measurement system called MOTAN. 52, (1): 67-82.

Johnston, M., R. Ritch, and R. Gagnon. 2008. Comparison of impact forces measured by different instrumentation systems on the CCGS terry fox during the bergy bit trials. Cold Regions Science and Technology 52, (1): 83-97.

Johnston, M., Timco, G.W., 2002. Temperature changes in first year arctic sea ice during the decay process. Proceedings of the 16th IAHR International Symposium on Ice, vol. 2, pp. 194–202. Dunedin, New Zealand.

Johnston, M., Timco, G.W., 2008. Understanding and identifying Old Ice in summer. NRC Canadian Hydraulics Centre Report CHC-TR-055, Ottawa, Ont., Canada.

Johnston, M., Timco, G.W., Frederking, R., 2003c. In situ borehole strength measurements on multi-year sea ice. Proceedings of the 13th International Offshore and Polar Engineering Conference, pp. 445–452. Honolulu, Hawaii, USA.

Johnston, M.E., Masterson, D., Wright, B., 2009. Multi-year ice thickness: knowns and unknowns. Proceedings 20th POAC Conference, Paper POAC09-120, Lulea University of Technology, Lulea, Sweden.

Jones, N., "A Note on Ice Scaling", Jnl. of Ship Research, Vol. 30, No. 2, June 1986.

Jones, S. J. 1989. Review of ship performance in level ice. Paper presented at Proceedings of the Eighth International Conference on Offshore Mechanics and Arctic Engineering - 1989, March 19, 1989 - March 23, .

Jones, S. J.,  2007. A review of the strength of iceberg and other freshwater ice and the effect of temperature. Cold Regions Science and Technology 47, (3): 256-62.

Jones, S.J., " A Review of Ship Performance in Level Ice", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Jones, Stephen J. 1997. High strain-rate compression tests on ice. Journal of Physical Chemistry B 101, (32): 6099-101.

Jones, Stephen J. 2007. A review of the strength of iceberg and other freshwater ice and the effect of temperature. Cold Regions Science and Technology 47, (3): 256-62.

Jones, Stephen J., 1997. High strain-rate compression tests on ice. Journal of Physical Chemistry B 101, (32): 6099-101.

Jones, Stephen J., R. E. Gagnon, A. Derradji, and A. Bugden. 2003. Compressive strength of iceberg ice. Canadian Journal of Physics 81, (1-2): 191-200.

Jordaan, I.J., "Numerical and Finite Element Techniques in Calculation of Ice-Structure Interaction", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

Jordaan, I.J., and McKenna, R.F., 1991 “Processes of deformation and fracture of ice in compression”, in Ice-Structure Interaction, Jones S.J., et.al. (Eds), IUTAM-IAHR Symposium St. John’s, Newfoundland, Canada, Pub. by Springer Verlag.

Jordaan,. I.J., Maes, M., Nadreau, J-P, "The Crushing and Clearing of Ice in Fast Spherical Indentation Tests", Proc. Seventh Conference on Offshore Mechanics and Arctic Engineering, Houston, 1988.

Kämäräinen, J., "Icebreaker Tanker Transit Simulation Model", Proc. Polar Tech'88, Helsinki, Finland, 1986.

Kamio, Zenji, Junichiro Ushikoshi, Hisao Matsushita, Takashi Terashima, and Hiroshi Saeki. 2002. Mechanical strength of consolidated sea ice rubble- in case of first-year sea ice at notoro lagoon, hokkaido. Paper presented at Proceedings of the Twelfth (2002) International Offshore and Polar Engineering Conference, May 26, 2002 - May 31, .

Kankaanpää, P., "Structure of First Year Pressure Ridges in the Baltic", Proc. POAC '89, Luleå, Sweden, 1989.

Kashteljan, V.I., "Approximate Determination of the Forces which Destroy the Ice Cover", Problems of the Arctic, No.5, 1960.

Kashteljan, V.I., et.al., "Ice Resistance to Motion of a Ship" Sudostroenie, Leningrad, 1968.

Kawakami, S., et al, "Impact Experiments on Ice", Jnl. of Geophysical Research, Vol.88, No. B7, 1983.

Kayo, Y., Kawasaki, T., Minami, T., Tozawa, S., Tanaka, A., Abdelnour, R., 1983. Field study on mechanical properties of sea ice at east coast of Hokkaido. Proceedings POAC'83, vol. 1, pp. 109–118. Helsinki, Finland.

Keinonen, A.,"An Analytical Method for Calculating the Pure Ridge Resistance Encountered by Ships in First Year Ice Ridges" Thesis for the degree of Doctor of Technology, Helsinki University of Technology, 1979.

Keinonen, A.J., "Ship Performance and Resistance in Ice Ridges", Proc. WEGEMT Seventh Graduate School, Ships and Structures in Ice, Helsinki, 82p., 1983.

Kendrick, A., C. Daley, V. Santos-Pedro, and R. Hayward. 2007. Classification of polar ships. Paper presented at RINA, Royal Institution of Naval Architects International Conference - Design and Construction of Vessels Operating in Low Temperature Environments, May 30, 2007 - May 31, .

Kennedy, K.P., Mamer, K.J., Dempsey, J.P., Adamson, R.M., Spencer, P.A., Masterson, D.M., 1994. Large scale ice fracture experiments: Phase 2. Proc. 12th IAHR Ice Symposium, vol. 1, pp. 315–324. Trondheim, Norway.

Kerr, A.D., "The Bearing Capacity of Floating Ice Plates Subjected to Static of Quasi-Static Loads", Jnl. of Glaciology, Vol17, No.76, 1976.

Kheysin, D.Y., "On the Ice Navigation Speed of Ships in Extremely Solid Ice", The Navigational Qualities of Ships, Ed. by Kheysin, D.Y., Popov, I.N., CRREL Translation TL 417, 1973.

Kheysin, D.Ye., Likhomanov, V.A., Kurdyumov, V.A., "Determination of Specific Breakup Energy and Contact Pressures Produced by the Impact of a Solid Against Ice", Symp. on Physical Methods in Studying Snow and Ice, Leningrad, CRREL Translation TL539, 1973.

Kim, B. J., S. K. Park, B. S. Jang, and J. K. Paik. 2009. Crushing behaviour of steel plated structures in cold temperature: Experimental and numerical studies. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Kim, Hyonny, and John N. Keune. 2007. Compressive strength of ice at impact strain rates. Journal of Materials Science 42, (8): 2802-6.

Kim, Hyun-Soo, Mun-Keun Ha, Dang Ahn, Soo-Hyung Kim, and Jong-Woo Park. 2005. Development of 115K tanker adopting baltic ice class 1A. Paper presented at 15th International Offrshore and Polar Engineering Conference, ISOPE-2005, June 19, 2005 - June 24, .

Kivisild, H. R., D. Blanchet, and C. Revill. 1987. SHIP/ICE INTERACTION PRESSURES AND ENERGIES DURING SHIP RAMMING. Paper presented at Proceedings of the Sixth (1987) International Offshore Mechanics and Arctic Engineering Symposium. .

Kivisild, H.R., 1992. In situ borehole testing in ice: a historical perspective. Proceedings IAHR Symposium on Ice, vol. 2, pp. 841–855. Banff, AB, Canada.

Knight, Rosemary. 2001. GROUND PENETRATING RADAR FOR ENVIRONMENTAL APPLICATIONS. Annual Review of Earth and Planetary Sciences 29, (1) (05/01): 229-55, http://dx.doi.org.qe2a-proxy.mun.ca/10.1146/annurev.earth.29.1.229.

Koehler, P. E., and L. Jorgensen. 1985. SHIP ICE IMPACT ANALYSIS. Paper presented at Proceedings of the Fourth International Offshore Mechanics and Arctic Engineering Symposium. Presented at the 1985 ASME Energy-Sources Technology Conference & Exhibition. .

Kohnen, H., 1972. Seismic and ultrasonic measurements on the sea ice of Eclipse Sound near Pond Inlet, N.W.T. on Northern Baffin Island. Polarforschung 42, 66–74.

Korri, P., Varsta, P., "On the Ice Trial of a 14500 DWT Tanker on the Gulf of Bothnia", Proc. of NSTM-79 Helsinki, Finland.

Korzhavin, K.N. 1962 “Action of ice on engineering structures” USSR Acad. of Sci. Siberian Branch. CRREL Draft Translation No. 260, Hanover, USA, 1971.

Kostilainen, V., "Modelling the Ship Performance in Ice-Clogged Channels", Proc. WEGEMT Seventh Graduate School, Ships and Structures in Ice, Helsinki, 44p., 1983.

Kotras, T.V., Baird, A.V., Naegle, J.N., "Predicting Ship Performance in Level Ice", Transactions of the SNAME Annual Meeting, New York, 1983.

Kovacs, A., 1975. A study of multi-year pressure ridges and shore ice pile-up. APOA Project Report 89. 45 pages, Hanover, NH, USA.

Kovacs, A., 1996a. Sea Ice Part 1. Bulk salinity versus ice floe thickness. CRREL Report 96-7, Hanover, NH, USA.

Kovacs, A., 1996b. Sea Ice Part II. Estimating the full-scale tensile, flexural, and compressive strength of first-year ice. CRREL Report 96-11. 17 pages, Hanover, NH, USA.

Kovacs, A.,Mellor, M., 1971. Sea ice pressure ridges and ice islands. Report by Creare Inc. for Arctic Petroleum Operators Association, Technical Note, vol. 122. Hanover, NH, USA.

Krupina, Nina A., and Nickolay V. Kubyshkin. 2007. Flexural strength of drifting level first-year ice in the barents sea. Paper presented at 17th 2007 International Offshore and Polar Engineering Conference, ISOPE 2007, July 1, 2007 - July 6, .

Krupina, Nina A., Vladimir A. Likhomanov, Alexey V. Chernov, and Yury P. Gudoshnikov. 2009. Full-scale ice impact study of icebreaker kapitan nikolaev: General description. Paper presented at 19th (2009) International OFFSHORE AND POLAR ENGINEERING CONFERENCE, June 21, 2009 - June 26, .

Kry, P.R., 1978 "A statistical prediction of effective ice crushing stresses on wide structures" Proceedings IAHR Ice Symposium , Luleå, Sweden.

Kubat, I., and G. W. Timco. 2005. NRC marine icing database. Paper presented at Proceedings of the 11th International Workshop on Atmospheric Icing of Structures, Montreal.

Kubat, I., Timco, G.W., 2003. Vessel damage in the Canadian Arctic. Proceedings 17th International Conference on Port and Ocean Engineering under Arctic Conditions, POAC'03, vol. 1, pp. 203–212. Trondheim, Norway.

Kuehn, G.A., Lee, R.W., Nixon, W.A., Schulson, E.M., 1990. The structure and tensile behavior of first-year sea ice and laboratory grown saline ice. ASME Journal of Offshore Mechanics and Arctic Engineering 112 (4), 357–363.

Kujala, P. 1994. On the statistics of ice loads on ship hull in the baltic. Acta Polytechnica Scandinavica, Mechanical Engineering Series(116): 1-98.

Kujala, P. 1996. Semi-empirical evaluation of long term ice loads on a ship hull. Marine Structures 9, (9): 849-71.

Kujala, P., Vuorio, J., "Results and Statistical Analysis of Ice Load Measurements On Board Icebreaker SISU in Winters 1979 to 1985", Winter Navigation Research Board Report 43, 52p., 1986

Kujala,P., "Results of Long Term Ice Load Measurements On Board the Chemical Tanker Kemira in the Baltic Sea During the Winters 1985 to 1988", Winter Navigation Research Board Report 47, 1989.

Kwak, Myo-Jung, Joong-Hyo Choi, Jae-Hyung Park, and Je-Hyouk Woo. 2009. Strength assessment for bow structure of arctic tanker (107K) under ship-ICE interaction. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Kwok, R., G.F. Cunninham, H.J. Zwally, and D. Yi (2006) ICESat over Arctic sea ice: Interpretation of altimetric and reflectivity profiles. J. Geophys. Res.,111, C06006, doi: 10.1029/2005JC003175.

Kwok, R., Rothrock, D.A., 2009. Decline in Arctic sea ice thickness from submarine and ICESat records: 1958–2008. JGR 36, L15501. doi:10.1029/2009GL039035.

Lainey, L., Tinawi, R., 1981. The mechanical properties of sea ice — a compilation of available data. Canadian Journal of Civil Engineering 11 (4), 884–923.

Lake, R.A., Lewis, E.L., 1970. Salt rejection by sea ice during growth. Journal of Geophysical Research 75 (3), 583–597.

Lane, K., D. Power, I. Chakraborty, J. Youden, C. Randell, J. McClintock, and D. Flett. 2002. RADARSAT-1 synthetic aperture radar iceberg detection performance ADRO-2 A223.

Langleben, M.P., 1959. Some physical properties of sea ice II. Canadian Journal of Physics 37, 1438–1454.

Langleben, M.P., Pounder, E.R., 1963. Elastic parameters of sea ice. In: Kingery, W.D. (Ed.), Ice and Snow. MIT Press, USA, pp. 69–78.

Laskow, V., "Ship-Ice Interaction: A Designers Approach", SNAME Arctic Section Spring Meeting, Calgary, Canada, 1982.

Lavrov, V.V., 1969. Deformation and strength of ice. Gidrometeorologicheskoe Izdatel'stvo. Leningrad.

Lawson, W.A., Hadley, R.D., "Evaluation of Marine Oil Transportation Systems for Arctic Areas", Proc. Polar Tech'88, Trondheim, Norway, 1988.

Laxon, S., N. Peacock, and D. Smith (2003) High interannual variability of sea ice thickness in the Arctic Region. Nature, 425, 947–950.

Lee, J., Ralston, T.D., Petrie, D.H., 1986. Full-thickness sea ice strength tests. Proceedings IAHR Ice Symposium, vol. 1, pp. 293–306. Iowa City, Iowa, USA.

Lee, R.W., 1986. A procedure for testing cored ice under uniaxial tension. Journal of Glaciology 32 (112), 540–541.

Lee, S. G., J. S. Lee, Y. H. Baek, J. K. Paik, and B. J. Kim. 2009. Structural safety assessment in membrane-type CCS in LNGC under iceberg collisions. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Lee,J., Wang, Y.S., "Vessel Transit Through Ridged Ice", Proc. Sixth Conference on Offshore Mechanics and Arctic Engineering, Houston, 1987.

Leira, B. J., L. Borsheim, and Asme. 2008. ESTIMATION OF ICE LOADS ON A SHIP HULL BASED ON STRAIN MEASUREMENTS. Proceedings of the 27th international conference on offshore mechanics and arctic engineering - 2008, vol 3.

Leira, B. J., L. Brsheim, Espeland, and J. Amdahl. 2009. Assessment of ice-induced loads on ship hulls based on continuous response monitoring. Paper presented at MARSTRUCT 2009, 2nd International Conference on Marine Structures - Analysis and Design of Marine Structures, March 16, 2009 - March 18, .

Leppäranta, M., Hakala, R., "Field Measurements of the Structure and Strength of First-year Ice Ridges in the Baltic Sea", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Levine, G.H., et.al., "A Full Scale Test Program on a Great Lakes Ore Carrier during Winter Operations" Report to the U.S. Maritime Administration by Arctec Inc., July, 1973.

Lewis, J.E., Leppäranta, M., Granberg, H.B., "Airborne Laser Profiling of Ice Ridges in the Baltic Sea", Proc. POAC '89, Vol.3, Luleå, Sweden, 1989.

Lewis, J.W., "Recent Developments in Prediction of Level Ice Resistance", Proc. WEGEMT Seventh Graduate School, Ships and Structures in Ice, Helsinki, 54p., 1983.

Lewis, J.W., and Edwards, R.Y.Jr., "Methods for Predicting Icebreaking and Ice Resistance Characteristics of Icebreakers", Transactions of the SNAME Annual Meeting, Vol.78, New York, 1970.

Li, Zhijun, Yongxue Wang, and Guangwei Li. 2002. On the flexural strength of DUT-1 synthetic model ice. Cold Regions Science and Technology 35, (2): 67-72.

Li, Zhijun, Yongxue Wang, and Guangwei Li. 2002. On the flexural strength of DUT-1 synthetic model ice. Cold Regions Science and Technology 35, (2): 67-72.

Light, B., Maykut, G.A, Grenfell, T.C., 2003. Effects of temperature on the microstructure of first-year Arctic sea ice. Journal of Geophysical Research 108 (C2), 3051. doi:10.1029/2001 JC000887.

Likhomanov, V., O. Timofeev, I. Stepanov, V. Kashtelyan, and L. Tsoy. 1998. Ice passport for icebreaker 'pierre radisson' and passport's concept: Further development. Paper presented at Part 2 (of 4), May 24, 1998 - May 29, .

Likhomanov, V.A., "The Strength of Icebreakers and Transport Vessels (According to data from strain-gauge tests)", The Navigational Qualities of Ships, Ed. by Kheysin, D.Y., Popov, I.N., CRREL Translation TL 417, 1973.

Likhomanov, Vladimir A., Nina A. Krupina, Alexey V. Chernov, and Yury P. Gudoshnikov. 2009. Results of definition of the global ice load during in-situ research on impact of the icebreaker "kapitan nikolaev" on various ice formations. Paper presented at 19th (2009) International OFFSHORE AND POLAR ENGINEERING CONFERENCE, June 21, 2009 - June 26, .

Lindqvist, G., "A Straightforward Method for Calculation of Ice Resistance of Ships", Proc. POAC '89, Luleå, Sweden, 1989.

Lin'kov, E.M., 1958. Study of the elastic properties of an ice cover in the Arctic. Vestnik Leniagradskogo Universiteta 13, 17–22 (in Russian).

Liu, S., H. Yu, and R. Basu. 2009. Plastic grillage analysis for ship structure under ICE loads. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Loset, S., and T. Carstens. 1996. Sea ice and iceberg observations in the western barents sea in 1987. Cold Regions Science and Technology 24, (4): 323-40, Colony, R., "A New Look at Sea Ice Thickness", Proc. POAC '87, Fairbanks, Alaska, 1987.

Lu, P., Z. Li, B. Cheng, R. Lei, and R. Zhang. 2010. Sea ice surface features in arctic summer 2008: Aerial observations. Remote Sensing of Environment 114, (4): 693-9 (accessed 23 March 2010).

Luk, C.H., "A Three-Dimensional Plasticity and Momentum Model for Ship Resistance in Level Ice", Proc. Seventh Conference on Offshore Mechanics and Arctic Engineering, Houston, 1988.

Määttänen, M., 1975. On the flexural strength of brackish water in in situ tests. Proceedings POAC'75, vol.1, pp. 349–359. Fairbanks, AL, USA.

Mahmood, Mutmaz, and Arturo Revenga. 2006. Design aspects of winterized and arctic LNG carriers - A classification perspective. Paper presented at 25TH International Conference on Offshore Mechanics and Arctic Engineering, OMAE 2006, June 4, 2006 - June 9, .

Mäkinen, E., Keinonen, A.J., Laine, A., "Ice Resistance Measurements in Ridges with I/B APU in the Baltic Sea", Proc. POAC '75, Fairbanks, Alaska, 1975.

Malmgren, F., 1927. Norwegian north polar expedition with the “Maud”, 1918–1925. Scientific Results 1 (5).

Mansour, A., Seirig, A., "Computer-based Simulation of Ice-Breaking by Impact", Jnl. of Energy Resources Technology, Vol 105, No.4, 6p., 1983.

Margo H. Edwards, Bernard J. Coakley, SCICEX Investigations of the Arctic Ocean System, Chemie der Erde - Geochemistry, Volume 63, Issue 4, 2003, Pages 281-328, ISSN 0009-2819, DOI: 10.1078/0009-2819-00039.

Marion, G.M., Farren, R.E., Komrowski, A.J., 1999. Alternative pathways for seawater freezing. Cold Regions Science and Technology 29 (3), 259–266.

Masterson, D. M., and R. M. W. Frederking. 1993. Local contact pressures in ship/ice and structure/ice interactions. Cold Regions Science and Technology 21, (2): 169-85.

Masterson, D. M., W. P. Graham, S. J. Jones, and G. R. Childs. 1997. A comparison of uniaxial and borehole jack tests at fort providence ice crossing, 1995. Canadian Geotechnical Journal 34, (3): 471-5.

Masterson, D. M., W. P. Graham, S. J. Jones, and G. R. Childs. 1997. A comparison of uniaxial and borehole jack tests at fort providence ice crossing, 1995. Canadian Geotechnical Journal 34, (3): 471-5.

Masterson, D.M., 1992. Interpretation of in situ borehole ice strength measurement tests. Proceedings IAHR Symposium on Ice, vol. 2, pp. 802–815. Banff, AB, Canada.

Masterson, D.M., 1996. Interpretation of in situ borehole ice strength measurement tests. Canadian Journal of Civil Engineering 23 (1), 165–179.

Masterson, D.M., 2009. State of the art of ice bearing capacity and ice construction. Cold Regions Science and Technology 58 (3), 99–112. doi:10.1016/j.coldregions.2009.04.002.

Masterson, D.M., Frederking, R., 2006. Experience with the Canadian Standards Association Offshore Structures Code. Proceedings 16th International Offshore and Polar Engineering Conference, ISOPE'06, vol. 1, pp. 14–19. San Francisco, CA, USA.

Masterson, D.M., Frederking, R.M.W., Truskov, P.A., 2000. Ice force and pressure determination by zone. Proceedings ICETECH2000, pp. 383–390. St. Petersburg,Russia.

Masterson, D.M., Graham, W.P., 1992. Development of the original ice borehole jack. Proceedings IAHR Symposium on Ice, vol. 2, pp. 748–759. Banff, AB, Canada.

Masterson, D.M., Graham,W.P., Jones, S.J., Childs, G.R., 1997. A comparison of uniaxial and borehole jack tests at Fort Providence ice crossing, 1995. Canadian Geotechnical Journal 34, 471–475.

Masterson, D.M., in press. Ice strength: In situ measurement. In: Eicken, H., Gradinger, R., Salganek, M., Shirasawa, K., Perovich, D., Leppäranta, M., editors. Field techniques for sea ice research. University of Alaska Press, Fairbanks, AK, p. 181–213.

Matlock, H., W. P. Dawkins, and J. J. Panak. 1971. Analytical model for ice-structure interaction. Journal of the Engineering Mechanics Division 97, (4) (Aug.): 1083-92.

Matskevitch, D. G. 1997. Eccentric impact of an ice feature: Linearized model. Cold Regions Science and Technology 25, (3): 159-71.

Matsuishi, M., Ikeda, J., Hawakami, H., Hirago, M., "Ship-Ice Floe Collision Analysis Considering the Elastic Deflection of Hull Girder", Proc. SNAME Ice Tech '84, Calgary, Canada, 1984.

Matthew L. Druckenmiller a,⁎, Hajo Eicken a, Mark A. Johnson b, Daniel J. Pringle a,c, Christina C. Williams a Toward an integrated coastal sea-ice observatory: System components and a case study at Barrow, Alaska, Cold Regions Science and Technology 56 (2009) 61–72

McPhee, M.G., "Inferring Ice/Ocean Surface Roughness from Horizontal Current Measurements", Proc. Seventh Conference on Offshore Mechanics and Arctic Engineering, Houston, 1988.

Mejlaender-Larsen, M., and H. Nyseth. 2007. Ice load monitoring. Paper presented at RINA, Royal Institution of Naval Architects International Conference - Design and Construction of Vessels Operating in Low Temperature Environments, May 30, 2007 - May 31, .

Meldrum, D. 2009. Review of satellite data telecommunication systems. Global Ocean Observing System, SOT-V-Doc- I-5.2(2), http://www.ioc-goos.org/index.php?option=com_oe&task=viewDocumentRecord&docID=3675.

Melling, H., Reidel, D.A., Gedalof, Z., 2005. Trends in thickness and extent of seasonal pack ice, Canadian Beaufort Sea. Geophysical Research Letters 32 (24), 1–5.

Mellor, M., "Ship Resistance in Thick Brash Ice", Pub. in Cold Regions Science and Technology, 3(1980) pp. 305-321.

Mellor, M., 1983. Mechanical behavior of sea ice. USA CRREL Monograph 83-1, Hanover, N.H., USA.

Mellor, M., 1986. Mechanical behavior of sea ice. In: Untersteiner, N. (Ed.), Geophysics of Sea Ice, vol. 146. Plenum Press, New York, pp. 165–281.

Melsheimer, C., G. Heygster, and L. Toudal Pedersen. 2008. Integrated retrieval of surface and atmospheric parameters over the arctic from AMSR-E satellite microwave radiometer data using inverse methods.

Milano, V.R., "Ship Resistance to Continuous Motion in Ice", Ph.D. Thesis, Stevens Institute of Technology, Hoboken, N.J., 1972.

Milano, V.R., "Ship Resistance to Continuous Motion in Ice", Transactions of the SNAME Annual Meeting, Vol.80, New York, 1972.

Moritz, R.E., Colony, R., "Statistics of Sea Ice Motion, Fram Strait to North Pole", Proc. Seventh Conference on Offshore Mechanics and Arctic Engineering, Houston, 1988.

Moshaiov, A., and M. R. Steinhilber. 1991. Theoretical assessment of light structural damage due to ship collision with ice. Journal of Offshore Mechanics and Arctic Engineering 113, (1): 61-6.

Moslet, P.O., 2007. Field testing of uniaxial compression strength of columnar sea ice. Cold Regions Science and Technology 48 (1), 1–14.

Mueller, C., "Strengthening of Hull Structures in Ice", Proc. WEGEMT Seventh Graduate School, Ships and Structures in Ice, Helsinki, 54p., 1983.

Muhonen, A., 1991 “Medium scale indentation tests - PVDF pressure measurements, ice face measurements and Interpretation of crushing video”, Client Report by Helsinki University of Technology, Ship Laboratory, Feb.20, 1991.

Muller, L., Payer, H.G., "Loads on Research Vessel POLARSTERN Under Arctic Conditions", Proc. POAC '87, Fairbanks, Alaska, 1987.

Muller, L., Payer, H.G., Moore, C., "Ice Impact on Ship Hulls",Proc. IAHR Ice Symp. Sapporo, Japan, 1988.

Mulmule, S.V., Dempsey, J.P., 2000. LEFM size requirements for the fracture testing of sea ice. International Journal of Fracture 102 (1), 85–98.

Murat, J.R., Lainey, L.M., 1982. Some experimental observations on the Poisson's ratio of sea ice. Cold Regions Science and Technology 6, 105–113.

Murray, John J., and Donald S. Spencer. 1997. Simulation model for a turret moored tanker in pack ice cover. Paper presented at Part 1-B (of 6), April 13, 1997 - April 17, .

Murrell, S.A.F., Sammonds, P.R., and Rist, M.A., 1991 “Strength and failure modes of pure ice and multi-year sea ice under uniaxial loading”, in Ice-Structure Interaction, Jones S.J., et.al. (Eds), IUTAM-IAHR Symposium St. John’s, Newfoundland, Canada, Pub. by Springer Verlag.

Naegle, J.N., "Ice-Resistance Prediction and Motion Simulation for Ships Operating in Continuous Mode of Icebreaking", Ph.D. Thesis, University of Michigan, Ann Arbor, 1980.

Nakawo,M., 1983.Measurements on air porosity of sea ice.Annals of Glaciology 4, 204–208.

Nawwar, A, Noble, P.G., "Ice Force Math Model Investigations", Report by Arctec Canada Limited to Transportation Development Centre, Montreal, Report No. TP 2596, 1980.

Nawwar, A.M., El-Tahan, H., "A Simple Model for Collision of Floating Bodies", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Nevel, D.E., "A Semi-Infinite Plate on an Elastic Foundation" CRREL Research Report No. 136, 1965.

Noble, P.G., Tam, W.K., Menon, B., Bayly, I.M., "Ice Forces and Accelerations on a Polar Class Icebreaker", Proc. POAC '79, Trondheim, Norway, 1979.

Nogid, L.M., "Impact of Ships with Ice", Trans. Leningrad Shipbuild. Inst. No. 26, BSRA Translation No. 1867, 1959.

Nolin, A. W., F. M. Fetterer, and T. A. Scambos. 2002. Surface roughness characterizations of sea ice and ice sheets: Case studies with MISR data. IEEE Transactions on Geoscience and Remote Sensing 40, (7): 1605-15.

NSIDC. All about sea ice - studying. in NSIDC [database online]. Available from http://nsidc.org/seaice/study/index.html.

Nyseth, Havard, and Gabriel Holtsmark. 2006. Analytical plastic capacity formulation for plates subject to ice loads and similar types of patch loadings. Paper presented at 25TH International Conference on Offshore Mechanics and Arctic Engineering, OMAE 2006, June 4, 2006 - June 9, .

Oh, Hoonkyu, Whasoo Kim, and Jaemyung Lee. 2009. Safety of membrane type cargo containment system in LNG carrier under accidental iceberg collision. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Ostaja-Starzewski, M., Jessup, R., Venkatesh, S., "Micromechanics Approach to Sea Ice Dynamics", Proc. Canadian East Coast Conf. on Sea Ice, Bedford Inst. of Oceanography, Dartmouth, N.S., Canada, Jan. 1986.

Otto, T., J. Lu, and M. Chandra. 2009. Polarisation basis transformation of weather radar measurements in the power domain. Advances in Radio Science 7: 279-84.

Paden, J. 2003. Bistatic/Monostatic synthetic aperture radar for ice sheet measurements. University of Kansas: CReSIS, CReSIS TR 118.

Paige, R.A., Lee, C.W., 1967. Preliminary studies on sea ice in McMurdo Sound, Antarctica, during Deep Freeze 65. Journal of Glaciology 6 (46), 515–528.

Paik, J. K. 2010. Some recent advances and future trends in nonlinear structural mechanics for ships and offshore structures. Marine Technology 47, (1): 17-26 (accessed 22 March 2010).

Paik, J. K., and J. K. Seo. 2009. Nonlinear finite element method models for ultimate strength analysis of steel stiffened-plate structures under combined biaxial compression and lateral pressure actions-part II: Stiffened panels. 47, (8-9): 998-1007.

Palmer, A.C., and Sanderson, T.J.O., 1991,“Fractal crushing of ice and brittle solids”, Proc. R.Soc. Lond. A, 433: 469-477.

Parsons, B.L., Lai, M., Williams, F.M., Dempsey, J.P., Snellen, J.B., Everard, J., Slade, T., Williams, J., 1992. The influence of beam size on the flexural strength of sea ice, freshwater ice and iceberg ice. Philosophical Magazine. A 66 (6), 1017–1036.

Peschanskii, I.S., 1960. Arctic and Antarctic sea ice (R). Problemy Arktiki i Antarktiki, vol. 4, pp. 111–129. St. Petersburg, Russia.

Peyton H.R. 1966 "Sea ice strength" Univ. of Alaska Geophysical Institute Report AUG R-182, Dec. 1966.

Peyton, H.R., 1963. Some mechanical properties of sea ice. In: Kingery, W.D. (Ed.), Ice and Snow-Processes, Properties and Applications. MIT Press, Cambridge,MA, pp. 107–113.

Peyton, H.R., 1966. Sea ice strength. University of Alaska Report UAG-182. Geophysical Institute, Fairbanks, AK, USA. 187 pp.

Phillips, L. D., and H. Tanaka. 1994. Simulation of ship-ice collision dynamics: Ice interface modeling considerations. In , 295-302Computational Mechanics Publ.

Popov, Yu.N., et al. Strength of Ships Sailing in Ice, Sudostroyeniye Publishing House, Leningrad, 1967, (English Translation by U.S. Army Foreign Science and Technology Center)

Pounder, E.R., Little, E.M., 1959. Some physical properties of sea ice. Canadian Journal of Physics 37, 443–473.

Pounder, E.R., Stalinski, P., 1960. General Properties of Arctic Sea Ice. Assoc. Science Hydrology, Pub. No 54, pp. 25–34.

Poznyak, I.I., Ionov, B.P., "The Division of Icebreaker Resistance into Components", Proc. SNAME STAR Symp., Ottawa, 1981.

Quincey, D. J., and A. Luckman. 2009. Progress in satellite remote sensing of ice sheets. Progress in Physical Geography 33, (4) (/8/1): 547-67, http://ppg.sagepub.com.qe2a-proxy.mun.ca/cgi/content/abstract/33/4/547.

Richter-Menge, J., J. Overland, A. Proshutinsky, V. Romanovsky, L. Bengtsson, L. Brigham, M. Dyurgerov, J.C. Gascard, S. Gerland, R. Graversen, C. Haas, M. Karcher, P. Kuhry, J. Maslanik, H. Melling, W. Maslowski, J. Morison, D. Perovich, R. Przybylak, V. Rachold, I. Rigor, A. Shiklomanov, J. Stroeve, D. Walker, and J. Walsh (2006) State of the Arctic Report. NOAA OAR Special Report, NOAA/OAR/PMEL, Seattle, WA, 36 pp.

Richter-Menge, J.A., Claffey, K.J., Walsh, M.R., 1993. End-capping procedure for cored ice samples used in tension tests. Journal of Glaciology V39 (133), 698–700.

Richter-Menge, J.A., Cox, G.F.N., 1984. Structure, salinity and density of multi-year sea ice pressure ridges. Proceedings OMAE'84, vol. 2, pp. 194–198. Dallas, TX, USA.

Richter-Menge, J.A., Cox, G.F.N., 1985. A preliminary examination of the effect of structure on the compressive strength of ice samples from multi-year pressure ridges. Journal of Energy Resources Technology V107, 99–102.

Richter-Menge, J.A., Jones, K.F., 1993. The tensile strength of first-year sea ice. Journal of Glaciology 39 (133), 609–618.

Rim, C. W., and T. K. Lee. 2009. Estimation of ICE loads on the bow of an icebreaking research vessel. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Riska, K., "An Analysis of Factors Influencing Ship Response in Collision with Multi-Year Ice Floes", Proc. POAC '89, Luleå, Sweden, 1989.

Riska, K., "On the Mechanics of the Ramming Interaction between a Ship and a Massive Ice Floe", Thesis for degree of Doctor of Technology, Technical Research Centre of Finland, Publications 43, Espoo, Finland, 1987.

Riska, K., "on the Role of Failure Criterion of Ice in Determinating Ice Loads", VTT Ship Lab Report No.7, Espoo, March, 1980.

Riska, K., "Ship Ramming Multi-year Ice Floes, Model Test Results", Technical Research Centre of Finland, Research Notes 818, Espoo, Finland, 1988.

Riska, K., "Theoretical Modelling of Ice-Structure Interaction", to be Presented at IUTAM/IAHR Symp. on Ice-Structure Interaction, St. John's, Canada, 1989.

Riska, K., Daley, C., "M.V. Arctic Ramming Model Test Results", Report 1 of the Joint Research Project Arrangement III between Technical Research Centre of Finland and Canadian Coast Guard, Espoo 1986.

Riska, K., Frederking, R., "Ice Load Penetration Modelling", Proc. POAC '87, Fairbanks, Alaska, 1987.

Riska, K., Frederking, R., "Modelling Ice Load During Penetration into Ice", Joint Report by National Research Council of Canada and Technical Research Centre of Finland, (in Canada Transport Canada Report No. TP8237) 57p.,1987.

Riska, K., P. Kujala, and J. Vuorio. 1983. ICE LOAD AND PRESSURE MEASUREMENTS ON BOARD I. B. SISU. Paper presented at 7th International Conference on Port and Ocean Engineering under Arctic Conditions, .

Riska, K., Rantala, H. and Joensuu A., 1990 "Full scale observations of ship-ice contact" Helsinki University of Technology, Laboratory of Naval Architecture and Marine Engineering, Report M-97, 1990.

Riska, K., Varsta, P., "Structural Ice Loads in the Baltic", Proc. WEGEMT Seventh Graduate School, Ships and Structures in Ice, Helsinki, 22p., 1983.

Riska, Kaj, Shotaro Uto, and Jukka Tuhkuri. 2002. Pressure distribution and response of multiplate panels under ice loading. Cold Regions Science and Technology 34, (3): 209-25.

Riska, Kaj. 1987. ON THE MECHANICS OF THE RAMMING INTERACTION BETWEEN A SHIP AND A MASSIVE ICE FLOE. Publications - Technical Research Centre of Finland(43).

Ritch, Ron, R. Frederking, M. Johnston, Robin Browne, and Freeman Ralph. 2008. Local ice pressures measured on a strain gauge panel during the CCGS terry fox bergy bit impact study. Cold Regions Science and Technology 52, (1): 29-49.

Robert N. Dawson and John W. Grainge, Proposed Design Criteria for Wastewater Lagoons in Arctic and Sub-Arctic Regions Source: Journal (Water Pollution Control Federation), Vol. 41, No. 2, Part I (Feb., 1969), pp. 237-246 Published by: Water Environment Federation

Roth, D.R., Marcellus, R.W.W., "Bearing Capacity of Broken Ice Zones", Report by Canadian Marine Engineering Limited to Public Works Canada, 1986

Rothrock, D.A., Y. Yu, and G.A. Maykut (1999) Thinning of the Arctic sea-ice cover. Geophys. Res. Lett., 26, 3469–3472.

Rudkin, P. 2005. PERD comprehensive iceberg management database. National Research Council of Canada - Canadian Hydraulics Centre, PERD/CHC Report 20-72.

Runeberg, R., "On Steamers for Winter Navigation and Icebreaking", Proc. Institution of Civil Engineers, Vol. XCVII, part III, London, 1888-89.

Rychnovsky, R., "Deep Ice Penetrator", Proc. POAC '87, Fairbanks, Alaska, 1987.

Saeki, H., Ozaki, A., Kubo, Y., 1981. Experimental study on flexural strength and elastic modulus of sea ice. Proceedings of Port and Ocean Engineering under Arctic Conditions, POAC'81, vol. 1, pp. 536–547. Quebec City, PQ, Canada.

Sammonds, P.R., Murrell, S.A.F., Rist, M.A., 1998. Fracture of multi-year sea ice. Journal of Geophysical Research 103 (C10), 21,795–21,815.

Sanderson, T. J. O., Ice Mechanics: Risk to Offshore Structures, Graham & Trotman, London, UK, 1988.

Sanderson, T.J.O., 1988. Ice mechanics: risks to offshore structures. Graham & Trotman, London. pp. 148–151.

Sandwell 1198 “Comparison of International Codes for Ice Loads on Offshore Structures” PERD/CHC Report 11-20,Prepared by:Sandwell Engineering Inc. and Central Marine Research & Design Institute

Sandwell., 1990 “1990 ice indentator tests - Field test report and executive summary” Report of Project 112390 by Sandwell Inc., Calgary, Alberta, Vol. I and II November 1990.

Sandwell., 1992 “Reduction and analysis of 1990 and 1989 Hobson’s Choice ice indentation tests data” Final Report, Project 112588 by Sandwell Inc., Calgary, Alberta, to Conoco Inc. Exxon Prod. Res. Co., Mobil R and D Corp. and National Research Council of Canada, August 1992.

Sandwell., 1993 “Medium scale uniform pressure tests on first-year sea ice at Resolute Bay, N.W.T. 1993” Draft Final Report, Project 113077 by Sandwell Inc., Calgary, Alberta, to National Research Council, Institute for Mechanical Engineering, Ottawa, Vol. I and II, August 1993.

Sawamura, Junji, Kaj Riska, and Torgeir Moan. 2009. Numerical simulation of breaking patterns in level ice at ship's bow. Paper presented at 19th (2009) International OFFSHORE AND POLAR ENGINEERING CONFERENCE, June 21, 2009 - June 26, .

Sayed, M., Frederking, R., "measurement of Ridge Sails in the Beaufort Sea", Can. Jnl. of Civil Engineering, Vol.16, No.1, February 1989.

Sayed, M., Frederking, R., "On Modelling of Ice Ridge Formation", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

Schulson, E., 2002. Compressive shear faults in ice: plastic vs. Coulombic faults. Acta Materialia 50 (13), 3415–3424. doi:10.1016/S1359-6454(02)00154-4.

Schulson, E.M., Duval, P., 2009. Creep and Fracture of Ice. Cambridge University Press, Cambridge, UK.

Schulson, E.M., Fortt, A.L, Iliescu, D., Renshaw, C.E., 2006. Failure envelope of first-year Arctic sea ice: the role of friction in compressive fracture. Journal of Geophysical Research 111 (C11S25). doi:10.1029/2005JC003235.

Schulson, E.M., Nickolayev, O.Y., 1995. Failure of columnar saline ice under biaxial compression: failure envelopes and the brittle to ductile transition. Journal of Geophysical Research 100 (B11), 22,383–22,400.

Schwarz, J., Frederking, R., Gavrillo, V., Petrov, I.G., Hirayama, K.I., Mellor, M., Tryde, P., Vaudrey, K.D., 1981. Standardized testing methods for measuring mechanical properties of sea ice. Cold Regions Science and Technology 4, 245–253.

Schwarz, J., Weeks, W.F., 1977. Engineering properties of sea ice. Journal of Glaciology 19 (81), 499–531.

Seligman, Alain R., and Peter Zahn. 1985. DESIGN OF AN ICEBREAKING OFFSHORE SUPPLY VESSEL. Paper presented at Civil Engineering in the Arctic Offshore, .

Shafrova, Svetlana, and Knut V. Hyland. 2008. The freeze-bond strength in first-year ice ridges. small-scale field and laboratory experiments. Cold Regions Science and Technology 54, (1): 54-71.

Shafrova, Svetlana, and Per Olav Moslet. 2006. In-situ uniaxial compression tests of level ice part II: Ice strength spatial distribution. Paper presented at 25TH International Conference on Offshore Mechanics and Arctic Engineering, OMAE 2006, June 4, 2006 - June 9, .

Shapiro, L.H., Weeks, W.F., 1995. Controls on the flexural strength of small plates and beams of first-year sea ice. “Ice Mechanics-95”, Joint Mechanics Meeting of the American Society for Civil Engineering–Engineering Mechanics Division/American Society of Mechanical Engineers–Applied Mechanics Division / Society of Engineering Science, AMD-Vol. 207, pp. ASME, UCLA, USA, pp. 179–188.

Shemendjuk, G. P., V. A. Babtsev, and O. I. Bratukhin. 1988. DESIGN OF HULL STRUCTURES FOR ICE-NAVIGATING SHIPS. Paper presented at OMAE 1988 Houston, .

Shen, W., Lin, S.Z., 1986. Fracture toughness of Bohai Bay sea ice. Proc. 5th OMAE Symposium, vol. IV, pp. 354–357. Tokyo, Japan.

Shimansky, Yu.,A., "Conditional Guages of the Ice Qualities of Ships", Trans. Arctic Scientific Research Institute, Vol.130, 1938.

Sinha, N.K., 1978. Rheology of columnar-grained ice. Experimental Mechanics 18 (12), 464–470.

Sinha, N.K., 1979. Grain-boundary sliding in polycrystalline materials. Philosophical Magazine. A 40 (6), 825–842.

Sinha, N.K., 1981. Rate sensitivity of compressive strength of columnar-grained ice. Experimental Mechanics 21 (6), 209–218.

Sinha, N.K., 1983a. Field test 1 of compressive strength of first year sea ice. Annals of Glaciology 4, 253–259.

Sinha, N.K., 1983b. Field tests on rate sensitivity of vertical strength and deformation of first-year columnar-grained sea ice. Proceedings POAC 83, vol. 1, pp. 231–242. Helsinki, Finland.

Sinha, N.K., 1984. Uniaxial compressive strength of first-year and multi-year sea ice. Canadian Journal of Civil Engineering 11, 82–91.

Sinha, N.K., 1985. Confined strength and deformation of second-year columnar-grained sea ice in Mould Bay. Proceedings OMAE'85, vol. 2, pp. 209–291. Dallas, TX, USA.

Sinha, N.K., 1986a. Young Arctic frazil sea ice: field and laboratory strength tests. Journal of Materials Science 21 (5), 1533–1546.

Sinha, N.K., 1987. Effective Poisson's ratio of isotropic ice. Proceedings OMAE'87, vol. IV, pp. 189–195. Houston, TX, USA.

Sinha, N.K., 1989, “Kinetics of microcracking and dilation in polycrystalline ice” in Ice-Structure Interaction, Jones, McKenna, Tillotson and Jordaan (Eds), IUTAM/IAHR Symposium, St. John’s, Canada, 1989, publ. by Springer-Verlag, 1991

Sinha, N.K., 1989a. Microcrack-enhanced creep in polycrystalline material at elevated temperatures. Acta Metallurgica 37, 3107–3118.

Sinha, N.K., 1989b. Experiments on anisotropic and rate-sensitive strain ratio and modulus of columnar-grained ice. Journal of Offshore Mechanics and Arctic Engineering 111, 354–360.

Sinha, N.K., 1989c. Closed-loop controlled tensile strength testing method for multiyear sea ice. Proceedings OMAE, vol. IV. The Hague, The Netherlands, pp. 1–6.

Sinha, N.K., 1990. Is minimum creep rate a fundamental material property? Proc. 9th OMAE Conference, vol. IV, pp. 283–288. Houston, TX, USA.

Sinha, N.K., 1991 “In situ multi-year sea ice strength using NRCC borehole indentor”, Proc. of the 10th Intl. Conf. O.M.A.E.

Sinha, N.K., 1991. In Situ multi-year ice strength using NRCC borehole indentor. Proc. of 10th Int. Offshore Mechanics and Arctic Engineering Symposium (OMAE), vol. IV, pp. 229–236. Stavanger, Norway.

Sinha,N.K., 1986b. The borehole jack: is it a useful tool? Proc. of 5th Int. Offshore Mechanics and Arctic Engineering Symposium (OMAE), vol. IV, pp. 328–335. Tokyo, Japan.

Slesarenko, Y.E., Frolov, A.D., 1974. Comparison of elasticity and strength characteristics of salt-water ice. Proceedings of the IAHR Symposium on Ice, vol. 2, pp. 85–87. Leningrad, Russia.

Smith, T.R., Schulson, E.M., 1994. Brittle compressive strength of salt-water columnar ice under biaxial loading. Journal of Glaciology 40 (135), 265–276.

SNiP 2.01.07-85, Loads and influences

SNiP 2.06.01-86, Marine structures, general design provisions

SNiP 2.06.04-82*, Loads and influences on marine structures (from waves, ice and vessels)

Sodhi, D. S., L. Button, R. Boetes, and M. Arockiasamy. 1978. ESTIMATION OF ICE FORCES ON THE HULL OF M. V. ARCTIC EXPLORER BY STRAIN GAUGE MEASUREMENTS. 1, : 475-84.

Spencer, D., and W. D. Molyneux. 2009. Predicting pack ICE loads on moored vessels. Paper presented at International Conference on Ship and Offshore Technology: Ice Class Vessels, September 28, 2009 - September 29, .

Squire, V. A., J. P. Dugan, P. Wadhams, P. J. Rottier, and A. K. Liu. 1995. Of ocean waves and sea ice. Annual Review of Fluid Mechanics 27, (1): 115-68.

St. John, J.W., and Minnick, P., "A Design Method For Icebreaker Hull Loads Due to Level Icebreaking" Proc. SNAME Ice Tech '90, Calgary, Canada, March 1990.

St. John, J.W., Coburn, J.L., Kotras, T.V., "Study of Ice Clogged Channel Clearing Problems", Report by Arctec Inc., Report No. USCG-D-34-81, NTIS no. AD A104 671/3, 1981.

St. John, J.W., Daley, C.G., "Shipboard Measurements of Ice Pressures in the Bering, Chukchi and Beaufort Seas", Proc. Third Conference on Offshore Mechanics and Arctic Engineering, New Orleans, 7p., 1984.

St. John, James W., and James R. Meyer. 1986. STRUCTURAL DESIGN METHODS FOR SURFACE SHIPS OPERATING AT THE ICE EDGE. Naval Engineers Journal 98, (3): 88-94.

Stander, E., Michel, B., 1989. The effect of fluid flow on the development of preferred orientations in sea ice: laboratory experiments. Cold Regions Science and Technology 17 (2), 153–161.

Stehn, L., 1994. Fracture toughness and crack-growth of brackish ice using chevron notched specimens. Journal of Glaciology 40, 415–426.

Stern, F., Ettema, R., Lazaro, J., "Dynamics of Continuous-Mode Icebreaking by a Polar-Class Hull - Part 2: Spectral Analysis", Jnl. of Ship Research, Vol33, No.3, 1989.

Su, Biao, Kaj Riska, and Torgeir Moan. 2010. A numerical method for the prediction of ship performance in level ice. Cold Regions Science and Technology 60, (3) (3): 177-88.

Svec, O.J., Thompson, J.C., Frederking, R.M.W., 1985. Stress concentrations in the root of an ice cover cantilever: model tests and theory. Cold Regions Science Technology 11, 63–73.

Swamidas, A. S. J., Ian J. Jordaan, Stephen J. Jones, and Richard E. McKenna. 1991. Modelling of the ice failure processes in ship/ice interaction. Paper presented at 11th International Conference on Port and Ocean Engineering under Arctic Conditions - POAC'91, September 24, 1991 - September 28, .

Swamidas, A. S. J., Ian J. Jordaan, Stephen J. Jones, and Richard E. McKenna. 1991. Modelling of the ice failure processes in ship/ice interaction. Paper presented at 11th International Conference on Port and Ocean Engineering under Arctic Conditions - POAC'91, September 24, 1991 - September 28, .

Swearengen, J.C., "Deformation and Fracture of Arctic Sea Ice During Kinetic Energy Penetration", Proc. Seventh Conference on Offshore Mechanics and Arctic Engineering, Houston, 1988.

Tabata, T., 1960. Studies on mechanical properties of sea ice V. Measurement of flexural strength. Low Temperature Science, Series A 19, 187–201.

Tabata, T., 1966. Studies on the mechanical properties of sea ice IX. Measurement of the flexural strength in situ. Low Temperature Science, Series A 24, 259–268.

Tabata, T., 1967. The flexural strength of small sea ice beams. Physics of Snow and Ice, vol. I. Hokkaido University, Japan, pp. 481–497. Part 1.

Tabata, T., Fujino, K., 1964. Studies on the mechanical properties of sea ice VIII. Measurement of the flexural strength in situ. Low Temperature Science, Series A 23, 157–166.

Tabata, T., Fujino, K., Aota, M., 1967. Studies on the mechanical properties of sea ice: the flexural strength of sea ice in situ. Physics of Snow and Ice, vol. I(1). Hokkaido University, Japan, pp. 539–550.

Tabata, T., Suzuki, Y., Aota, M., 1975. Ice study in the Gulf of Bothnia: II. Measurement of flexural strength. Low Temperature Science, Series A 33, 199–206.

Takekuma, K., Kayo, Y., "Hull Resistance Test in Floe of Synthetic Ice", Mitsubishi Technical Review, Vol 17, No.4, 6p., (in Japanese, w/ Eng. Summary) 1980.

Tedesco, M., and P. S. Narvekar. 2010. Assessment of the NASA AMSR-E SWE product. IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing 3, (1): 141-59.

Temarel, P. 2007. Loads on ships and offshore structures. Paper presented at 26th International Conference on Offshore Mechanics and Arctic Engineering 2007, OMAE2007, June 10, 2007 - June 15, .

Thompson, E.W., Cheung, H.C., "Continuation on Stress Analysis - Mathematical Model of the Hull Girder Structure of the M.V. Arctic", Report by Melville Shipping Ltd. to Canadian Coast Guard, 1982.

Thompson, E.W., Cheung, H.C., "Development of Prediction Model for Dynamic Response and Bow Loading of Ships" Report by Melville Shipping Ltd. to Transportation Development Centre, Montreal, Report No. TP 4191, 1983.

Timco, G. W., and W. F. Weeks. 2010. A review of the engineering properties of sea ice. Cold Regions Science and Technology 60, (2): 107-29.

Timco, G.W., "Confined Compression Tests: Outlining the Failure Envelope of Columnar Sea Ice", Cold Regions Science and Technology, Vol 12, Elsevier Sci. Pub., 1986.

Timco, G.W., "Indentation and Penetration of Edge-Loaded Freshwater Ice Sheets in the Brittle Range", Proc. Fifth Conference on Offshore Mechanics and Arctic Engineering, Tokyo, 1986.

Timco, G.W., 1986 "Indentation and penetration of edge-loaded freshwater ice sheets in the brittle range", Proc. Fifth Conference on Offshore Mechanics and Arctic Engineering, Tokyo.

Timco, G.W., and Croasdale, K.R, “How Well Can We Predict Ice Loads?”  Proceedings 18th International Symposium on Ice, IAHR’06  Vol. 1, pp167-174, Sapporo, Japan, 2006.

Timco, G.W., Burden, R.P., 1997. An analysis of the shapes of sea ice ridges. Cold Regions Science and Technology 25, 65–77.

Timco, G.W., Frederking, R.M.W., 1982a. Comparative strength of freshwater ice. Cold Regions Science and Technology 6, 21–27.

Timco, G.W., Frederking, R.M.W., 1982b. Compressive strength of multi-year ridge ice. Proceedings Workshop on Sea Ice Ridging and Pile-up. NRC/DBR Technical Memo 134, Calgary, Al, Canada.

Timco, G.W., Frederking, R.M.W., 1983a. Flexural strength and fracture toughness of sea ice. Cold Regions Science and Technology 8, 35–41.

Timco, G.W., Frederking, R.M.W., 1983b. Confined compressive strength of sea ice. Proc. POAC 83, vol. I, pp. 243–253. Helsinki, Finland.

Timco, G.W., Frederking, R.M.W., 1984. An investigation of the failure envelope of granular/discontinuous-columnar sea ice. Cold Regions Science and Technology 9, 17–27.

Timco, G.W., Frederking, R.M.W., 1986. Confined compression tests: outlining the failure envelope of columnar sea ice. Cold Regions Science and Technology 12 (1), 13–28.

Timco, G.W., Frederking, R.M.W., 1990. Compressive strength of sea ice sheets. Cold Regions Science and Technology 17, 227–240.

Timco, G.W., Frederking, R.M.W., 1991. Seasonal compressive strength of Beaufort Sea ice sheets. In: Jones, S., et al. (Ed.), Proceedings IUTAM-IAHR Symposiumon Ice/Structure Interaction, St. John's, Nfld. Springer Verlag, Berlin, Heidelberg, pp. 267–282.

Timco, G.W., Frederking, R.M.W., 1996. A review of sea ice density. Cold Regions Science and Technology 24, 1–6.

Timco, G.W., Johnston, M.E., 2002. Sea ice strength during the melt season. Proceedings of the 16th IAHR International Symposium on Ice, vol. 2, pp. 187–193. Dunedin, New Zealand.

Timco, G.W., Johnston, M.E., 2004. Ice loads on the caisson structures in the Canadian Beaufort Sea. Cold Regions Science and Technology 38, 185–209.

Timco, G.W., O'Brien, S., 1994. Flexural strength equation for sea ice. Cold Regions Science and Technology 22, 285–298.

Timco, G.W., Sayed, M., "Model Tests of the Ridge Building Process in Ice", Proc. IAHR Ice Symp. Iowa City, USA, 1986.

Timofeev, Oleg Ya, and Nina A. Krupina. 2002. Calculation of parameters of local ice loads on data of full scale tests of ice belt structures. Paper presented at Proceedings of the Twelfth (2002) International Offshore and Polar Engineering Conference, May 26, 2002 - May 31, .

Tomin, M., M. M. Zadeh, A. Cormeau, and M. Nessim. 1988. Fracture control for arctic vessels and structures. Paper presented at Probabilistic Methods in Civil Engineering, Proceedings of the 5th ASCE Specialty Conference, May 25, 1988 - May 27, .

Totman, C. A., O. E. Uzorka, J. P. Dempsey, and D. M. Cole. 2007. Sub-size fracture testing of FY sea ice. Paper presented at 6th International Conference on Fracture Mechanics of Concrete and Concrete Structures, FraMCoS-6, June 17, 2007 - June 22, .

Totman, C.A., Uzorka, O.E., Dempsey, J.P., Cole, D.M., 2007. Sub-size fracture testing of FY sea ice. Proceedings of the 6th International Conference on Fracture Mechanics of Concrete and Concrete Structures, vol. 3, pp. 1683–1690. Catania, Italy.

Tratteberg, A., Gold, L.W., Frederking, R., 1975. The strain rate and temperature dependence of Young's modulus of ice. Proceedings IAHR Symposium on Ice Problems, pp. 479–486. Hanover, NH, USA.

Tremblay, L. -B, and M. Hakakian. 2006. Estimating the sea ice compressive strength from satellite-derived sea ice drift and NCEP reanalysis data. Journal of Physical Oceanography 36, (11): 2165-72.

Tsoi, L.G., "Modelling of Vessel's Movement in a Channel Broken Up by Icebreaker", Proc. POAC '83, Helsinki, Finland, 10p., 1983.

Tuhkuri, J. 1996 “Experimental investigations and computational fracture mechanics modelling of brittle ice fragmentation” Acta Polytechnica Scandinavica, Mechanical Engineering Series No. 120, Helsinki, 105p.

Tuhkuri, J., 1987. The applicability of LEFM and the fracture toughness of sea ice. Proc. 9th International POAC Conference, vol. I, pp. 21–32. Fairbanks, Alaska, USA.

Tuhkuri, J., 1994 “Analysis of ice fragmentation process from measured particle size distributions of crushed ice” Cold Regions Science and Technology, 23: 69-82.

Tuhkuri, J., 1995 “Experimental observations of brittle failure process of ice and ice-structure contact” Cold Regions Science and Technology, 23: 265-278.

Tuhkuri, Jukka. 1996. Sandwich structures under ice loading - theoretical and experimental investigations. Marine Structures 9, (2): 259-80, http://dx.doi.org/10.1016/0951-8339(94)00011-G.

Tulk, C. A., H. Kiefte, M. J. Clouter, and R. E. Gagnon. 1997. Elastic constants of ice III, V, and VI by brillouin spectroscopy. Journal of Physical Chemistry B 101, (32): 6154-7.

Tunik, A. 2000. Safe speeds of navigation in ice as criteria of operational risk. 10, (4): 285-91.

Tunik, A.L., "Continuous Motion of a Ship in Ridged Ice", Proc. SNAME Ice Tech '84, Calgary, Canada, 1984.

Tunik, A.L., "Dynamic Ice Loads on a Ship", Proc. IAHR Ice Symp. Hamburg, West Germany, 17p., 1984.

Tunik, A.L., "Hull Girder Bending Forces Due to Ramming Icebreaking", Proc. POAC '85, Narssarssuaq, Greenland, 1985.

Tunik, A.L., Minnick, P.V., St. John, J.W., Chen, Y.K., Chen, A.P.Y., "Ramming Forces on the USCGC Polar Sea", Proc. IAHR Ice Symp. Sapporo, Japan, 1988.

Tunik, Alfred L. 1983. STRENGTH STANDARD FOR ARCTIC SHIPS. Paper presented at 7th International Conference on Port and Ocean Engineering under Arctic Conditions, .

Tunik, Alfred. 2000. Safe speeds of navigation in ice as criteria of operational risk. International Journal of Offshore and Polar Engineering 10, (4): 285-91.

Urabe, N., Inoue, M., 1986. Mechanical properties of Antarctic sea ice. ASME OMAE Symposium, vol. 4, pp. 303–309. Tokyo, Japan.

Urabe, N., Iwasaki, T., Yoshitake, A., 1980. Fracture toughness of sea ice. Cold Regions Science and Technology 3, 29–37.

Urabe, N., Yoshitake, A., 1981a. Strain rate dependent fracture toughnessK1c of pure ice and sea ice. Proc 6th IAHR Ice Symposium, vol. II, pp. 551–563. Quebec City, PQ, Canada.

Urabe, N., Yoshitake, A., 1981b. Fracture toughness of sea ice—in situmeasurement and its application. Proc 6th POAC Conference, vol. I, pp. 356–365. Quebec City, PQ, Canada.

Valanto, P., "Experimental and Theoretical Investigation of the Icebreaking Cycle in Two Dimensions", Ph.D. Dissertation, Univ. of California at Bekkeley, August 1989.

Valanto, P., "Experimental Study of the Icebreaking Cycle in 2-D", Proc. Eighth Conference on Offshore Mechanics and Arctic Engineering, The Hague, 1989.

Vance, G.P., "A Scaling System for Vessels in Ice", Proc. SNAME Ice Tech '75, Montreal, Canada, 1975.

Varsta, P. 1983. MODELLING OF IMPACT BETWEEN SHIP HULL AND ICE. Paper presented at 7th International Conference on Port and Ocean Engineering under Arctic Conditions, .

Varsta, P., "Modelling of Impact Between Ship Hull and Ice", Proc. POAC '83, Helsinki, Finland, 1983.

Varsta, P., "On the Mechanics of Ice Load on Ships in Level Ice in The Baltic Sea", Thesis for degree of Doctor of Technology, Technical Research Centre of Finland, Publications 11, Espoo, Finland, 1983.

Varsta, P., and Riska, K., 1977 “Failure process of ice edge, caused by impact with a ship’s side”, in Ice, Ships and Winter Navigation, Symp. in Oulu Univ, Oulu, Dec. 1977.

Vaudrey, K., 1977. Ice engineering—— study of related properties of floating sea ice sheets and summary of elastic and viscoelastic analyses. U.S. Naval Civil Engineering Laboratory, Report TR860, Port Hueneme, CA.

Vaughan, H., "Design Formula for the Shear Area and Section Modulus of Ice-Breaking Ships", RINA Transactions, 1983.

Vaughan, H., "Flexural Response of Ice-Breaking Ships to Impact Loads", RINA Transactions, 7p., 1986.

Vaughan, Henry. 1986. GLOBAL RESPONSE OF ICEBREAKERS RAMMING HEAVY ICE. Paper presented at Proceedings - Eleventh Ship Technology and Research (STAR) Symposium, .

Verbit, S., G. Comfort, and G. W. Timco. 2006. Development of a database for iceberg sightings off Canada's east coast. Paper presented at Proceedings 18th International Symposium on ice, Sapporo, Japan.

Vinogradov, I.V., "Ships for Ice Navigation", Moscow, Oborongiz, 1946.

Vinogradov, O.C., "Simulation Methodology of Vessel-Ice Floes Interaction Problem", Proc. Fifth Conference on Offshore Mechanics and Arctic Engineering, Tokyo, 1986.

Vuorio, J., Riska, K., Varsta, P., "Long Term Measurements of Ice Pressure and Ice Induced Stresses on the Icebreaker Sisu in Winter 1978", Winter Navigation Research Board Report 28, 50p., 1979.

Wang, Ge, and Christopher J. Wiernicki. 2006. Using nonlinear finite element method to design ship structures for ice loads. Paper presented at 2005 Society of Naval Architects and Marine Engineers Annual Meeting, October 19, 2005 - October 21, .

Wang, Ge, Roger Basu, Dipak Chavda, Shewen Liu, Myong-Sup Lee, Yong-Suk Suh, and Yoo-Jhung Han. 2005. Rationalization of design of side structure of ice-strengthened tankers. International Journal of Offshore and Polar Engineering 15, (3): 210-4.

Wang, Jungyong, Ayhan Akinturk, and Neil Bose. 2006. Numerical prediction of model podded propeller-ice interaction loads. Paper presented at 25TH International Conference on Offshore Mechanics and Arctic Engineering, OMAE 2006, June 4, 2006 - June 9, .

Wang, Y.S., 1979. Crystallographic studies and strength tests of field ice in the Alaskan Beaufort Sea. Proc. POAC 79, vol. I, pp. 651–665. Trondheim, Norway.

Wang, Y.S., 1981. Uniaxial compression testing of Arctic sea ice. Proceedings of the Sixth Int. Conf. POAC, vol. 1, pp. 346–355. Quebec City, PQ, Canada.

Wang, Y.S., Poplin, J.P., 1986. Laboratory compressive tests of sea ice at slow strain rates from a field test program. Proc. OMAE 86, vol. 4, pp. 379–384. Tokyo, Japan.

Wang, Zhiguo, Derek B. Muggeridge, Stephen J. Jones, and A. S. J. Swamidas. 1997. Numerical simulation of ridge and sheet ice loads on a proposed faceted conical structure. Paper presented at Part 1-B (of 6), April 13, 1997 - April 17, .

Weeks, W.F., 1985. The variation of sea ice strength within and between multiyear pressure ridges in the Beaufort Sea. Journal of Energy Resources Technology 107 (2), 167–172.

Weeks, W.F., 1998. Growth conditions and the structure and properties of sea ice. In: Leppäranta, M. (Ed.), Physics of Ice-covered Seas, vol. 1. Helsinki University Printing House, Helsinki, Finland, pp. 25–104.

Weeks, W.F., Ackley, S.F., 1982. The growth, structure and properties of sea ice. USA CRREL Monograph 82-1, Hanover, N.H., USA.

Weeks, W.F., Ackley, S.F., 1986. The growth, structure, and properties of sea ice. In: Untersteiner, N. (Ed.), The Geophysics of Sea Ice. NATO ASI Series, vol. 146. Plenum Press, New York, pp. 9–164.

Weeks, W.F., Anderson, D., 1958. An experimental study of strength of young sea ice. Trans. American Geophysical Union 4 (4), 641–647.

Weeks, W.F., Assur, A., 1967. The mechanical properties of sea ice. U.S. Army CRREL Monograph II-C3, Hanover, NH, USA.

Weeks, W.F., Assur, A., 1968. Themechanical properties of sea ice. Proceedings, Conference on Ice Pressures Against Structures, Quebec City, Laval University, National Research Council of Canada NRC-DBR Report Tech. Memo.No. 92, pp. 25–78. Ottawa, ON, Canada.

Weeks, W.F., Gow, A.J., 1980. Crystal alignments in the fast ice of Arctic Alaska. Journal of Geophysical Research 85 (C2), 1,137–1,146.

Weeks, W.F., in press. On Sea Ice. University of Alaska Press, Fairbanks AL, USA.

Weeks, W.F., Lee, S.O., 1958. Observations on the physical properties of sea ice at Hopedale, Labrador. Arctic II (3), 135–155.

Wettlaufer, J.S., 1998. Introduction to crystallization phenomenon in natural and artificial sea ice. In: Leppäranta, M. (Ed.), Physics of Ice-covered Seas, vol. 1. Helsinki University Printing House, pp. 105–194. Helsinki, Finland.

White, R.M., "Prediction of Icebreaker Capabilities", Royal Institution of Naval Architects, 1961.

Wiernicki, Christopher J. 1987. DAMAGE TO SHIP PLATING DUE TO ICE IMPACT LOADS. Marine Technology 24, (1): 43-58.

Wilchinsky, Alexander V., and Daniel L. Feltham. 2004. Dependence of sea ice yield-curve shape on ice thickness. Journal of Physical Oceanography 34, (12): 2852-6.

Williams, F.M., Everard, J., Butt, S., 1992. Ice and snow measurements in support of the operational evaluation of the Nathanial B. Palmer in the Antarctic winter environment. NRC/IMD Report TR-1992-14, St. John's, NL, Canada.

Williams, T. D., and V. A. Squire. 2010. On the estimation of ice thickness from scattering observations. Dynamics of Atmospheres and Oceans 49, (2-3): 215-33.

Wong, T.T., Gale, A.D., Sego, D.C., Morgenstern, N.R., "Shear Box Tests on Broken Ice", Proc. POAC '87, Fairbanks, Alaska, 1987.

Woolgar, Reeni C., and D. Bruce Colbourne. 2010. Effects of hull-ice friction coefficient on predictions of pack ice forces for moored offshore vessels. Ocean Engineering 37, (2-3): 296-303.

Wright, Christopher, Seymour, Jonatha, Clinton, Graeme, Lumsden, Wayne and Stubbs, Thom. Canadian Arctic Shipping Assessment - Main Report June 2007

Zahn, P. B., and P. V. Minnick. 1988. LOCAL IMPACT PRESSURES DUE TO FIRST YEAR ICE IN THE MARGINAL ICE ZONE. Paper presented at OMAE 1988 Houston.

Gpus were traditionally not used for scientific simulation because they were limited to simplified single-precision floating point architectures as that was all that was required to produce graphics in up to 32-bit colour.

http://nsidc.org/seaice/study/visible_remote_sensing.html

http://nsidc.org/seaice/study/passive_remote_sensing.html

http://nsidc.org/seaice/study/active_remote_sensing.html